BMW
Press

12 / 2004
valid from:

2005
Specifications BMW 5 Series touring

523i, 525i, 530i, 545i  

	Body
fsdfd

d

Anzahl Türen / s
	
	523i
	525i
	530i
	545i

	No. of doors / seats

l
	
	5 / 5
	5 / 5
	5 / 5
	5 / 5

	Length / width / height (unladen) 

)
	mm
	4843 / 1846 / 1491
	4843 / 1846 / 1491
	4843 / 1846 / 1491
	    4843 / 1846 / 1491

	Wheelbase
	mm
	2886
	2886
	2886
	2886

	Track front / rear

wgsdfg


	mm
	1558 / 1581
	1558 / 1581
	1558 / 1581
	1558 / 1581

	Turning circle
	m
	11,4
	11,4
	11,4
	11,4

	Fuel tank capacity
	approx. ltr
	70
	70
	70
	70

	Cooling system incl. heating

l
	ltr
	10,0 (10,5)
	9,8 (10,2)
	10,0 (10,5)
	13,8 (14,2)

	Engine oil

Getriebeöl


	ltr
	6,5
	6,5
	6,5
	8,0

	Transmission fluid
	ltr
	Permanent filling
	Permanent filling
	Permanent filling
	Permanent filling

	Final drive fluid
	ltr
	Permanent filling
	Permanent filling
	Permanent filling
	Permanent filling

	Weight, unladen (EU 
)
	kg
	1635 (1655) 
	1640 (1665) 
	1655 (1665) 
	1795 (1800)

	Max load (DIN)
	kg
	660
	600
	660
	560

	Max. permissible weight (DIN)
	kg
	2160 (2180) 
	2165 (2190) 
	2180 (2190) 
	2280 (2285)

	Permissible axle load front/rear
	kg
	980 / 1280
	980 / 1280
	980 / 1280
	1050 / 1300

	Max. trailer load 

	
	
	
	
	

	braked (12%) / unbraked


	kg
	              1800 / 750 
	              1800 / 750 
	              2000 / 750 
	2000 / 750

	Max. roof load/trailer nose weight
	kg
	100 / 90
	100 / 90
	100 / 90
	100 / 90

	Luggage comp. capacity VDAKofferrauminhalt nach VDA
	ltr
	500 – 1650 5
	500 – 1650 5
	500 – 1650 5
	500 – 1650 5

	Drag coefficient
	cX x A
	0,30 x 2,26
	0,30 x 2,26
	0,30 x 2,26
	0,30 x 2,26

	Engine
	
	
	
	
	

	Layout / No. of cylinders / Valves Zylinder/Ventile


	
	Reihe / 6 / 4
	Reihe / 6 / 4
	Reihe / 6 / 4
	V90 / 8 / 4

	Engine management
	
	MSV70
	MSV 70
	MSV70
	           Bosch ME9.2.1

	Displacement
	cc
	2497
	2494
	2996
	4398

	Bore / Stroke
	mm
	78,8 / 82,0
	78,8 / 82,0
	88,0 / 85,0
	82,7 / 92,0

	Compression ratio


	:1
	11,0 
	11,0 
	10,7 
	10,0 

	Fuel grade
	RON
	ROZ 91-98 3
	ROZ 91-98 3
	ROZ 91-98 3
	ROZ 91-98 3

	Max. output
	kW / bhp
	130 / 177
	160 / 218
	190 / 258
	245 / 333

	At engine speed
	rpm
	5800
	6500
	6600
	6100

	Torque
	Nm
	230
	250
	300
	450

	At engine speed
	rpm
	3500-5000
	2750-4250
	2500-4000
	3600

	Electrics
	
	
	
	
	

	Battery / Location
	Ah / -
	80 / boot
	80 / boot
	80 / boot
	90 / boot

	Alternator
	A / W
	155/ 2170
	155 / 2100
	155 / 2170
	170 / 2380

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust-rod spring strut axle in aluminium; compensation of transverse forces; anti-dive


	Rear suspension
	
	Integral IV multi-arm axle in aluminium; wheel suspension with special effect and anti-squat/anti-dive

	Brakes, front
	
	Single-piston floating-caliper disc brakes / vented

	Diameter
	
	310 x 24, vented
	324 x 30, vented
	324 x 30, vented
	       348 x 30, vented

	Brakes, rear
	
	Single-piston floating-caliper disc brakes / vented

	Diameter
	mm
	320 x 20, vented
	320 x 20, vented
	320 x 20, vented 
	       345 x 24, vented

	Driving Stability Systems
	
	ABS+CBC, ASC; DSC, DTC, DBC; Optional: Dynamic Drive

ABS+CBC, ASC

ABS, CBC, ASC
	

	Steering
	mm
	Rack-and-pinion Servotronic,  Optional: Active Steering wiServotronic
	

	Overall ratio
	:1
	13,7
	13,7
	13,7
	                            13,7

	Type of transmission
	
	6-gear Steptronic automatic transmission

	Transmission ratio 
I

  
	:1
	4,32 (4,17)
	4,323 (4,171)
	4,350 (4,17)
	4,055 (4,171)

	
II
	              :1
	2,456 (2,34)
	2,456 (2,340)
	2,496 (2,34)
	2,396 (2,340)

	
III
	:1
	1,659 (1,52)
	1,659 (1,521)
	1,665 (1,52)
	1,582 (1,521)

	
IV
	:1
	1,23 (1,14)
	1,230 (1,143)
	1,230 (1,14)
	1,192 (1,143)

	
V
	:1
	1,0 (0,87)
	1,000 (0,867)
	1,0 (0,87)
	1,000 (0,867)

	
VI
	:1
	                0,848 (0,69)
	0,848 (0,690)
	                0,851 (0,69)      0,8510,
	0,872 (0,691)      0,8510,

	
R
	:1
	3,93 (3,40)
	3,938 (3,403)
	3,926 (3,40)
	3,677 (3,403)

	Final drive ratio
	:1
	3,23 (3,91)
	3,38 (3,91)
	2,23 (3,73)
	2,93 (3,38)

	Tyres 
	:1
	       225/55R16 95 W
	       225/55R16 95 W
	225/50R17 94 W
	225/50R17 94 Y

	Wheels 
	:1
	                 7 J x 16 LM
	                 7 J x 16 LM
	              7 J x 16 LM 
	                7,5 J x 17 Y 

	Performance
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	12,0 (12,2)   
	9,8 (9,9)
	8,3 (8,4)
	7,0

	Output per litre
	kW/ltr
	52,1

,,1
	64,1
	63,4
	55,7

	Acceleration        0–62mph
	sec
	8,8 (9,5)
	7,8 (8,3)
	6,6 (6,9)
	5,9 (6,0)

	                            0–1000 m
	sec
	29,6 (30,3)
	28,2 (28,7)
	26,6 (26,8)
	25,4 (25,6)

	50 – 75 mph in 4th gear
	sec
	9,7 
	8,3 (-) 
	6,9/- (-) 
	6,1/7,7 (-) 

	Top speed
	km/h
	227 (224)
	240 (237)
	250 (250)
	250 (250)

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	ltr/100km
	12,1 (13,7)
	12,5 (13,5)
	13,1 (13,3)
	16,8 (16,4)

	Out of town
	ltr/100km
	6,7 (7,1)
	6,8 (7,2)
	6,9 (7,3) 
	8,3 (8,2) 

	Overall
	ltr/100km
	8,7 (9,5)
	8,9 (9,5)
	9,2 (9,5)
	11,3 (11,2)

	CO2 
	g/km
	210 (230)
	215 (229)
	222 (230)
	274 (271)

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU4
	EU4
	EU4
	EU4



 Weight of the car in road trim (DIN) plus 75 kg for driver and luggage

2 Deviations are possible under certain conditions
















