BMW
Press

11/2004
effective from:

2005
Specifications BMW 5 series touring

525xi, 530xi

	Body

Fsdfd

d

Anzahl Türen / s
	
	525xi
	
	530xi
	

	No. of doors / seats
	
	5 / 5
	
	4 / 5
	

	Length/width/height (unladen) iiun(unladen

)
	mm
	4843 / 1846 / 1491
	
	5039 / 1902 / 1491
	

	Wheelbase
	mm
	2886
	
	
        2990 
	

	Track, front / rear
	mm
	1558 / 1581
	
	1578 / 1596
	

	Turning circle
	m
	11,9
	
	12,1
	

	Fuel tank capacity / range
	ca. ltr
	70
	
	88
	

	Cooling system incl. heater
	ltr
	10,0 (10,5)
	
	10,5
	

	Engine oil
	ltr
	6,5
	
	10,8
	

	Transmission fluid
	ltr
	Permanent filling
	
	Permanent filling
	

	Final drive fluid
	ltr
	Permanent filling
	
	Permanent filling
	

	Weight, unladen (EU) 1
	Kg
	1760 (1775) 
	
	2115
	

	Max. load (DIN)
	kg
	600
	
	580
	

	Max. permissible weight (DIN)
	kg
	2285 (2300) 
	
	2620
	

	Max. permissible axle load front/rear / r *frfront/ r
	kg
	1050 / 1300
	
	1275 / 1400
	

	Max. trailer load 2
	
	
	
	
	

	braked (12%) / unbraked
	kg
	1800 (2000) /750 
	
	2100 / 750
	

	Max. roof load / trailer nose
	kg
	100 / 90
	
	100 / 100
	

	Luggage comp. capacity, VDA
	ltr
	500 – 1650 5
	
	500
	

	Drag coefficient
	cX x A
	0,33x 2,28
	
	0,29 x 2,38
	

	Engine
	
	
	
	
	

	Layout/No. of cylinders/valves
	
	R / 6 / 4
	
	V / 8 / 4
	

	Engine management
	
	DDE6
	
	DDE6
	 

	Displacement
	cm³
	2993
	
	3423
	

	Bore / stroke
	Mm
	84,0 / 90,0
	
	93,0 / 87,0
	

	Compression
	:1
	17,0
	
	17,0
	

	Fuel grade
	ROZ
	Diesel
	
	Diesel
	

	Max. output
	kW 
	170
	
	220
	

	At engine speed
	rpm
	4000
	
	4000
	

	Max. torque
	Nm 
	520
	
	700
	

	At engine speed
	rpm
	2000 - 2750
	
	2000 - 2500
	

	Electronic
	
	
	
	
	

	Battery / location
	Ah / -
	90 / boot
	
	90 / boot
	.

	Alternator
	A / W
	180 / 2520
	
	180 / 2520
	

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust-rod spring strut axle in aluminium; compensation of transverse 
	
	
	

	
	
	forces; anti-dive

	Rear suspension
	
	Integral IV multi-arm axle in aluminium; wheel suspension with special effect and 

	
	
	anti-squat/anti-dive (pneumatic suspension with self-levelling as an option)

	Driving stability system
	
	DSC (incl ABS, CBC, DBC, ASC); Dynamic Drive (sway stability system) and

 üdoifj , Dynamic stability control (DSC), 

ABS, ASC+T

ABS, ASC+T

	
	
	EDC-C (continuous electronic damper adjustment) as an option

	Brakes, front
	
	Single-piston swing-calliper disc brakes (inner-vented)

	Diameter
	mm
	348 x 30
	
	374 x 36
	

	Brakes, rear
	
	Single-piston swing-calliper disc brakes (inner-vented)
	
	
	

	Diameter
	mm
	345 x 24
	
	370 x 24
	

	Steering
	
	Rack-and-pinion Servotronic 

	Overall ratio
	:1
	13,1
	
	13,1
	

	Type of transmission
	
	6–speed Steptronic automatic transmission 


	
	
	               ZF 6HP26
	                 
	ZF 6HP32
	

	Transmission ratio     I

  
	:1
	4,17
	
	4,17
	

	                                  II
	:1
	2,34
	
	2,34
	

	                                  III
	:1
	1,52
	
	1,52
	

	                                  IV
	:1
	1,14
	
	1,14
	

	                                  V
	:1
	0,87
	
	0,87
	

	                                  VI
	:1
	0,69
	
	0,69
	

	                                   R
	:1
	3,40
	
	3,40
	

	Final drive ratio
	:1
	2,81
	
	3,93
	

	Tyres
	
	245 / 55R17 102W
	
	245 / 50R18 100W 102W
	

	Wheels 
	
	8 J x 17 LM
	
	8 J x 18 LM
	

	Performance
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	11,2
	
	9,3
	

	Output per litre
	ltr/kW
	56,8
	
	49,7
	

	Acceleration       0 - 100 km/h
	s
	7,8
	
	6,8
	

	Top speed
	km/h
	238
	
	250
	

	Fuel consumption (EU-cycle)
	
	
	
	
	

	urban
	ltr/100km
	11,3
	
	13,5
	

	extra-urban
	ltr/100km
	6,4
	
	7,2
	

	composite
	ltr/100km
	8,2
	
	9,5
	

	CO2 
	g/km
	216
	
	251
	

	
	
	
	
	
	


Values in brackets are valid for auto transmission


 Weight of the operable vehicle (DIN) plus 75 kg for driver and luggage


2 May be increased under certain condition

3 Performance and power figures are based on ROZ 98, fuel economy on ROZ 95


These specifications are for the German market. Deviations from the model variants described here are possible in various countries.
