	
	M
	Media Information

	Date 
	
	1st August 2005 

	Subject
	
	Run Flat Tyre : an exclusive technology BMW.

	Page
	
	2


BMW Group

Africa & Caribbean

Corporate Communication

BMW Group
Africa & Caribbean 

Corporate Communication 
Media Information

1st August 2005

Company
Bayerische
Motoren Werke
Aktiengesellschaft

Postal Address

BMW AG

D-80788 München

Internet

www.bmwgroup.com

Run Flat Tyre: an exclusive technology BMW.

BMW is the world’s first manufacturer to provide run flat tyres as standards for the BMW 1 Series, BMW 3 Series, BMW Z4, and available as an option for all other model series. Run Flat Tyres are also provided as standards for MINI.

The concept

Two advantages of the run flat technology are rigidity and heat resistance, so that the tyre can carry weight when it is rotating with total air loss.

Thanks to reinforced sidewalls run flat tyres continue to operate up to specified maximum distance after a loss of tyre pressure:

· The sidewall is constructed with reinforced rubber, carrying the vehicle’s weight at zero pressure

· The bead wire is wider and reinforced, ensuring a secure fit on the rim even at zero pressure, and a special bead filler generating very little heat is used

Advantages

Maximum active safety: safety is at the top of the motoring agenda. The first safety benefit is that run flat tyres enable the driver to maintain control and possibly avoid a potentially fatal situation if a vehicle in motion suffers a rapid loss of tyre pressure. In addition to this, run flat tyres enable the driver to leave the potential risk area, continue the journey, and avoid the dangerous steps of jacking up the vehicle on the roadside, especially at night.

Safety warning system: an on-board tyre pressure monitoring system is mandatory for all run flat tyres fitments providing the driver with an immediate warning in case of pressure loss. In this case safety systems such as ABS, SAC or DSC continue to be operational.

Comfort increase: run flat tyre provides more comfort and convenience. Within the specified speed and distance limits (typically 80 km/h up to 250km), the driver can either complete his journey or travel to the next qualified service station to replace the damaged tyre.

Weight and space benefits: run flat tyres save weight, by eliminating the need to carry a spare tyre. Of course, the free space created by eliminating the classic space wheel gives BMW engineers extra design opportunities and offers the end user more storage space.

Unchanged price: run flat tyres cost at present time only 25% more than regular wheel. This cost is compensated through savings of spare wheel and rim.

If you have any questions, please contact:

BMW Group

Africa & Caribbean 

Jean-Michel Juchet, General Manager Marketing & PR, Regional Office Africa & Caribbean 

E-mail: Jean-Michel.JA.Juchet@bmw.de
Julien Chobert, Manager Communication, PR & Events, Regional Office Africa & Caribbean

E-Mail: Julien.Chobert@bmw.de
Please make sure to have registered to our BMW Group Africa & Caribbean PressClub:

http://www.press.bmwgroup.com/pressclub/ka02.nsf
PAGE  

