21
BMW
Media

Information

12/2004
Page 2

[image: image1.png]


The new BMW R 1200 RT.
Contents.
1.
Technical Features, Special Features of the Motorcycle. 
 3

2.
Range of Equipment. 
 12

3.
Engine Output and Torque. 
 15

4.
Specifications. 
 16

5.
Colours. 
 17

[image: image2.png]kW

=
Z

Output (to 95/1/EC)

90

80

70

00

50

40

30

20

10

160

Torque (to 95/1/EQG)

140

120

100

80

00

40

20

0

2000 3000 4000 5000

Engine speed [min]

6000

7000

8000

9000



1. Technical Features, 
Special Features of the Motorcycle.

The new Boxer generation in the tourer segment – more powerful, lighter and simply ideal for active, relaxed riding.

BMW Motorrad is introducing the R 1200 RT long-distance tourer as the 
se​cond model in the new Boxer generation. Proceeding from the technical found​ation established in spring 2004 by the R 1200 GS, BMW’s devel​opment engineers have raised the new machine to an even higher level of perfection in all its features and fortes.

With the R 1150 RT already acknowledged in the past as the epitome for superior comfort and long-distance riding pleasure, the R 1200 RT, boasting 16 per cent more power on approximately 20 kg less weight, offers a 
signifi​cant boost in dynamic performance and agility. Combined with excellent rid​ing characteristics, superior riding comfort and a generously designed body for highly effective protection from wind and weather, as well as ergonom​ic​ally perfect, active riding features, the new R 1200 RT has everything it takes 
for absolute leadership in convincing and winning over the motorcycle touring enthusiast.

The new R 1200 RT is rounded off by a highly versatile range of equipment and accessories tailored to the rider and meeting all his – or her – individual  wishes, thus once again setting the standard for dynamic, comfortable and safe motorcycling at all times. 

The most important features of the new R 1200 RT at a glance:

•
Weight in road trim (without cases) 259 kg (571 lb);
dry weight (without cases) 229 kg (505 lb).

•
Air-cooled flat-twin power unit displacing 1,170 cc and featuring a newly aligned balance shaft.    

•
More power and performance: 81 kW (110 bhp) maximum output and 115 Nm (85 lb-ft) maximum torque.       

•
Electronic BMS-K engine management with dual ignition and anti-knock control.             

•
Fully controlled three-way catalytic converter with two oxygen sensors.

•
Six-speed gearbox with helical gears.  

•
Innovative on-board network with CAN-bus technology.

•
Highly stable, light frame in tubular construction.

•
Lightweight Paralever with weight-optimised drive shaft, adjustable TDD spring strut. 

•
Extra-stiff Telelever ensuring a smooth and comfortable response. 
•
New cast wheels.
•
EVO brakes with BMW Motorrad Integral ABS (partly integrated version)
  
fitted as standard.
•
New body with very good wind and weather protection as well as 
     in​te​grated rear-view mirrors.
•
Electrical windshield adjustment.
•
Fuel tank made of special, extra-strong synthetic material, 
 
27 litres (5.9 Imp gal) capacity for long range.
•
Two-piece, adjustable seat in two versions and seat height ranging from
      780–840 mm (30.7–33.1´´).
•
New hard-shell cases (32 litres volume per case) featured as standard, 
      with accurately fitting four-point locking system and covers finished in 
      body colour. 
•
Fuel tank railing for fastening accessories with maximum convenience.

Power unit – special modifications for even more output.

The all-new flat-twin power unit featuring a balance shaft and displacing 
1,170 cc presented for the first time in the R 1200 GS is the “heart” of the new R 1200 RT. The concept and features of this new engine are described in detail in the press kit for the R 1200 GS.                                    

Now featured in the Tourer, the engine offers even more power and torque increased to 81 kW (110 bhp). Inter alia, this extra power comes from an increase in engine speed by 500 rpm to 7,500 rpm, a different intake system, new silencers as well as modified cam​shafts also serving to boost engine output.      

This power pack is rounded off by an increase in the geometric compression ratio to 12.0:1 ensured, inter alia, by higher pistons.               

Featuring BMS-K electronic engine management presented for the first time in the R 1200 GS and boasting both anti-knock control as well as separate oxygen sensors controlling the fuel/air mixture individually for each cylinder, the R 1200 RT carries over the technological leadership of the new Boxer gen​eration into the touring motorcycle range. As on the GS, this latest gen​eration of engine electronics combines automatic idle speed control with an automatic cold start function. Anti-knock control enables the motorcycle to run on lower fuel grades down to 95 RON without suffering any damage (the engine is designed for 98 RON, the only difference with 95 RON being a slight decrease in engine output). Precise adjustment of the fuel/air mixture 
to each cylinder, finally, together with the high compression ratio and dual 

ignition, ensures an even higher standard of fuel economy than on the form​er model, the R 1150 RT. For comparison, fuel consumption of the R 1200 RT at a steady 120 km/h or 75 mph is 4.8 ltr/100 km or 58.8 Imp gals, while the 
R 1150 RT required 5.5 ltr/100 km or 51.4 Imp gals for the same exer​cise at the same speed. A fully controlled three-way catalytic converter, fi​nal​ly, ensures supreme environmental compatibility also on the R 1200 RT, offering the latest generation of exhaust gas management and the technical features so characteristic of BMW Motorrad for so many years.

The transmission – active riding features ensured by an extra-short sixth gear.

An all-new development for the R 1200 GS, the helical-gear transmission weighing only 13 kg or 28.7 lb, is also featured in the R 1200 RT. 
Shifting gears smoothly and precisely, and featuring extra-short gear travel between the individual gears, the gearbox has been improved once again 
by detailed optimisation of the shift mechanism.

Designed for active riding, sixth gear is particularly short and dynamic. 
In all, the optimum gear increments allow fast acceleration from 0–100 km/h 
in an impressive 3.6 seconds.

The frame – re-designed in proven BMW technology.

The two-piece main/rear tubular steel spaceframe combines maximum 
stiff​ness with supreme strength. Naturally, it retains the weight-saving truss struc​ture already featured on the R 1200 GS, incorporating the engine as 
a load-bearing unit. But at the same time the particular structural configuration of the new RT required a new frame design offering exactly the right quali​ties for this machine.

The front frame is a welded steel structure, the rear frame is made of straight steel tubes welded to one another. As on all four-valve Boxers, the front and rear frame are bolted firmly to the engine/gearbox unit, jointly forming the load-bearing structure. The longitudinal arm incorporated in the front-wheel Telelever rests as before on the engine block, the Paralever swinging arm pivots on the rear frame.  

Telelever front-wheel suspension – all the qualities of the previous system plus helpful modifications.

Conceived and designed strictly for road use and superior comfort, 
the Tele​lever comes with a fixed tube measuring 35 mm or 1.38´´ in diameter, as on the former model, since this is the best solution for a motorcycle of this char​acter. In conjunction with the modified longitudinal arm of the R 1200 GS and suitable geometric adjustment, the result is a supreme combination 


of-fer​ing a perfect balance of sporting and comfort-oriented spring action. 
Like on the R 1200 GS, this maintenance-free system is designed to 
reduce brake dive to virtually zero, contributing, inter alia, to greater stability when applying the brakes while leaning over at an angle in bends. 
Front-wheel spring travel is 120 mm or 4.72´´.

Rear-wheel guidance and suspension – the Evo-Paralever and 
TDD spring strut. 

The lightweight drive shaft featuring the BMW Motorrad Paralever 
(Evo-Paralever) is hardly heavier than a conventional swinging arm with a drive chain. And apart from the modifications required for the wider tyres, this wheel guidance system has been carried over completely from the R 1200 GS, where this new design has already proven the significant progress it is able to offer: Despite lower weight, the entire unit is even stiffer than on the former model.

Placing the support bar for taking up forces on the rear wheel drive 
housing above the swinging arm helps to provide extra ground clearance, 
the 10 per cent reduction in weight versus the former model reducing unsprung mass​es accordingly. The bottom line for the rider is a smooth and sensitive res​ponse at all times plus load change reactions reduced to 
a minimum.

The new design of the rear wheel drive also comes in a particularly dynamic look thanks to the 50-millimetre (1.97-inch) hole drilled through the axle tube in the housing. A further advantage is the lifetime fluid filling, with no further need to change the transmission fluid for the entire life of the motorcycle.

The rear central spring strut on the Paralever swinging arm incorporates 
travel-dependent damper technology referred to as TDD for short, ensuring a progressive increase in damper forces as a function of spring travel. 
The suspension therefore responds smoothly and comfortably to even the small​est bumps on the road, at the same time offering generous reserves 
on bad roads and tracks with grooves along the surface or a very uneven structure at the top. This spring strut system was featured for the first time in 2002 on the R 1150 GS Adventure, with spring travel at the rear measuring 135 mm or 5.3´´.

As an option the new R 1200 RT will also be available with BMW Motorrad’s ESA (Electronic Suspension Adjustment). Introduced as a production fea​ture for the first time worldwide on the new K 1200 S, this system enables the 
rider to adjust the spring base and damper rate – the latter even while riding – conveniently at the touch of a button from the handlebar. 

Wheels and tyres – light-alloy cast wheels in new five-spoke design. 

The newly developed, extra-light cast aluminium wheels in almost filigree five-spoke design combine the advantage of easy care with superior strength and stiffness. The front wheel measures 3.5 x 17 inches, the rear wheel 5.5 x 17. Tyre dimensions are 120/70 ZR17 at the front and 180/55 ZR17 at the rear. 

Brakes – EVO brake system with partly integrated ABS for safe and dependable deceleration.

The proven EVO brake system featuring BMW Motorrad Integral ABS fitted as standard offers supreme safety under all conditions. Maximum, easily 
con​trollable deceleration using just minimum brake power gives the rider ad​ditional safety especially when applying the brakes in an emergency. 
And to meet many customer requests, the new R 1200 RT comes exclusively with the partly integral ABS version for an even more active style of riding.

Diameter of the front brake discs is 320 millimetres or 12.6´´. To reduce un​sprung masses and allow an even higher standard of brake precision, 
the diameter of the rear brake disc now measures 265 millimetres or 10.4´´, 
as opposed to 276 millimetres or 10.9´´ on the former model.

Woven steel cladding on the brake hoses and brake callipers stiffer than 
on the former model offer not only a slight decrease in weight, but also a more precisely defined, consistently stable brake pressure point. 

On-board network – enhanced range of functions based on CAN-bus technology.

The new on-board network ensures a high standard of reliability plus an even wider range of functions on fewer cables than before: The single-wire system introduced on BMW motorcycles for the first time on the R 1200 GS transmits all relevant data via one single wire to the respective control units. Physically, this is a double-wire system, CAN-bus (Controller Area Network) technology consistently interconnecting all control units and the central vehi​cle elec​tronics (CVE) to form a permanent data network, the various units ex​changing data among one another. Apart from simpler design of the main wiring harness, this also allows efficient, comprehensive diagnosis of all rele​vant vehicle functions. 

A further advantage is that the system no longer requires conventional 
melt-down fuses, CAN-bus technology automatically switching off the component involved whenever necessary and checking whether 
the function is still de​fec​tive when the rider starts his motorcycle again.

The network also serves to feed all data to the cockpit displays fea​tured in the info-flatscreen. To easily adapt electronic accessories and ensure their simple integration in the on-board network, there is also a plug for special equipment.

The low-weight alternator supplying the energy required by the motorcycle has an output of 720 W at 60 Amps. The battery is maintenance-free, offer​ing a capacity of 19 Amp-hours.                               

Electronic immobiliser featured as standard.

BMW Motorrad’s electronic immobiliser is an integral part of the modern on-board network. Making life very difficult for the potential thief, this security system offers the best technology currently available to prevent unauth​ori​sed use of the rider’s machine. The authorised rider, in turn, benefits from a supreme standard of convenience: All you do is pull the key out of the igni​tion, the immobiliser intervening in the engine management software and 
de​activating both fuel injection and the ignition. Then, as soon as you start the engine the next time, a chip in the key compares its data with the engine management data to determine their compliance. Once the proper match has been established, the engine control unit switches on the ignition and the 
rider can start the engine.

Instruments – everything clearly in sight.

The easy-to-read, all-new instrument panel exactly in the rider’s line of vision provides a clear view of all relevant functions in the interest of secure control and absolute clarity at all times. The control panel comprises the speedometer and rev counter in analogue, classic round design as well as an extra-large info-flatscreen placed conveniently right in the middle, with permanent display of the oil temperature, tank level, residual range on the fuel remaining in the tank, the gear currently in mesh, and the current time. And if the motorcycle is equipped with seat heating available as an option, the info-flatscreen also informs the rider of the current heating level. 

An on-board computer complete with an oil level warning unit comes as an option, again providing information shown on the info-flatscreen.       

The LED telltales monitoring all other functions of the motorcycle are 
posi​tioned just above the screen, with instrument illumination masterminded by a photo-cell automatically adjusting to light conditions. 

Headlight unit – clear glass look and free-form technology. 

To meet growing demands for optimum illumination of the road ahead, the main headlight incorporates free-form surface technology featuring 
geo​met​rically optimised surface contours. Two H7 headlights to the right and left of the centrepoint provide a dual low beam, the central bulb gua​rantees 
a powerful and bright high beam. Offering output of 2 x 5 W, the parking light is integrated in the lights unit finished completely in clear glass look and 

featuring a polycarbonate front lens. All components are easily ac​cessible for changing bulbs quickly and conveniently, and the headlight may be adjusted just as conventiently by way of a folding lever while the motor​cycle is at rest. 

Handlebar, controls and electrical switches – proven technology all the way. 

The controls and handles as well as the reservoirs for the hydraulic brake and clutch fluids harmoniously integrated in the motorcycle have been car​ried 
over directly from the former model. A new feature is the lifetime clutch filling, meaning that the rider is never required to top up the fluid level in the clutch reservoir. Grab width on the clutch and brake levers, in turn, is vari​able, 
the R 1200 RT maintaining the specific arrangement of all switches and direction indicator control units typical of BMW.

Only slightly modified and boasting a different surface finish, the handlebar is almost exactly the same as on the former model.              

The body structure – striking looks combined with perfect protection from wind and weather.

The new BMW Tourer is unmistakable even at very first sight, challenging the enthusiast to set out right away. Clear lines and striking surfaces ensure 
a modern and dynamic flair, exuding a powerful touch of class and perfec​tion in conjunction with the motorcycle’s perfect finish. 

The very design of the R 1200 RT accentuates the sheer lightness of the 
ma​chine even exceeding the nimble look of the former model by far, despite the large fairing elements with their perfect protection from wind and weath​er.

Dominating the front end of the R 1200 RT, the headlight with its sharp 
con​tours gives the motorcycle a truly unique look and ensures powerful pre​sence at all times. Extra-large rear-view mirrors integrated in the fairing 
to​geth​er with their direction indicators not only provide excellent visibility to the rear and to all motorists following from behind, but also give the rider’s hands highly effective protection from wind and weather. 

Compared with the windshield on the previous R 1150 RT, the new wind​shield is higher and wider, and once again comes with infinite electrical 
ad​justment. The effective height of the windshield at the upper end may be varied by up to 140 millimetres or 5.5´´, giving riders of virtually any size optimum protection from wind and weather and reducing the noise level even further than before. 

Boasting large and impressive surfaces, the fairing on the R 1200 RT has been re-interpreted in new style. Attractive and skilful subdivision into 
indi​vidual sections arranged on different levels creates a soothing, almost airy picture. This effect is then further enhanced by the colour concept, with 
the engine spoiler and seat bench coming in constrasting colours with an even greater appeal. 

The outer elements of the body, the tank cover and side panels come in three basic colours providing optimum choice: Granite Grey Metallic, Dark Graphite Metallic, and Piemont Red Metallic. The customer can even choose his favourite colour on the engine spoiler (Slate Dark Metallic Matt or White Aluminium Metallic Matt) and on the seat (Grey or Black), supple​ment​ing the basic colour of the motorcycle and thus providing a highly indi​vidual, 
personal touch.

Sophisticated, classic but modern – Granite Grey Metallic offers particular style and class. Dark Graphite Metallic, in turn, emphasizes the premium standard 
of the new R 1200 RT, particularly in conjunction with the engine spoiler in Slate Dark Metallic and the seat in Grey. Piemont Red Metallic, in turn, offers an extra-lively and dynamic look, combined with a special touch of harmony and class. And on request, the seat finished in Grey further ac​centuates the powerful look and striking effect of this equally powerful colour.

The seat and seating position – a perfect combination of long-distance comfort and a firm position on the ground.

The new, highly comfortable seat of the R 1200 RT is split up into two sec​tions, with manual adjustment of the rider’s section for height – either 820 or 840 millimetres (32.3 and, respectively, 33.1´´). A lower seat is available as 
an option, adjusting to a level of either 780 or 800 millimetres (30.7 or 31.5´´).

The particular layout and configuration of the “ergonomics triangle” made up of the footrests, the seat, and handlebar, ensures a particularly active seat​ing position with supreme long-distance riding comfort, giving both the rider and passenger a very comfortable and convenient knee bending angle. With variable seat height offering the rider exactly the right step arch length – a new dimension introduced on the R 1200 GS measuring the distance from one rider’s foot to the other over the seat – the R 1200 RT enables the rider to reach the ground with his feet much more easily and conveniently in the interest of easy manoeuvring. To provide absolute perfection, the step arch length measured considers not only the height of the seat in absolute terms, but also the shape and width of the rider’s seat section.

Equipment features – the side and main stand.

A new main stand and a new side stand complete with its starter interruption function hold the motorcycle securely in place on any surface. Both stands have been specially designed and configured for the R 1200 RT and are easy to operate, allowing the rider to fold out the side stand with ease while still 
on the seat.

Case system and baggage rack.

The BMW R 1200 RT comes as standard with newly designed hard-shell cases. With the case supports being integrated almost completely into 
the body of the motorcycle, they provide a perfect match and perfect looks from every angle. 

High-quality case covers finished in body colour and harmonious design making the cases an integral element of the motorcycle ensure perfect harmony and looks from all perspectives. A strong and secure locking system acting on four different points keeps the case covers exactly in position 
and seals off the contents reliably against water getting in from outside. 
The capacity of each case if 32 litres.

Ample storage space for additional luggage is offered by the newly designed luggage rack serving to accommodate two topcases (capacity either 
49 or 28 litres) available from BMW Motorrad’s wide range of accessories. 
The smaller topcase is finished entirely in Black, the larger features 
a Black lower section with the cover painted in White Aluminium Metallic.

A firmly integrated railing on the top of the tank serves to hold the tankbag especially developed for this purpose securely in position. And a number 
of lashing points for luggage, finally, round off the touring package.

2. Range of Equipment.  

Optional extras and special equipment.

BMW Motorrad’s usual wide range of optional extras and special equipment tailored specifically to the R 1200 RT and, of course, the customer’s 
require​ments, ensures further customisation in every respect. Optional extras come straight from the plant and are integrated in the actual production 
of the mo​tor​cycle on the assembly line in Berlin. Special equipment, in turn, 
is fitted by the BMW motorcycle dealer either from the start or, at the rider’s request, at any later point in time.

Optional extras.    

•
Low rider seat. 

•
Seat heating.

•
Heated handles.

•
Additional power socket.

•
Radio preparation kit.

•
Radio with CD player. 

•
On-board computer with oil level warning unit.

•
Cruise control.

•
Anti-theft warning system.

•
ESA Electronic Suspension Adjustment. 

•
Direction indicators in white.

•
Exhaust system chrome plated.

•
Seats in Grey or Black.           

•
Engine spoiler in Slate Dark Metallic Matt   
or White Aluminium Metallic matt.

On-board computer.

The on-board computer offers additional display functions over and above the standard info-flatscreen display, presenting the outside temperature, 
a black ice warning signal, average fuel consumption, average speed, and an oil level warning. With the oil level being measured at regular intervals, 
the rider receives a warning requesting him to check the oil level as soon as it drops below a certain mark.

Anti-theft warning system.

Should the position of the R 1200 RT while parked be changed or shifted around, manipulated or jolted, the anti-theft warning system will immediately actuate both an optical and an acoustic signal. Wherever appropriate, 
it can then be deactivated by the authorised rider either by remote control or with the key to the motorcycle.

Special equipment.

•
Topcase, large.

•
Topcase, small.

•
Backrest for topcase, small. 

•
Inner bag for topcase, large.

•
Inner bag for topcase, small. 

•
Inner bags for cases, left and right.    

•
Tankbag.     

•
Valve cover protection.

•
Impact protector for cases.

•
Insert for radio socket.

•
Second and third power take-off.

•
Radio with CD player.

•
BMW Motorrad Navigator II.

•
Connection cable and support for BMW Motorrad Navigator II.

•
Service toolkit.        

Topcase.

The topcase comes in two different sizes, the large version offering a ca​pa​city of 49 litres for two helmets and featuring a backrest unit as standard. The small topcase, in turn, offers a capacity of 28 litres for one helmet and is available with an optional backrest. On request, both variants of the topcase come with one key for the cases and motorcycle locks and can be connect​ed 
single-handedly to the luggage rack, without requiring an adapter plate. 

Tankbag.      

The watertight main compartment in the tankbag comes with a capacity of approximately 13 litres and may be enlarged to 19 litres. Beneath the map folder there is an additional pocket for small odds and ends such as your keys, purse, sunglasses, etc. The inner bag section serves again to keep out water, making an additional cover superfluous even in heavy rain. The map section is also watertight, and the tankbag fits easily on to the tank rail​ing featured as standard, requiring the rider to use only one hand in the process.

BMW Motorrad Navigator II.

Reading a map was yesterday. Today all the rider has to do is pick an ad​dress, a city, a special sight or a particular spot on the way, and BMW Mo​torrad Navigator II will take him to his destination safe and sound, pro​viding detailed instructions as he proceeds. In the process Navigator II offers three options for reaching your destination: the fastest route, the shortest, or, as far as possible, the “straight” route “as the crow flies”. 

Map presentation is also variable, offering the choice of a map alone, a com​bination of map and instructions, a roadbook and a compass function showing the rider the distance and direction to his next point on the way. A zoom function, finally, ensures quick and convenient orientation at road junctions, with optional voice messages in nine languages transmitted via the helmet.

3. Engine Output and Torque.   

Specifications.  

R 1200 RT.

	
	
	R 1200 RT
	

	Engine
	
	 
	

	Capacity 
	cc
	1,170
	

	Bore/stroke
	mm
	101/73
	

	Max output
	kW/bhp
	81/110
	

	at 
	rpm
	7, 500
	

	Max torque
	Nm (lb-ft)
	115 ( 85)
	

	at 
	rpm
	6,000
	

	Configuration
	
	Boxer (flat-twin)
	

	No of cylinders
	
	2
	

	Compression ratio/fuel grade
	
	12/premium plus
	

	Valve/gas management
	
	HC (high camshaft)
	

	Valves per cylinder  
	
	4
	

	Intake/outlet valve diameter
	mm
	36/31
	

	Fuel supply
	
	BMS-K
	

	
	
	
	

	Electrical systen  
	
	
	

	Alternator
	W
	720
	

	Battery
	V/Ah
	12/19 maintenance-free
	

	Headlight  
	W
	2 x H7 
	

	Starter
	kW
	1,2
	

	
	
	
	

	Power transmission/gearbox
	
	
	

	Clutch 
	
	Single-plate dry clutch, dia 180 mm
	

	Gearbox
	
	Dog-shift six-speed gearbox
	

	Primary transmission ratio
	
	1.882
	

	Gear ratios 
I
	
	2.277
	

	
II
	
	1.583
	

	
III
	
	1.259
	

	
IV
	
	1.033
	

	
V
	
	0.903
	

	
VI
	 
	0.805
	

	Final drive    
	
	Driveshaft
	

	Final drive transmission ratio
	
	2.62
	

	
	
	
	

	Running gear and suspension
	
	
	

	Frame design
	
	Tubular steel spaceframe, load-bearing engine
	
	

	Wheel guidance, front
	
	BMW Telelever
	
	

	Wheel guidance, rear
	
	BMW Paralever
	
	

	Spring travel, front/rear
	mm
	120/135
	
	

	Castor 
	mm
	109.9
	
	

	Wheelbase
	mm
	1 485
	
	

	Handlebar head angle
	°
	63.4
	
	

	Brakes
	front
	Double disc brake, dia 320 mm
	

	
	rear
	Single disc brake, dia 265 mm
	

	
	
	BMW Motorrad Integral ABS (partly integral) 
fitted as standard
	   

	Wheels
	
	Cast aluminium
	

	
	front
	3.50 x 17
	

	
	rear
	5.00 x 17
	

	Tyres
	front
	120/70 ZR 17
	

	
	rear 
	180/55 ZR 17
	

	
	
	
	

	Dimensions and weights
	
	
	

	Length, overall
	mm
	2,230
	

	Width, overall, with mirrors
	mm
	905
	

	Handlebar width, without mirrors
	mm
	800
	

	Seat height
	mm
	Standard: 820–840/optional: 780–800
	

	DIN unladen weight in road trim
	kg
	(without cases) 259
	

	Dry weight   
	kg
	229
	

	Max permissible 
	kg
	495
	

	Tank capacity
	ltr
	27
	

	
	
	
	

	Performance data
	
	
	

	Fuel consumption 
	
	
	

	90 km/h
	ltr/100 km
	3.6
	

	120 km/h
	ltr/100 km
	4.8
	

	
	
	
	

	Acceleration  
	
	
	

	0–100 km/h
	sec
	3.6
	

	Standing-start km
	sec
	22.6
	

	Top speed            
	km/h
	>200
	


5. Colours.
	
	Colour
	Seat
	

	R 1200 RT
	Granite Grey Metallic
	Black or Grey    
	

	
	Dark Graphite Metallic
	Black or Grey
	

	
	Piemont Red Metallic
	Black or Grey
	






































