[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI Cooper. (May 2007)
	Body
d

Anzahl Türen / s
	
	MINI One 
	MINI One  automatic
	

	No of doors/seats

L
	 
	3/4
	3/4
	

	Length/width/height (unladen) 

)
	mm
	3699 / 1683 / 1407
	3699 / 1683 / 1407 
	

	Wheelbase
	mm
	2 467
	2 467
	

	Track front/rear

wgsdfg


	mm
	1459 / 1467
	1459 / 1467
	

	Turning circle
	m
	10,07
	10,07
	

	Fuel tank capacity
	approx. ltr
	40
	40
	

	Cooling system including heater

l
	ltr
	5,2
	5,2
	

	Engine oil

Getriebeöl


	ltr
	4,2
	4,2
	

	Transmission fluid incl final drive
	ltr
	Permanent filling 
	Permanent filling
	

	Weight, unladen, according to DIN/EU1 
	kg
	1060 / 1135
	1100 / 1175
	

	Max load according to DIN
	kg
	450
	450
	

	Max permissible weight to DIN
	kg
	1510
	1550
	

	Max permissible axle load front/rear
	kg
	850 / 740
	850 / 740
	

	Max permissible trailer load
	
	
	
	

	braked (12%)/unbraked
	kg
	650 / 500
	650 / 500
	

	Max roof load / max towbar download
	kg
	75 / 50
	75 / 50
	

	Luggage compartment according to DIN
	l
	160-680
	160-680
	

	Air resistance
	cX x A
	0,33 x 1,97
	0,33 x 1,97
	

	Engine
d

Anzahl Türen / s
	
	
	
	

	Configuration/No of cylinders/valves Zylinder/Ventile


	 
	Inline/ 4/ 4
	Inline/ 4/ 4
	

	Engine management
	
	MEV 17.2
	MEV 17.2
	

	Capacity
	Cc
	1397
	1397
	

	Bore/stroke
	mm
	77 / 75
	77 / 75
	

	Compression ratio


	:1
	11:1
	11:1
	

	Fuel
	RON
	From 91 ROZ
	From 91 ROZ
	

	Max output
	kW/bhp
	70/95
	70/95
	

	at
	rpm
	6000
	6000
	

	Max torque
	Nm
	140
	140
	

	at
	rpm
	4000
	4000
	

	Electrical system
	kg
	
	
	

	Battery/installation
	kg
	46/ in front
	55/ in front
	

	Alternator
	kg
	120/1680
	120/1680
	

	Chassis and suspension
	
	
	
	

	Front suspension
	
	Singe-joint Mc Pherson spring strut axle with anti-dive
	

	Rear suspension
	
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	

	Brake, front
	
	Disc, vented
	Disc, vented
	

	Diameter
	mm
	280
	280
	

	Brake, rear
	
	Disc brake
	Disc brake
	

	Diameter
	mm
	259
	259
	

	Driving stability systems
	
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with 
Hill Start Assistant, Limited-slip differential
	

	Steering
	
	Electric power steering (EPS)
	

	Overall transmission ratio
	:1 
	14,1
	14,1
	

	Type of transmission
	
	6-gear-manual
	6-gear-automatic
	

	Gear ratios
I


	:1 
	3,214
	4,148
	

	
II
	:1 
	1,792
	2,370
	

	
III
	:1 
	1,194
	1,556
	

	
IV
	:1 
	0,914
	1,155
	

	
V
	:1 
	0,784
	0,859
	

	
VI
	:1
	0,683
	0,686
	

	
R
	:1 
	3,143
	3,394
	

	Final drive ratio
	:1
	4,353
	4.103
	

	Tyres
	 
	175 / 65 R15 84H
	175 / 65 R15 84H
	

	Wheels
	 
	5,5Jx15 Steel
	5,5Jx15 Steel
	

	Performance
	
	
	
	

	Power-to-weight ratio, DIN
	kg/kW
	15,1
	15,7
	

	Output per litre
	kW/ltr
	50,1
	50,1
	

	Acceleration
0–62 mph
	s
	10,9
	12,6
	

	
0–1000 m
	s
	33,3
	34,1
	

	in 4th/5th/6th gear
50–75 mph
	s
	11,6 / 14,8
	-
	

	Top speed
	km/h
	185
	180
	

	Consumption in the EU cycle
	
	
	
	

	Urban
	ltr/100 km
	6,8
	9
	

	Extra-urban
	ltr/100 km
	4,4
	5,0
	

	Combined
	ltr/100 km
	5,3
	6,5

	

	CO2 
	g/km
	128
	155
	

	Miscellaneous
	
	 
	
	

	Emission category
	–
	EU4
	EU4
	

	Ground clearance
	mm
	139
	139
	


1 Weight of car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain conditions.


[image: image1.png][image: image2.png]