	
	
	Media Information

	Date
	
	28 September 2007

	Subject
	
	Statement by Peter Müller, 

Executive Vice President Development and Model Lines BMW Motorrad,

International Motorcycle Show "Mondial Du Deux Roues“

Paris, 28 September 2007     


	Page
	
	2

	
	
	



Media Information

28 September 2007
- Check against delivery -

Company

Bayerische

Motoren Werke

Aktiengesellschaft

Postal Address

BMW AG

80788 München

Telephone

089-382-51376
Internet

www.bmwgroup.com

Statement by Peter Müller, 

Executive Vice President Development and Model Lines BMW Motorrad,

International Motorcycle Show "Mondial Du Deux Roues“

Paris, 28 September 2007     

Ladies and Gentlemen,

Today I would like to present you a very special motorcycle – the HP2 Sport - the street version of our racing boxer.

With racing prototypes of the HP2 Sport at international racing events, we seek to prove the competitiveness of this motorcycle at an early point in time, showing the world what kind of performance our Boxer is able to offer.

At the same time such racing activities were also an integral and essential part of our development process.

According to our definition of HP, high performance stands for the sporting performance, spirit and capability of the entire motorcycle in all its features, and not primarily just for maximum engine power.

It is of course well-known that an air-cooled concept reaches its natural limits here, and cannot offer the same specific output as a water-cooled engine.

Precisely this is why this motorcycle – both the series model and the racing version – comes with an overall concept tailored directly to its specific purpose. 

The HP2 Sport offers its outstanding performance through the overall configuration of the motorcycle.
That is its weight, the power characteristics of the engine in combination with the excellent running gear and perfect brakes. 

With the excellent racing results we achieved in Le Mans,  Oschersleben and Magny Cours, we have already proven what our concept is able to offer.

We are offering the HP2 Sport, the street-legal version of our racing boxer, for the customer who wishes to experience this kind of performance himself.

This truly outstanding motorcycle will be built in a small and exclusive series with virtually all the high-quality and sophisticated components to be admired on the racing version.

There are five features I would like to highlight in particular:

· The self-supporting rear subframe made of carbon-fibre

· The high-performance brake system with its radially bolted monobloc brake callipers

· The dashboard featured only in MotoGP 

· The power shifter

· The new DOHC cylinder heads with direct operation of the valves

The rider’s seat is integrated into the structure of the self-supporting rear subframe made of carbon-fibre. The rear section is absolutely unique in the production of series motorcycles and is featured in this technology for the first time on a series motorcycle.

The entire unit weighs just 2.5 kilos.

Compared with a conventional aluminium rear frame, this reduces the weight of this component alone by 2.1 kilos or 46 per cent. 

The other components on the running gear are modified series parts carried over from the R 1200 S. Examples are the light and torsionally stiff tubular spaceframe, the Paralever swinging arm, and the Telelever further developed to an even higher standard. 

Both wheel mounting points incorporate top-quality Öhlins components.

The Telelever is modified in its connection to the radially bolted brake callipers.

The high-performance monobloc brake callipers come from Brembo and feature four brake pistons.

Brake disc diameter at the front is 320 millimetres. 

The rear wheel features the single-disc brake system carried over from the R 1200 S, with brake disc diameter of 265 millimetres.

As an option, the HP2 Sport is also available with our light Sport ABS from our R 1200 S appropriately modified for sports riding.

You naturally have the opportunity to switch off ABS when racing on the race track. 

Two other features we have carried over from motorsport are the dashboard and the power shifter:  
Focusing on the dashboard, we have chosen a unit from 2D Systems, the manufacturer that also supplies its components to teams in the MotoGP. 

Apart from the usual displays for road use, this dashboard also gives the rider a wide choice of information he is able to save on the race track for subsequent evaluation.   

The gearshift assistant enables the rider to shift up quickly without using the clutch and with hardly the slightest interruption in the flow of power. It does this by briefly taking back the ignition angle and cancelling out the injection in order to shift gears.

The power shifter can also be converted into inverse shifting order.

So far the cylinder heads of our racing bikes were a strict secret. But now I can tell you that for the first time since the legendary vertical drive shaft racing engines on the RS 54 back in the 1950s we now have a Boxer – the HP2 Sport – featuring a cylinder head with two overhead camshafts. 

Our engineers have found a very intelligent technical solution for direct operation of the valves: Positioned horizontally, the camshafts act, respectively, on one intake and one exhaust valve each. 

The valves are operated by means of the light and stiff cam followers already well-known from the K 1200 S. 

Conically polished cams and radial arrangement of the valves in the cylinder head serve to optimise intake ports while maintaining the horizontal flow of gas in the cylinder head.

Both camshafts are driven by a chain with fine links.
We are particularly proud of the fact that despite DOHC control of the valves, the cylinder head is 10 millimetres smaller than on our current boxer engines.

This new valve drive system allows a higher cylinder charge and provides a higher standard of mechanical strength and stiffness, enabling the Boxer to rev reliably beyond 9,000 rpm. The speed limit is 9,500 rpm and the engine reaches maximum power at 8,750 revs.

The further modifications on the engine versus the existing 1200-cc engines involve the chain guide leading to the cylinder heads and modifications in the crankcase, taking the extra power of the engine and the higher engine speed into account.

For reasons of weight, we use light cylinder head covers made of carbon-fibre with exchangeable skid pads for the first time on a series motorcycle. 

The engine's periphery, that is the Digital Motor Electronics with new mapping, the throttle butterflies and intake system all come straight from the R 1200 S. 

The intake funnel has been modified and the intake ducts carefully machined to provide the air throughout required for this kind of power. 

The manifold of the exhaust system has been moved underneath the oil sump for aerodynamics and leaning angle reasons.

The rear muffler has been integrated in its looks and appearance in the self-supporting carbon-fibre rear section.

The exhaust system comes with a flap system to optimise both engine power and sound.

 It almost goes without saying that the exhaust system on our new HP2 Sport comes complete with a three-way catalytic converter and easily outperforms the EU 3 standard required by law.

Maximum output of the production machine is 98 kW or 133 hp. This ensures the right kind of performance without requiring any concessions in terms of the reliability of the air-cooled power unit.

Ladies and Gentlemen,

I'm afraid we don't have enough time to look at the many other refined details on this new motorcycle.

I would however like to highlight the milled fork bridge, the adjustable, short and dynamic handlebar, the milled and adjustable footrests as well as the extremely light fairing made completely of carbon-fibre.

Take your time afterwards to check out the motorcycle from close up and look at all the details.

Naturally, we are at your disposal for any questions.

But before you take a look at the HP2 Sport, let me make an important announcement regarding the future road racing activities of BMW Motorrad.

There has already been a lot of speculation on this point from your side.

Today I am happy to tell you that in 2008 we will be entering the HP2 Sport in further classic endurance races.

In 2009 we will then be opening up a new chapter in the road racing activities of BMW Motorrad, entering the World Super Bike Championship with our own team!

We will inform you of further details such as the motorcycle concept and the structure of the team in good time.

Ladies and Gentlemen,

Thank you very much for your attention.

PAGE  

