[image: image1.png]


BMW
Media
Information

2/210
Valid from 3/2010

Seite 1
BMW
Media
Information

2/2010
Valid from 3/2010


4/2009
Seite 2

Specifications BMW 5 Series Gran Turismo.
535i, 550i
	
	
	BMW 535i Gran Turismo
	Turismo
	BMW 550i GranTurismo TurTurTTurismo

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4998 / 1901 / 1559
	
	4998 / 1901 / 1559

	Wheelbase
	mm
	3070
	
	3070

	Track, front/rear
	mm
	1611 / 1654
	
	1621 / 1636

	Ground clearance
	mm
	145
	
	145

	Turning circle
	m
	12.2
	
	12.2

	Tank capacity
	app ltr
	70
	
	70

	Cooling system incl heater
	ltr
	9.3
	
	11.4

	Engine oil1)
	ltr
	6.5
	
	8.5

	Weight, unladen, to DIN/EU
	kg
	1940 / 2015
	
	2060 / 2135



	Max load to DIN
	kg
	630
	
	630

	Max permissible
	kg
	2570
	
	2690

	Max axle load, front/rear
	kg
	1165 / 1470
	
	1245 / 1500

	Max trailer load (12%),

braked/unbraked
	kg
	2100 / 750
	
	2100 / 750



	Max roofload/max towbar downl
	kg
	75 / 100
	
	75 / 100

	Luggage comp capacity
	ltr
	440 / 590 /1700
	
	440 / 590 / 1700

	Air drag
	Cd  x A
	0.31 x 2.57
	
	0.32 x 2.57

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-six / 4
	
	V8 / 4

	Engine technology
	
	BMW TwinPower Turbo, High Precision Injection, fully variable intake valve management (VALVETRONIC)
	
	BMW TwinPower Turbo, High Precision Injection

	Capacity, effective
	cc
	2979
	
	4395

	Stroke / bore
	mm
	89.6 / 84.0
	
	88.3 / 89.0

	Compression ratio
	:1
	10.2
	
	10.0

	Fuel 
	
	min RON 91
	
	min RON 91

	Max output
	kW/hp
	235 / 306
	
	300 / 407

	at 
	rpm
	5800
	
	5500 – 6400

	Max torque
	Nm/lb-ft
	400/295
	
	600/442

	at
	rpm
	1200–5000
	
	1750-4500

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	90 / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	180 / 2 520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double wishbone axle in aluminium construction, steel springs

	Suspension, rear
	
	Integral-V multi-arm axle in aluminium construction, air suspension with automatic self-levelling, optional Integral Active Steering

(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 36 / vented
	
	374 x 36 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345 x 24 / vented
	
	370 x 24 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, optional: Adaptive Drive incl electronic Dynamic Damping Control and Dynamic Drive anti-roll stability management

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags at the rear on the outer seats, three-point inertia-reel seat belts on all seats, belt limiters and belt latch tensioners at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion power steering with speed-related steering assistance (Servotronic), 3.1 turns from-lock-to lock,  Integral Active Steering

	Steering transmission, overall
	:1
	19.1
	
	19.1

	Tyres, front/rear
	
	245/55 R17 100Y  RSC
	
	245/45 R19 98Y RSC/

275/40 R 19 101 Y RSC



	Rims, front/rear
	
	8J x 18 light-alloy
	
	8.5J x 19 light-alloy/

9.5Jx19 LM

):%Jx!9




Specifications BMW 5 Series Gran Turismo

	
	
	BMW 535i Gran Turismo
	
	BMW 550i Gran Turismo

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including features such as Parkinfo, National Info, Google directory service, News, realtime weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and Adaptive Headlight Range Control, High-Beam Assistant, Park Distance Control, back-up camera, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, Proximity Warning with application of the brakes in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Eight-speed automatic transmission with Steptronic function

	Gear ratios
I
	:1
	4.696
	
	4.696

	                                  
II
	:1
	3.130
	
	3.130

	                                  
III
	:1
	2.104
	
	2.104

	                                  
IV
	:1
	1.667
	
	1.667

	                                  
V
	:1
	1.285
	
	1.285

	                                   
VI
                                        VII                

                                        VIII                                      
	:1
:1
:1              
	1.000
0.839
0.667
	
	1.000
0.839
0.667



	                                   
Reverse
	:1
	3.300
	
	3.300

	Final drive
	:1
	3.077
	
	3.077

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	8.6
	
	6.9

	Output per litre
	kW/hp
	75.5/102.7
	
	68.3/92.9

	Acceleration       0–100 km/h
	sec
	6.3
	
	5.5

	                      Standing-start km
	sec
	26.2
	
	24.4

	Top speed
	km/h
	250
	
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Brake Energy Regeneration, BMW EfficientDynamics Display, Varioserv steering assistance pump, intelligent lightweight construction, on-demand operation of ancillary units, final drive with optimised thermal management, tyres with reduced roll resistance, a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	12.3
	
	16.2

	Extra-urban
	ltr/100 km
	5.9
	
	8.3

	Combined
	ltr/100 km
	8.9
	
	                                            11.2

	CO2
	g/km
	209
	
	263

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1)  Oil change




















































