BMW

Corporate Communications

[image: image1.png]




Technical specifications BMW 320i ETCC 2004.

Vehicle.
Length:
4,490 mm

Width:
1,809 mm 

Height:
approx. 1,415 mm

Wheelbase:
2,742 mm 

Vehicle weight:
1,140 kg (with driver)

Tank capacity:
approx. 45 litres 

Chassis/body:
unitary construction steel body with welded safety cell made of highly rigid precision steel tubing; 
safety fuel tank made of CFRP sandwich;
pneumatic three-stamp rapid jack system 

Aerodynamic 

package: 
front apron, rear apron, rear wings, front wings and flared rear 
  

wheel arches made of CRFP/GFRP

Transmission:              single-disc carbon-fibre clutch operated by a hydraulic central 
  
                             slave cylinder;



five-speed racing transmission with straight interlinked unsynchronised wheel sets based on the production gearbox and additional oil/air cooler; 
quick-shift system with ignition suppression controlled by shifting force;


 
production rear axle differential with mechanical limited-slip differential and additional oil/air cooler 

Front axle:
single-pivot strut suspension with increased wheel caster angle, enlarged track width and enhanced wheel camber compared to the production version; 
four-position adjustable shock absorbers; 
tubular stabiliser bar

Rear axle:
central link-arm axle with double wishbone, with enlarged track width and enhanced wheel camber compared to the production version;
four-position adjustable shock absorbers; 
stabiliser 

Brake system, front:
four-piston brake callipers made of aluminium;
inner-vented grey cast iron brake discs of 295 mm diameter

Brake system, rear:
two-piston brake callipers of aluminium;
grey cast iron brake discs of 280 mm diameter

Steering:
rack-and-pinion steering with electric-hydraulic power assistance

Wheels:
aluminium rims, 9 x 17 inches

Tyres:
Michelin race tyres, 24/61-17 (front and rear)

BMW P54 engine.

Type:
six-cylinder in-line

Capacity:
1,990 cc

Bore x stroke:
80 x 66 mm

Compression:
11 : 1

max. output:
approx. 270 bhp at approx. 8,800 rpm

max. torque:
approx. 230 Nm at approx. 7,000 rpm

max. engine speed:
9,000 rpm (as per regulations)

Cylinder block:
single-cylinder block construction made of aluminium with 
inserted grey cast iron liners

Crankshaft:
steel crankshaft with reduced bearing diameters 

Pistons:
forged skirt pistons

Conrod:
high-performance steel

Cylinder head:
aluminium construction with parallel drive valve arrangement; 
four valves per cylinder

Valve drive:
two overhead chain-driven camshafts;  
mechanical bucket tappets

Intake system:
single-throttle valve (64 mm, as per regulations); air box made of CFRP/GFRP with load change-optimised ram tubes

Exhaust system:
high-performance header, silencer and catalytic converter

Fuel system:
single-cylinder multipoint injection with one injection valve per 
cylinder; injection pressure 5.0 bar

Lubrication:
wet sump lubrication with optimised oil sump/oil suction system

Cooling:
water/air cooler and oil/water heat exchanger

Flywheel:
steel, weight-optimised to 4 kg

Fuel:
standard fuel, Super Plus unleaded

Electrical system/electronics.
Engine control:
BMW Motorsport ECU 12A-R6 engine control unit with three high-performance microprocessors; 
individual cylinder injection and ignition; 
pit speed limiter function, quick shift function; 
engine data memory system

Coil:

high-performance pencil coils with integrated ignition drivers

Spark plugs:
Champion high-performance spark plugs

Cockpit:
programmable LCD display with integrated indicators

Steering wheel: 
quick-release multifunction steering wheel with integrated display control

Press contact: Heike Bartsch, BMW Corporate Communications Motorsport, 
Mobile: +49 (0)172 84 85 998, tel. office: +49-(0)89–382 33115, heike.bartsch@bmw.de


BMW press releases and photos for media use are available at www.press.bmw.com.







[image: image1.png]

