–4–

IMMEDIATE

David J. Buchko

BMW Product Communications Manager

201-307-3789 / dave.buchko@bmwna.com

William Scully
BMW Product Communications Specialist

201-307-3790 / william.scully@bmwna.com

2005 BMW 6 Series

Technical Specifications

General
645Ci
645Ci


Coupe
Convertible

Curb weight, lb.:


Manual transmission
3781
4178


Sequential Manual Gearbox


  (SMG)
3792
4189


Automatic transmission
3792
4189

Weight distribution, front/rear, %:


Manual transmission
51.6/48.4
48.3/51.7


Sequential Manual Gearbox


  (SMG)
51.7/48.3
48.4/51.6


Automatic transmission
51.7/48.3
48.4/51.6

GVWR, lbs
4619

Wheelbase, in.
109.4 1
Track, front/rear, in.
61.3/62.7 1, 

Length, in.
190.2 1

Width, in.
73.0 1

Height, in.
54.1 1

Body
645Ci
645Ci


Coupe
Convertible

Type
2-door coupe
2-door convertible

Aerodynamic drag coefficient
0.30
0.32

EPA size classification
Subcompact
Subcompact

Accommodations
Seating capacity, persons
4
4

Shoulder room, front/rear, in.
56.4/49.4 1


Head room, front/rear, in.
37.3/35.5
37.8/36.7 

Leg room, front/rear, in.
42.0/28.8 1

EPA interior volume, cu ft.
81.0
82.0

EPA cargo volume, cu ft.
13.0
12.4/10.6 

Engine & electrical

Engine type
DOHC 32-valve (4-cam) V-8, Valvetronic



variable valve lift & Double VANOS 
 steplessly



variable intake- & exhaust-valve timing 1

Bore x stroke, mm/in.
92.0 x 82.7/3.62 x 3.26 1

Displacement, cc/cu in.
4398/268 1

Compression ratio
10.5:1 1

Power @ rpm, horsepower
325 @ 6100 1

Torque @ rpm, lb-ft.
330 @ 3600 1

Engine-management system
Motronic ME9 with knock control; variable



valve lift & timing, electronic throttle system,



steplessly variable intake system, engine cooling



& other functions included in control strategy 1

Fuel requirement
Premium unleaded 1

Fuel capacity, U.S. gallons
18.5 1

Battery capacity, amp-hr.
110 1
Alternator output rating, amp./W
170/2380 1

Drivetrain
645Ci
645Ci


Coupe
Convertible

Drive system
Front engine/rear-wheel drive 1

Manual transmission
ZF Type G, 6-speed 1

Ratios:
1st
4.05:1 1



2nd
2.40:1 1



3rd
1.58:1 1



4th
1.19:1 1



5th
1.00:1 1



6th
0.87:1 1



Reverse
3.68:1 1

Final drive ratio
3.23:1
3.46:1
Sequential Manual Gearbox
Electrohydraulic/electronic controls applied to




6-speed manual transmission (above);




transmission & final drive ratios same as for




manual transmission 1

Automatic transmission
ZF 6 HP 26, 6-speed STEPTRONIC with Adaptive




Transmission Control, selectable Sport & Manual




modes 1

Ratios:
1st
4.17:1 1



2nd
2.34:1 1


3rd
1.52:1 1


4th
1.14:1 1


5th
0.87:1 1


6th
0.69:1 1


Reverse
3.40:1 1
Final drive ratio
3.46:1 1

Chassis
645Ci
645Ci


Coupe
Convertible

Body/frame construction
Unitized; aluminum front-end structure, aluminum



hood & doors, thermoplastic front fenders,



Sheet Molding Compound (SMC) trunklid 1
Front suspension
Struts, double-pivot lower arms, coil springs,



twin-tube gas-pressure shock absorbers, Active



Roll Stabilization; aluminum suspension system,



thrust plate & subframe 1 (Convertible: additional



diagonal braces)

Rear suspension
4-link Integral suspension, coil springs, twin-tube



gas-pressure shock absorbers, Active Roll Stabili-



zation; aluminum suspension system & subframe 1



(Convertible: additional diagonal braces)

Steering type
Rack & pinion, Servotronic vehicle-speed-sensitive



variable power assist 1

Overall steering ratio
Mechanically variable; mean ratio is 14.4:1 1

Overall ratio, Active Steering
Electrically/electronically variable over wide range


(Sport Package)
of 10:1 – 18:1 1

Turning circle, ft.
37.4 1
Brakes
4-wheel ventilated discs with lightweight aluminum/



cast-iron rotors, aluminum front/ cast-iron rear



calipers 1

Diameter, front, mm/in.
348/13.7 1

Diameter, rear, mm/in.
345/13.6 1
Wheels (standard)
Cast alloy, 18 x 8.0 1
Wheels (optional)
Cast alloy, 19 x 8.5 front/19 x 9.0 rear (Sport Package)1
Tires (standard)
245/45R-18 V-rated run-flat all-season 1
Tires (optional)
245/40R-19 front / 275/35R-19 rear



W-rated run-flat performance (Sport Package) 1
Stability-enhancement system
Dynamic Stability Control (DSC); includes Dynamic



Traction Control, electronic brake proportioning,



antilock braking (ABS), Dynamic Brake Control



& cornering/braking stability enhancement 1
Performance data
645Ci
645Ci


Coupe
Convertible
Acceleration, 0-60 mph, sec. 
:


6-Spd Manual Transmission
5.5
6.0


6-Spd Sequential Manual Gearbox


  (SMG)
5.5
6.0


6-Spd Automatic Transmission
5.7
6.1

EPA estimated fuel economy, MPG, city/highway:


6-Spd Manual Transmission
17/25
15/23


6-Spd Sequential Manual Gearbox


  (SMG)
16/24
15/22


6-Spd Automatic Transmission
18/26
18/26

� – Specification applies to both models.


� – With standard wheels.


� – With softtop raised.


� – Softtop raised/lowered, variable softtop storage compartment adjusted accordingly.


� – VANOS = VAriable NOckenwellen Steuerung = variable camshaft control, or variable valve timing.


� BMW AG test results. Actual acceleration results may vary depending on specification of vehicle; road and environmental conditions; testing procedures and driving style. These results should be used for comparison only, and verification should not be attempted on public roads. BMW urges you to obey all posted speed limits and to please wear your safety belt at all times.





–more–
–more–


