[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 6 Series Convertible.
630i, 650i, 635d
	
	
	BMW 630i Convertible
	BMW 650i Convertible
	BMW 635d Convertible

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 2+2
	2 / 2+2
	2 / 2+2

	Length/width/height (unladen)
	mm
	4820 / 1855 / 1374
	4820 / 1855 / 1374
	4820 / 1855 / 1374

	Wheelbase
	mm
	2780
	2780
	2780

	Track, front/rear
	mm
	1558 / 1596
	1558 / 1596
	1558 / 1596

	Ground clearance
	mm
	129
	129
	129

	Turning circle
	m
	11.4
	11.4
	11.4

	Tank capacity
	approx ltr
	70
	70
	70

	Cooling system incl heater
	ltr
	10.0 (10.5)
	13.8 (14.2)
	9.8

	Engine oil1)
	ltr
	6.5
	8.0
	8.5

	Weight, unladen, to DIN/EU
	kg
	1740 / 1815

(1750 / 1825)


	1860 / 1935

(1860 / 1935)


	1860 / 1935



	Max load to DIN
	kg
	400
	400
	400

	Max permissible
	kg
	2140 (2150)
	2260 (2260)
	2260

	Max axle load, front/rear
	kg
	1020 / 1210
	1100 / 1250
	1100 / 1250

	Max trailer load (12%),
braked/unbraked
	kg
	-


	-


	-



	Max roofload/towbar download
	kg
	-
	-
	-

	Luggage comp capacity
	ltr
	300-350
	300-350
	300-350

	Air resistance
	cD x A
	0.31 x 2.15
	0.31 x 2.15
	0.31 x 2.15

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	V90 / 8 / 4
	Straight-6 / 4

	Engine technology
	 
	Second-generation 

direct injection (High Precision Injection) 

with lean burn
	Fully variable

valve management

(VALVETRONIC), 

double-VANOS 
	Common-rail 

direct injection, 

variable Twin Turbo

	Capacity, effective
	cc
	2996
	4799
	2993

	Stroke/bore
	mm
	88.0 / 85.0
	88.3 / 93.0
	90.0 / 84.0

	Compression ratio
	:1
	12.0
	10.5
	17.0

	Fuel grade
	
	min RON 91
	min RON 91
	Diesel

	Max output
	kW/hp
	200 / 272
	270 / 367
	210 / 286

	at 
	rpm
	6700
	6300
	4400

	Max torque
	Nm / lb-ft
	320 / 236
	490 / 361
	580 / 427

	at 
	rpm
	2750-3000
	3400
	1750-2250

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	     90 / luggage compartm compartment
	90 / luggage compartment
	90 / luggage compartment

	Alternator
	A/W
	155 / 2170
	180 / 2520
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint tiebar spring strut axle, aluminium, 

compensation of lateral forces, anti-dive

	Suspension, rear
	
	Integral-IV multi-arm axle, aluminium, multi-dimensional suspension
with anti-squat and anti-dive, air suspension with
automatic self-levelling

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 30 / vented
	348 x 36 / vented
	348 x 36 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345 x 24 / vented
	345 x 24 / vented
	345 x 24 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, 
Fading Compensation, Start-Off Assistant, optional Dynamic Drive

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator

	Steering
	
	Rack-and-pinion steering with Servotronic; optional: Active Steering

	Transmission ratio, overall
	:1
	14.4
	14.4
	14.4

	Tyres, front/rear
	
	245/50 R17 99W RSC
	245/45 R18 96W RSC
	245/50 R17 99W RSC

	Rims, front/rear
	
	7.5J x 17 light-alloy
	8J x 18 light-alloy
	7.5J x 17 light-alloy


Specifications BMW 6 Series Convertible.
	
	
	BMW 630i Convertible
	BMW 650i Convertible
	BMW 635d Convertible

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,
Park Distance Control, BMW Night Vision, Head-Up Display,
Active Cruise Control with Stop & Go, Lane Departure Warning, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type
	
	Six-speed manual gearbox (optional: six-speed automatic with Steptronic)
	Six-speed automatic with Steptronic

	Gear ratios
I
	:1
	4.350 (4.171)
	4.055 (4.171)
	4.171

	                                  
II
	:1
	2.496 (2.340)
	2.396 (2.340)
	2.340

	                                  
III
	:1
	1.665 (1.521)
	1.582 (1.521)
	1.521

	                                  
IV
	:1
	1.230 (1.143)
	1.192 (1.143)
	1.143

	                                  
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	0.867

	                                   
VI
	:1
	0.851 (0.691)
	0.872 (0.691)
	0.691

	                                   
R
	:1
	3.926 (3.403)
	3.677 (3.403)
	3.403

	Final drive ratio
	:1
	3.385 (3.909)
	3.462 (3.385)
	3.154

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	8.7 (8.8)
	6.9
	8.9

	Output per litre
	kW/hp
	66.8 / 90.8
	56.3 / 76.6
	70.2 / 95.5 

	Acceleration
     0–100 km/h
	sec
	6.7 (7.0)
	5.5 (5.6)
	6.6

	Standing-start km
	sec
	26.6 (26.8)
	24.5 (24.7)
	26.2

	In 4th/5th gear          80–120 km/h
	sec
	7.3 (-)
	4.9 (-)
	-

	Top speed
	km/h
	250
	250
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), intelligent lightweight construction, on-demand control of ancillary units, Varioserv steering assistance pump (BMW 630i, BMW 635d), a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	11.8 (11.6)
	19.2 (16.5)
	9.6

	Extra-urban
	ltr/100km
	6.3 (6.0)
	8.8 (7.7)
	5.8

	Combined
	ltr/100km
	8.3 (8.1)
	12.6 (10.9)
	7.2

	CO2
	g/km
	198 (192)
	299 (258)
	190

	Emission rating
	
	EU4  
	EU4  
	EU4  


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































