[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 5 Series Touring.
525i xDrive, 530i xDrive
	
	
	BMW 525i xDrive
Touring
	
	BMW 530i xDrive
Touring

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	5 / 5
	
	5 / 5

	Length/width/height (unladen)
	mm
	4843 / 1846 / 1491
	
	4843 / 1846 / 1491

	Wheelbase
	mm
	2886
	
	2886

	Track, front/rear
	mm
	1558 / 1581
	
	1558 / 1581

	Ground clearance
	mm
	143
	
	143

	Turning circle
	m
	11.9
	
	11.9

	Tank capacity
	approx ltr
	70
	
	70

	Cooling system incl heater
	ltr
	10.0 (10.5)
	
	10.0 (10.5)

	Engine oil1)
	ltr
	6.5
	
	6.5

	Weight, unladen, to DIN/EU
	kg
	1720 / 1795

(1740 / 1815)
	
	1725 / 1800

(1735 / 1810)

	Max load to DIN
	kg
	600
	
	600

	Max permissible
	kg
	2320 (2340)
	
	2325 (2335)

	Max axle load, front/rear
	kg
	1050 / 1300
	
	1050 / 1300

	Max trailer load (12%),
braked/unbraked
	kg
	1800 / 2000 

(2000) / 750
	
	2000 / 750

	Max roofload/towbar download
	kg
	100/90
	
	100/90

	Luggage comp capacity
	ltr
	500-1650
	
	500-1650

	Air resistance
	cD x A
	0.31 (0.30) x 2.28
	
	0.31 (0.30) x 2.28

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	
	Straight-6 / 4

	Engine technology
	 
	Second-generation 

direct injection (High Precision Injection) 

with lean burn
	
	Second-generation 

direct injection (High Precision Injection) 

with lean burn

	Capacity, effective
	cc
	2996
	
	2996

	Stroke/bore
	mm
	88.0 / 85.0
	
	88.0 / 85.0

	Compression ratio
	:1
	12.0
	
	12.0

	Fuel grade
	
	min RON 91
	
	min RON 91

	Max output
	kW/hp
	160 / 218
	
	200 / 272

	at 
	rpm
	6100
	
	6700

	Max torque
	Nm / lb-ft
	270 / 199
	
	320 / 236

	at 
	rpm
	2400-4200
	
	2750-3000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	     80 / luggage compartm compartment
	
	80 / luggage compartment

	Alternator
	A/W
	155 / 2170
	
	155 / 2170

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint tiebar spring strut axle, aluminium, 

compensation of lateral forces, anti-dive

	Suspension, rear
	
	Integral-IV multi-arm axle, aluminium, multi-dimensional suspension
with anti-squat and anti-dive, air suspension with
automatic self-levelling

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	324 x 30 / vented
	
	324 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 20 / vented
	
	320 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, interaction of DSC and xDrive, HDC Hill Descent Control

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator, optional: side airbags and belt latch tensioners at the rear

	Steering
	
	Rack-and-pinion steering with Servotronic

	Transmission ratio, overall
	:1
	14.58
	
	14.58

	Tyres, front/rear
	
	225/50 R17 94Y
	
	225/50 R17 94Y

	Rims, front/rear
	
	7.5J x 17 light-alloy
	
	7.5J x 17 light-alloy


Specifications BMW 5 Series Touring.
	
	
	BMW 525i xDrive
Touring
	
	BMW 530i xDrive
Touring

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, Integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,
Park Distance Control, BMW Night Vision, Head-Up Display,
Active Cruise Control with Stop & Go, Lane Departure Warning, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic with Steptronic)

	Gear ratios
I
	:1
	4.350 (4.171)
	
	4.350 (4.171)

	                                  
II
	:1
	2.496 (2.340)
	
	2.496 (2.340)

	                                  
III
	:1
	1.665 (1.521)
	
	1.665 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	
	1.230 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (0.867)

	                                   
VI
	:1
	0.851 (0.691)
	
	0.851 (0.691)

	                                   
R
	:1
	3.926 (3.403)
	
	3.926 (3.403)

	Final drive ratio
	:1
	3.231 (3.909)
	
	3.385 (3.727)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	10.8 (10.9)
	
	8.6 (8.7)

	Output per litre
	kW/hp
	53.4 / 72.6
	
	66.8 / 90.8

	Acceleration
    0–100 km/h
	sec
	8.2 (8.5)
	
	6.9 (7.1)

	Standing-start km
	sec
	28.3 (28.5)
	
	26.9 (27.1)

	In 4th/5th gear         80–120 km/h
	sec
	9.2 (-)
	
	7.4 (-)

	Top speed
	km/h
	233 (230)
	
	246 (244)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), tyres with reduced roll resistance, intelligent lightweight construction, on-demand control of ancillary units, air flap control, Varioserv steering assistance pump,
a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	12.0 (11.8)
	
	12.3 (12.4)

	Extra-urban
	ltr/100km
	6.6 (6.7)
	
	6.6 (6.4)

	Combined
	ltr/100km
	8.6 (8.6)
	
	8.7 (8.6)

	CO2
	g/km
	203 (204)
	
	205 (204)

	Emission rating
	
	EU4  
	
	EU4  

	
	
	
	
	


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































