[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 3 Series Convertible.
320i, 325i 
	
	
	BMW 320i 
Convertible
	
	BMW 325i 
Convertible

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	
	2 / 4

	Length/width/height (unladen)
	mm
	4580 / 1782 / 1384
	
	4580 / 1782 / 1384

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1500 / 1513
	
	1500 / 1513

	Ground clearance
	mm
	138
	
	138

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
	approx ltr
	63
	
	63

	Cooling system incl heater
	ltr
	8.4 (9.1)
	
	8.2 (8.5)

	Engine oil1)
	ltr
	4.25
	
	6.5

	Weight, unladen, to DIN/EU
	kg
	1595 / 1670

(1625 / 1700)
	
	1655 / 1730

(1685 / 1760)

	Max load to DIN
	kg
	430
	
	430

	Max permissible
	kg
	2025 (2055)
	
	2085 (2115)

	Max axle load, front/rear
	kg
	915 / 1155
	
	945 / 1180

	Max trailer load (12%),
braked/unbraked
	kg
	1400 (1500) / 695
	
	1600 / 745

	Max roofload/towbar download
	kg
	- / 75
	
	- / 75 

	Luggage comp capacity
	ltr
	210-350
	
	210-350

	Air resistance
	cD x A
	0.27 x 2.08
	
	0.28 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-4 / 4
	
	Straight-6 / 4

	Engine technology
	 
	Second-generation 

direct injection (High Precision Injection) 

with lean burn
	
	Second-generation 

direct injection (High Precision Injection) 

with lean burn 

	Capacity, effective
	cc
	1995
	
	2996

	Stroke/bore
	mm
	90.0 / 84.0
	
	88.0 / 85.0

	Compression ratio
	:1
	12.0
	
	12.0 

	Fuel grade
	
	min RON 91
	
	min RON 91

	Max output
	kW/hp
	125 / 170
	
	160 / 218

	at 
	rpm
	6700
	
	6100

	Max torque
	Nm / lb-ft
	210 / 155
	
	270 / 199

	at 
	rpm
	4250
	
	2400-4200

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	90 / luggage compartment
	
	90 / luggage compartment

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint spring strut axle, aluminium, displaced castor, 

small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel construction

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	312 x 24 / vented
	
	330 x 24 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	
	300 x 20 / vented 

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), 

CBC Cornering Brake Control, DBC Dynamic Brake Control, BMW 325i also with Dry Braking, Fading Compensation, Start-Off Assistant

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger combined with head airbags, three-point inertia-reel seat belts on all seats, with belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, rollover safety system, tyre defect indicator

	Steering
	
	Rack-and-pinion power steering

	Transmission ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	225/45 R17 91W RSC
	
	225/45 R17 91W RSC

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J x 17 light-alloy


Specifications BMW 3 Series Convertible.
	
	
	BMW 320i
Convertible
	
	BMW 325i
Convertible

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, 

High-Beam Assistant, Park Distance Control, Cruise Control, 

Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.323 (4.171)
	
	4.323 (4.171)

	                                  
II
	:1
	2.456 (2.340)
	
	2.456 (2.340)

	                                  
III
	:1
	1.659 (1.521)
	
	1.659 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	
	1.230 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (0.867)

	                                   
VI
	:1
	0.848 (0.691)
	
	0.848 (0.691)

	                                   
R
	:1
	3.938 (3.403)
	
	3.938 (3.403)

	Final drive ratio
	:1
	3.636 (4.100)
	
	3.154 (3.636)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	12.8 (13.0)
	
	10.3 (10.5)

	Output per litre
	kW/hp
	62.7 / 85.3
	
	53.4 / 72.6

	Acceleration
 0–100 km/h
	sec
	9.1 (9.8)
	
	7.6 (8.1)

	Standing-start km
	sec
	30.0 (30.5)
	
	28.1 (28.5)

	In 4th/5th gear        80–120 km/h
	sec
	9.3 / 12.0 (-)
	
	8.3 / 10.5 (-) 

	Top speed
	km/h
	228 (226)
	
	245 (243)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Auto Start Stop Function (BMW 320i manual model) and gearshift point indicator (manual model), tyres with reduced roll resistance, intelligent lightweight construction, oil pump with map control, 

a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	8.8 (9.8) 
	
	10.2 (10.6)

	Extra-urban
	ltr/100km
	5.6 (5.8)
	
	5.9 (6.3) 

	Combined
	ltr/100km
	6.8 (7.3)
	
	7.5 (7.9) 

	CO2
	g/km
	159 (169)
	
	176 (185)

	Emission rating
	
	EU5
	
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































