[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 3 Series Convertible.
330i, 335i 
	
	
	BMW 330i 
Convertible
	
	BMW 335i 
Convertible

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	
	2 / 4

	Length/width/height (unladen)
	mm
	4580 / 1782 / 1384
	
	4580 / 1782 / 1384

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1500 / 1513
	
	1500 / 1507

	Ground clearance
	mm
	138
	
	138

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
	approx ltr
	63
	
	63

	Cooling system incl heater
	ltr
	8.2 (8.5)
	
	8.2

	Engine oil1)
	ltr
	6.5
	
	6.5

	Weight, unladen, to DIN/EU
	kg
	1705 / 1780

(1720 / 1795)
	
	1735 / 1810

(1750 / 1825)

	Max load to DIN
	kg
	430
	
	430

	Max permissible
	kg
	2135 (2150)
	
	2165 (2180)

	Max axle load, front/rear
	kg
	960 / 1200
	
	990 / 1200

	Max trailer load (12%),
braked/unbraked
	kg
	1700 / 750
	
	1700 / 750

	Max roofload/towbar download
	kg
	- / 75
	
	- / 75 

	Luggage comp capacity
	ltr
	210-350
	
	210-350

	Air resistance
	cD x A
	0.28 x 2.08
	
	0.31 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	
	Straight-6 / 4

	Engine technology
	 
	Second-generation 

direct injection (High Precision Injection) 

with lean burn
	
	BMW Twin Turbo,

second-generation 

direct injection

(High Precision Injection)  

	Capacity, effective
	cc
	2996
	
	2979

	Stroke/bore
	mm
	88.0 / 85.0
	
	89.6 / 84.0

	Compression ratio
	:1
	12.0
	
	10.2 

	Fuel grade
	
	min RON 91
	
	min RON 95

	Max output
	kW/hp
	200 / 272
	
	225 / 306

	at 
	rpm
	6700
	
	5800

	Max torque
	Nm / lb-ft
	320 / 236
	
	400 / 295

	at 
	rpm
	2750-3000
	
	1300-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	90 / luggage compartment
	
	90 / luggage compartment

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint spring strut axle, aluminium, displaced castor, 

small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel construction

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 30 / vented
	
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	336 x 22 / vented
	
	336 x 22 / vented 

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), 

CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger combined with head airbags, three-point inertia-reel seat belts on all seats, with belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, rollover safety system, tyre defect indicator

	Steering
	
	Rack-and-pinion power steering

	Transmission ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	225/45 R17 91Y RSC
	
	225/45 R17 91W RSC / 255/40 R17 94 W RSC 

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J/8.5J x 17 light-alloy


Specifications BMW 3 Series Convertible.
	
	
	BMW 330i
Convertible
	
	BMW 335i
Convertible

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, 

High-Beam Assistant, Park Distance Control, Cruise Control, 

Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic,

BMW 335i: seven-speed double-clutch)

	Gear ratios
I
	:1
	4.350 (4.171)
	
	4.055 (4.780) 

	                                  
II
	:1
	2.496 (2.340)
	
	2.396 (3.056)

	                                  
III
	:1
	1.665 (1.521)
	
	1.582 (2.153)

	                                  
IV
	:1
	1.230 (1.143)
	
	1.192 (1.678)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (1.390)

	                                   
VI
	:1
	0.851 (0.691)
	
	0.872 (1.203)

	                                   
VII
	:1
	
	
	(1.000)

	                                        R
	:1
	3.926 (3.403)
	
	3.677 (4.454)

	Final drive ratio
	
	3.154 (3.727)
	
	3.077 (2.563)

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	8.5 (8.6)
	
	7.7 (7.8)

	Output per litre
	kW/hp
	66.8 / 90.8
	
	75.5 / 102.7

	Acceleration
      0–100 km/h
	sec
	6.5 (6.7)
	
	5.8 (5.7)

	Standing-start km/h
	sec
	26.4
	
	25.2 (25.1)

	In 4th/5th gear           80–120 km/h
	sec
	7.1 / 9.0 (-)
	
	5.9 / 7.1 (-)

	Top speed
	km/h
	250
	
	250 

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), on-demand control of ancillary units, intelligent lightweight construction, tyres with reduced roll resistance, oil pump with map control, 

a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	10.5 (11.1) 
	
	13.6 (12.8)

	Extra-urban
	ltr/100km
	6.2 (6.5)
	
	7.1 (7.0) 

	Combined
	ltr/100km
	7.8 (8.2)
	
	9.5 (9.1) 

	CO2
	g/km
	182 (190)
	
	226 (217)

	Emission rating
	
	EU5
	
	EU4  

	
	
	
	
	


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































