	
	
	Press Information

	Datum
	
	01.10.2009

	Thema
	
	

	Seite
	
	2

	
	
	


[image: image2.png]



BMW
Corporate Communications

BMW
Corporate Communications
[image: image1.png]



Press Information
01.10.2009
Company
Bayerische

Motoren Werke

Aktiengesellschaft

Postal Adress
BMW AG

80788 München

Telefon

+49 89 382-0

Internet

www.bmwgroup.com

The new BMW G 450 X.

Fine-tuned for even more competitive potential.
BMW Motorrad has set yet another milestone in the company's 80-year offroad tradition. It is the first uncompromising, pure-blooded sports enduro to be produced in the popular 450cc class and to have been developed especially for enduro use, fit for taking part in the most demanding contests in the world.

Meanwhile BMW Motorrad can look back proudly at the numerous sporting successes it has achieved with the G 450 X. Preliminary highlight was the second place in the WEC, short hand for world enduro championship, for Juha Salminen in class E2 enabled by top-professional factory entries over the course of an entire season. As well as chalking up several sporting successes in the German Cross Country Championships, the Erzberg Rodeo, and most recently, the Red Bull Romaniacs. At this event in Rumania, Andreas Lettenbichler won through decisively with his G 450 X against an entire legion of worldwide motorcycling elite at a legendary City Prolog event. Four racing days in the Carpathians, with extremely steep uphill and downhill slopes, fast rally-style sections, and difficult and selective single trails through dark forests. To cap it all was a veritable daredevil ride on the roof of a five-storey building in the centre of the city of Sibiu.

And now, numerous technical optimisations have been planned for the BMW G 450 X in the model year 2010. For instance, a new dataset has been developed for the ECU (engine control unit) of the bike's digital engine management system. This results not only in an improvement in fuel consumption but also a reduction in engine break torque, improved start characteristics and more sensitive responses, particularly with load changes. 

In addition to the usual standard features, an optional extra is available in the form of a slip-on racing silencer (without ABE and not homologated for on-road use), which serves to further enhance the technical concept of the G 450 X as a pure-blooded, uncompromising sports enduro. Its main benefit compared to the standard silencer is that it is around one kilogram lighter. The stainless steel manifold, designed as a combined reflective and absorptive silencer, fulfils FIM regulations, is easy to assemble, and reduces the G 450 X's dry weight in competition conditions to no more than 110 kilograms.

The chassis of the BMW G 450 X has also received a number of modifications. The offset of the front axle to the middle of the fork rod has been reduced by 2.5 millimetres, resulting in more neutral handling with optimum directional stability. 

Furthermore, the new of the upside-down fork has been recalibrated with a stronger damping progression, resulting in more sensitive response characteristics and improvements in overcoming bumps, even in very tough off-road terrain. 

The calibration of the rear strut has also been modified, with the result that its damping characteristic now also displays a more progressive line, favouring more sensitive response characteristics. Ergonomics have been improved with a new seat design; it is now filled with soft foam and also displays an enlarged radius in the upper thigh area. The new design features increased comfort and affords the rider increased control over his machine.

The design of the new G 450 X is oriented towards the successful factory racing models of the BMW Motorrad Motorsport team, with matching, gold-anodised fork inner tubes. 

The market launch of the new BMW G 450 X has been set for October 2009. 
Technical Spezifications.
G 450 X, Model Year 2010.
	
	
	BMW G450  X 
	
	
	

	Engine
	
	
	
	
	

	capacity
	cm3
	449,5
	
	
	

	bore x stroke
	mm
	98/59,6
	
	
	

	rated output
	kW/PS
	ECE 30/40,8
	open 38/51,7
	
	

	at
	min–1
	7 000 
	9000
	
	

	max. torque
	Nm
	42,8
	44
	
	

	at
	min–1
	6500 
	7800
	
	

	compression ratio
	
	12:1/ unleaded Super(95 ROZ) 
	
	
	

	valve/gas control
	
	DOHC (double overhead  camshaft)
	
	
	

	mixture control
	
	electronic intake pipe injection, Keihin digital engine management   
	
	
	

	emission control
	
	elosed-loop 3-way catalytic converter
	
	
	

	
	
	
	
	
	

	Electrics
	
	
	
	
	

	alternator
	W
	280
	
	
	

	battery
	V/Ah
	12/7  maintenance free
	
	
	

	
	
	
	
	
	

	Power transmission/ gearbox
	
	
	
	
	

	clutch
	
	multiple disc clutch in oil bath, primary clutch sits directly on the  crankshaft, mechanically operated               
	
	

	gearbox
	
	synchromesh 5-speed gearbox built into crankcase 
	
	
	

	rear-wheel drive
	
	chain 
	
	
	

	
	
	
	
	
	

	Chassis
	
	
	
	
	

	frame bridge-type
	
	
frame made of stainless steel tubing
	
	

	front wheel location/suspension
	
	Marzocchi upside-down telescopic fork, diameter on fork inner tube 45mm
	
	
	

	rear wheel suspension  
	
	die-cut aluminium double swing arm
	
	
	

	strut
	
	Öhlins hydraulic strut
	
	
	

	travel front/rear
	mm
	300/320
	
	
	

	wheelbase
	mm
	1475
	
	
	

	brakes
	front
	single disc, diamter Ø 260 double piston floating calliper
	
	
	

	
	rear
	
single disc, diamter  Ø 220 single piston floating calliper
	
	
	

	wheels
	
	spoked
	
	
	

	
	front
	1,60 x 21“
	
	
	

	
	rear
	2,15 x 18“
	
	
	

	tyres
	front
	90/90-21
	
	
	

	
	rear
	140/80-18
	
	
	

	
	
	
	
	
	

	Dimensions and weights
	
	
	
	
	

	length
	mm
	2200
	
	
	

	width incl. mirrors
	mm
	806
	
	
	

	height excl. mirrors 
	mm
	1475
	
	
	

	seat height
	mm
	995 
	
	
	

	dry weight
	kg
	111
	
	
	

	DIN unladen weight 
	kg
	121
	
	
	

	permitted total weight
	kg
	280
	
	
	

	useable tank volume
	l
	ECE 8US-version 7
	
	
	

	
	
	
	
	
	

	Performance
	
	
	
	
	

	fuel consumption
	
	
	
	
	

	90 km/h 
	l/100 km
	4,3
	
	
	

	120 km/h
	l/100 km
	6,4
	
	
	

	maximum speed approx.
	km/h
	ca. 145
	
	
	


PAGE  

[image: image2.png]