[image: image1.png]


BMW
Media
Information

1/2010

Valid from 3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Coupé.
320d, 325d
	
	
	BMW 320d Coupé
	
	BMW 325d Coupé

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	
	2 / 4

	Length/width/height (unladen)
	mm
	4612 / 1782 / 1395
	
	4612 / 1782 / 1395

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1500 / 1513
	
	1500 / 1513

	Ground clearance
	
	143
	
	143

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
   appr ltr
	61
	
	61

	Cooling system incl heater
	ltr
	7.2 (7.5)
	
	7.9

	Engine oil1)
	ltr
	5.5
	
	7.5 (7.2)

	Weight, unladen, to DIN/EU
	kg
	1420 / 1495

(1430 / 1505)
	
	1515 / 1590

(1525 / 1600)

	Max load to DIN
	kg
	480
	
	480

	Max permissible
	kg
	1900 (1910)
	
	1995 (2005)

	Max axle load, front / rear
	kg
	900 / 1050
	
	960 / 1080

	Max trailer load (12%),

braked / unbraked
	kg
	1600 / 745
	
	1800 / 750

	Max roofload / max download
	kg
	75 / 75
	
	75 / 75 

	Luggage comp
	ltr
	440
	
	440

	Air drag
	cd x A
	0.28 x 2.08
	
	0.28 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-4 / 4
	
	Straight-6 / 4

	Engine technology
	
	Common-rail direct 

fuel injection, turbocharger with variable intake geometry
	
	Common-rail direct 

fuel injection, turbocharger with variable intake geometry

	Capacity, effective
	cc
	1995
	
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	
	16.5 

	Fuel
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	135 / 184
	
	150 / 204

	at
	rpm
	4000
	
	3750

	Max torque
	Nm/lb-ft
	380 / 280
	
	430 / 317

	at 
	rpm
	1900-2750
	
	1750

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	90 (80) / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	312 x 24 / vented
	
	330 x 24 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant,

BMW 325d also with Dry Braking, Fading Compensation, Start-Off Assistant

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Electric Power Steeering

	Steering ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	225/45 R17 91V RSC
	
	225/45 R17 91W RSC

	Wheels, front/rear
	
	8J x 17 light-alloy
	
	8J x 17 light-alloy


Specifications BMW 3 Series Coupé.
	
	
	BMW 320d Coupé
	
	BMW 325d Coupé

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.110 (4.171)
	
	5.080 (4.171) 

	                                  
II
	:1
	2.248 (2.340)
	
	2.804 (2.340)

	                                  
III
	:1
	1.403 (1.521)
	
	1.783 (1.521)

	                                  
IV
	:1
	1.000 (1.143)
	
	1.260 (1.143)

	                                  
V
	:1
	0.802 (0.867)
	
	1.000 (0.867)

	                                   
VI
	:1
	0.659 (0.691)
	
	0.835 (0.691)

	                                   
R
	:1
	3.727 (3.403)
	
	4.607 (3.403)

	Final drive
	:1
	3.154 (3.154)
	
	2.353 (2.813)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	10.5 (10.6)
	
	10.1 (10.2)

	Output per litre
	kW/hp
	67.7 / 92.1
	
	50.1 / 68.1

	Acceleration
0–100 km/h
	sec
	7.5 (7.6)
	
	6.9 (7.1)

	
standing-start
	sec


	28.1 (28.2)
	
	27.3 (27.3)

	In 4th/5th gear
80–120 km/h 
	sec
	5.9 / 7.5 (-)
	
	5.8 / 7.5 (-) 

	Top speed
	km/h
	237 (235)
	
	244 (242)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Auto Start Stop (on BMW 320d manual), 

gearshift point indicator (manual gearbox models), Electric Power Steering, 

on-demand management of ancillary units, intelligent lightweight construction, 
tyres with reduced roll resistance, a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	5.9 (6.8) 
	
	7.3 (7.9)

	Extra-urban
	ltr/100km
	4.0 (4.4)
	
	4.8 (5.1) 

	Combined
	ltr/100km
	4.7 (5.3)
	
	5.7 (6.1) 

	CO2
	g/km
	125 (140)
	
	151 (160)

	Emission rating
	
	EU5
	
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































