[image: image1.png]


BMW
Media
Information

1/2010

Valid from 3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Touring.
325i xDrive, 330i xDrive, 335i xDrive
	
	
	BMW 325i xDrive 

Touring
	BMW 330i xDrive Touring
	BMW 335i xDrive 

Touring

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	5 / 5
	5 / 5
	5 / 5

	Length/width/height (unladen)
	mm
	4527 / 1817 / 1418
	4527 / 1817 / 1418
	4527 / 1817 / 1418

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1506 / 1535
	1500 / 1529
	1500 / 1529

	Ground clearance
	
	140
	140
	140

	Turning circle
	m
	11.8
	11.8
	11.8

	Tank capacity
   appr ltr
	63
	63
	63

	Cooling system incl heater
	ltr
	8.2 (8.5)
	8.2 (8.5)
	8.2 (8.5)

	Engine oil1)
	ltr
	6.5
	6.5
	6.5

	Weight, unladen, to DIN/EU
	kg
	1640 / 1715

(1660 / 1735)
	1640 / 1715

(1660 / 1735)
	1660 / 1735

(1675 / 1750)

	Max load to DIN
	kg
	540
	540
	540

	Max permissible
	kg
	2180 (2200)
	2180 (2200)
	2200 (2215)

	Max axle load, front / rear
	kg
	1040 / 1180
	1045 / 1190
	1090 / 1200

	Max trailer load (12%),

braked / unbraked
	kg
	1700 / 750
	1800 / 750
	1800 / 750

	Max roofload / max download
	kg
	75 / 75
	75 / 75
	75 / 75 

	Luggage comp
	ltr
	460-1385
	460-1385
	450-1375

	Air drag
	cd x A
	0.31 x 2.17
	0.31 x 2.17
	0.32 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	Straight-6 / 4
	Straight-6 / 4

	Engine technology
	
	Second-generation 

direct fuel injection 

(High Precision Injection) with lean burn
	Second-generation 

direct fuel injection 

(High Precision Injection) with lean burn
	BMW TwinPower Turbo, second-generation 

direct fuel injection 
(High Precision Injection), fully variable valve management (VALVETRONIC)

	Capacity, effective
	cc
	2996
	2996
	2979

	Stroke / bore
	mm
	88.0 / 85.0
	88.0 / 85.0
	89.6 / 84.0

	Compression ratio
	:1
	12.0
	12.0
	10.2 

	Fuel
	
	min 91 RON
	min 91 RON
	min 95 RON

	Max output
	kW/hp
	160 / 218
	200 / 272
	225 / 306

	at
	rpm
	6100
	6700
	5800

	Max torque
	Nm/lb-ft
	270 / 200
	320 / 236
	400 / 295

	at 
	rpm
	2400-4200
	2750-3000
	1200-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	80 / luggage compartment
	80 / luggage comp
	80 / luggage comp

	Alternator
	A/W
	150 / 2100 (180 / 2520)
	150 / 2100 (180 / 2520)
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	330 x 24 / vented
	348 x 30 / vented
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	336 x 22 / vented
	336 x 22 / vented
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant, Dry Braking, Fading Compensation, Start-Off Assistant, DSC and xDrive networked through Integrated Chassis Management (ICM)

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion power steering

	Steering ratio, overall
	:1
	18.2
	18.2
	18.2

	Tyres, front/rear
	
	225/45 R17 91W RSC
	225/45 R17 91Y RSC
	225/45 R17 91Y RSC

	Wheels, front/rear
	
	8J x 17 light-alloy
	8J x 17 light-alloy
	8J x 17 light-alloy


Specifications BMW 3 Series Touring.
	
	
	BMW 325i xDrive Touring
	BMW 330i xDrive Touring
	BMW 335i xDrive Touring

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.350 (4.171)
	4.350 (4.171)
	4.110 (4.171) 

	                                  
II
	:1
	2.496 (2.340)
	2.496 (2.340)
	2.315 (2.340)

	                                  
III
	:1
	1.665 (1.521)
	1.665 (1.521)
	1.542 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	1.230 (1.143)
	1.179 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	                                   
VI
	:1
	0.851 (0.691)
	0.851 (0.691)
	0.846 (0.691)

	                                   
R
	:1
	3.926 (3.403)
	3.926 (3.403)
	3.727 (3.403)

	Final drive
	:1
	3.071 (3.462)
	3.154 (3.636)
	3.077 (3.462)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	10.3 (10.4)
	8.2 (8.3)
	7.4 (7.4)

	Output per litre
	kW/hp
	53.4 / 72.6
	66.8 / 90.8
	75.5 / 102.7

	Acceleration
0–100 km/h
	sec
	7.7 (8.1)
	6.4 (6.6)
	5.5 (5.6)

	
standing-start
	sec


	28.3 (28.6)
	26.3 (26.5)
	25.3

	In 4th/5th gear
80–120 km/h 
	sec
	8.9 / 11.0 (-)
	7.3 / 9.3 (-)
	6.0 / 7.2 (-) 

	Top speed
	km/h
	242 (240)
	250
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual gearbox models), on-demand management of ancillary units, intelligent lightweight construction, tyres with reduced roll resistance, oil pump with map control,  
a/c compressor detachable


	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	11.1 (11.2) 
	11.2 (11.3) 
	12.4 (13.2)

	Extra-urban
	ltr/100km
	6.5 (6.6)
	6.6 (6.7)
	6.7 (6.9) 

	Combined
	ltr/100km
	8.2 (8.3)
	8.3 (8.4)
	8.8 (9.2) 

	CO2
	g/km
	190 (194)
	193 (195)
	206 (215)

	Emission rating
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































