[image: image1.png]


BMW
Media
information

2/2010

Valid from 3/2010

BMW
Media
information

2/2010

Valid from 3/2010


Specifications BMW M3 Sedan.

	
	
	BMW M3 Sedan 
	
	

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	
	

	Length/width/height (unladen)
	mm
	4580 / 1817 / 1447
	
	

	Wheelbase
	mm
	2761
	
	

	Track, front/rear
	mm
	1540 / 1539
	
	

	Turning circle
	m
	11.7
	
	

	Tank capacity
	appr ltr
	63
	
	

	Cooling system incl heating Heizung
	ltr
	11.4 (11.6)
	
	

	Engine oil1)
	ltr
	8.8
	
	

	Weight, unladen, to DIN/EU
	kg
	1605 / 1680 (1625 / 1700)
	
	


	Max load to DIN
	kg
	545 (525)
	
	

	Max permissible
	kg
	2150
	
	

	Max axle load, front/rear
	kg
	1020 / 1190
	
	

	Max trailer load (12%),

braked/unbraked
	kg
	– / –
	
	

	Max roofload/max trail downld
	kg
	75 / –
	
	

	Luggage comp capacity
	ltr
	450
	
	

	Air resistance
	Cd x A
	0.31 x 2.23
	
	

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves Zylinder/Ventile


	
	V / 8 / 4
	
	

	Engine technology
	 
	High-speed concept with single butterflies, 
dual-Vanos, oil supply optimised for driving dynamics, ion flow anti-knock control 
	
	

	Capacity, effective
	cc
	3999
	
	

	Stroke / bore
	mm
	75.2 / 92.0
	
	

	Compression ratio
	:1
	12.0 
	
	

	Fuel grade 
	
	min RON 95
	
	

	Max output
	kW/hp
	309 / 420
	
	

	at 
	rpm
	8300
	
	

	Max torque
	Nm/lb-ft
	400/295
	
	

	at
	rpm
	3900
	
	

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	70 / luggage comp
	
	

	Alternator
	A/W
	180 / 2520
	
	

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Aluminium two-joint spring strut axle with tiebar; small 
positive steering roll radius; compensation of lateral forces; anti-dive

	Suspension, rear rearHinterradaufhängung
	
	Five-arm lightweight axle with ant-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper compound disc brakes

	Diameter
	mm
	360 x 30, vented and cross-drilled

	Brakes, rear 
	
	Single-piston swing-calliper compound disc brakes

	Diameter
	mm
	350 x 24, vented and cross-drilled

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Brake Assistant, Dry Braking, 
Fading Compensation, Start-Off Assistant, variable M differential lock, optional: Electronic Damper Control

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering 
	
	Rack-and-pinion steering with hydraulic assistance and Servotronic

	Transmission ratio, overall
	:1
	12.5
	
	

	Tyres, front/rear
	
	245/40 ZR18 /
 265/40 ZR18
	
	

	Rims, front/rear
	
	8.5J x 18 EH2+ IS 29 cast alu / 9.5J x 18 EH2+ IS 23 cast alu
	
	


Specifications BMW M3 Sedan.

	
	
	BMW M3 Sedan 
	
	

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote control functions and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, Park Distance Control, cruise control, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual 
(Optional: seven-speed M Double-Clutch Gearbox with Drivelogic)

	Gear ratios
I
	:1
	4.055 (4.780)
	
	

	                                  
II
	:1
	2.369 (3.056)
	
	

	                                  
III
	:1
	1.582 (2.153)
	
	

	                                  
IV
	:1
	1.192 (1.678)
	
	

	                                  
V
	:1
	1.000 (1.390)
	
	

	                                   
VI
	:1
	0.872 (1.203)
	
	

	                                   
VII
	:1
	– (1.000)
	
	

	                                   
R
	:1
	3.677 (4.454)
	
	

	Final drive 
	:1
	3.846 (3.154)
	
	

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio
(DIN-Leergew.)
	kg/kW
	5.2 (5.3)
	
	

	Output per litre 
	kW/hp
	77.3/105.1
	
	

	Acceleration
0–100 km/h
	sec
	4.9 (4.7)
	
	

	Standing-start km
	sec

ec
	23.7 (23.3)
	
	

	In 4th./5th gear    80–120 km/h km/h km/h km/h km/h
	sec
	5.1 / 6.2 (4.4 / 5.4)
	
	

	Top speed
	km/h
	250 
	
	

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Brake Energy Regeneration, Auto Start Stop, Gearshift Point Indicator, on-demand management of ancillary units, intelligent lightweight construction, map-controlled oil pump

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	17.7 (15.9)
	
	

	Extra-urban
	ltr/100km
	9.3 (8.5)
	
	

	Combined
	ltr/100km
	12.4 (11.2)
	 
	

	CO2
	g/km
	290 (263)
	
	

	Emission rating
	
	EU5
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications apply to ACEA markets/ homologation data apply in part only to Germany (weight)

Figures in brackets for automatic transmission model

1)  Oil change




















































