[image: image1.png]


BMW
Media
Information

3/2010

Valid from 3/2010
BMW
Media
Information

3/2010

Valid from 3/2010

Specifications BMW 3 Series Sedan.
320d EfficientDynamics Edition
	
	
	
	
	BMW 320d 
EfficientDynamics Edition

	
	
	
	
	

	Body     
	
	
	
	

	No of doors / seats     
	
	
	
	4 / 5

	Length/width/height (unladen)
	mm
	
	
	4531 / 1817 / 1406

	Wheelbase
	mm
	
	
	2760

	Track, front/rear     
	mm
	
	
	1506 / 1535

	Ground clearance
	mm
	
	
	129

	Turning circle
	m
	
	
	11.0

	Tank capacity
	app ltr
	
	
	61

	Cooling system incl heater
	ltr
	
	
	7.5

	Engine oil1)
	ltr
	
	
	5.2

	Weight, unladen, to DIN/EU
	kg
	
	
	1420 / 1495



	Max load to DIN
	kg
	
	
	520

	Max permissible  
	kg
	
	
	1940

	Max axle load, front/rear  
	kg
	
	
	925 / 1080

	Max trailer load (12%)

braked/unbraked
	kg
	
	
	– / –

	Max roofload/trailer download
	kg
	
	
	75 / – 

	Luggage compartment capacity
	ltr
	
	
	460

	Air resistance 
	cd x A
	
	
	0.27 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cylinders/valves    Zylinder/Ventile


	
	
	
	Straight- 4 / 4

	Engine technology 
	
	
	
	Common-rail direct fuel injection, turbocharger with variable intake geometry 

	Capacity, effective
	cc
	
	
	1995

	Stroke / bore
	mm
	
	
	90.0 / 84.0

	Compression ratio
	:1
	
	
	16.5 

	Fuel      
	
	
	
	Diesel

	Max output
	kW/hp
	
	
	120 / 163

	at           
	rpm  
	
	
	3250-4200

	Max torque
	Nm/lb-ft
	
	
	380 / 280

	at
	rpm  
	
	
	1900-2750

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	
	
	90 (80) / luggage compartment

	Alternator    
	A/W
	
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front  
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel construction

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	
	
	312 x 24 / vented  

	Brakes, rear 
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	
	
	300 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Dynamic Brake Control

	Safety features     
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Pressure Indicator

	Steering 
	
	EPS Electric Power Steering   

	Steering ratio, overall 
	:1
	
	
	16.0

	Tyres, front/rear  
	
	
	
	205/55 R16 91V

	Rims, front/rear  
	
	
	
	7J x 16 light-alloy


Specifications BMW 3 Series Sedan.
320d EfficientDynamics Edition
	
	
	
	
	BMW 320d
EfficientDynamics Edition

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Inquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, Google Directory Search, News, realtime weather, office functions and audio streaming via Bluetooth, BMW routes and online music title update

	Safety    
	
	Optional: Adaptive Headlights with Bending Light, High-Beam Assistant, Park Distance Control, cruise control, Automatic/Enhanced Emergency Call      

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	
	
	Six-speed manual     

	Gear ratios
I
	:1
	
	
	4.110

	                                  
II
	:1
	
	
	2.248

	                                  
III
	:1
	
	
	1.403

	                                  
IV
	:1
	
	
	1.000

	                                  
V
	:1
	
	
	0.802 

	                                   
VI
	:1
	
	
	0.659 

	                                   
R
	:1
	
	
	3.727 

	Final drive          
	:1
	
	
	2.790

	
	
	
	
	

	Performance   
	
	
	
	

	Power-to-weight ratio
(DIN-Leergew.)
	kg/kW
	
	
	11.8

	Output per litre 
	kW/hp
	
	
	60.2 / 81.9

	Acceleration
0–100 km/h
	sec
	
	
	8.0

	                      standing-start km
	sec
	
	
	28.7

	In 4th/5th gear
	sec
	
	
	6.7 / 9.1

	Top speed
	km/h
	
	
	228

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features    
	
	Brake Energy Regeneration, EPS Electric Power Steering,  
Auto Start Stop and gearshift point indicator, on-demand control of ancillary units, intelligent lightweight construction, active air flap control, tyres with reduced roll resistance, A/C compressor detachable

	
	
	
	
	

	Fuel Consumption in EU Cycle
	
	
	
	

	Urban    
	l/100kmkm kkm km
	
	
	5.0

	Extra-urban  
	l/100kmkm km
	
	
	3.6

	Combined
	l/100kmkm km
	
	
	4.1

	CO2
	g/km
	
	
	109

	Emission rating     
	
	
	
	EU5  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	18 / 23 / 24


Specifications apply to ACEA markets / homologation-relevant data apply in part only to Germany (weight)

1)  Oil change     




















































