Specifications.
The New BMW 5 Series Touring.
523i, 528i, 535i.

|  |  | BMW 523i Touring |  | BMW 528i Touring | BMW 535i Touring |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 5 / 5 |  | 5 / 5 | 5 / 5 |  |
| Length/width/height (unladen) | mm | 4907 / 1860 / 1462 1) |  | 4907 / 1860 / 1462 1) | 4907 / 1860 / 1462 1) |  |
| Wheelbase | mm | 2968 |  | 2968 | 2968 |  |
| Track, front/rear | mm | 1600 / 1626 |  | 1600 / 1626 | 1600 / 1626 |  |
| Ground clearance | mm | 141 |  | 141 | 141 |  |
| Turning circle | m | 11.95 |  | 11.95 | 11.95 |  |
| Tank capacity | app ltr | 70 |  | 70 | 70 |  |
| Cooling system incl heater | ltr | 9.0 |  | 9.0 | 9.3 |  |
| Engine oil1) | ltr | 6.5 |  | 6.5 | 6.5 |  |
| Weight, unladen, to DIN/EU | kg | 1715 / 1790 (1730 / 1805) |  | 1725 / 1800 (1745 / 1820) | 1765 / 1840 (1770 / 1845) |  |
| Max load to DIN | kg | 650 |  | 650 | 650 |  |
| Max permissible | kg | 2365 (2380) |  | 2375 (2395) | 2415 (2420) |  |
| Max axle load, front/rear | kg | 1070 / 1380 |  | 1070 / 1380 | 1100 / 1400 |  |
| Max trailer load (12%), braked/unbraked | kg | 1900 / 750 (2000 / 750) |  | 2000 / 750 (2000 / 750) | 2000 / 750 (2000 / 750) |  |
| Max roofload/max towbar dwnl | kg | 100 / 90 |  | 100 / 90 | 100 / 90 |  |
| Luggage comp capacity | ltr | 560-1670 |  | 560-1670 | 560-1670 |  |
| Air resistance | Cd x A | 0.31 x 2.35 |  | 0.31 x 2.35 | 0.31 x 2.35 |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-six / 4 |  | Straight-six / 4 | Straight-six / 4 |  |
| Engine technology |  | Direct fuel injection (High Precision Injection), lean burn | Direct fuel injection (High Precision Injection), lean burn |  |  | BMW TwinPower Turbo, direct fuel injection (High Precision Injection), VALVETRONIC variable valve management |
| Capacity, effective | cc | 2996 |  | 2996 | 2979 |  |
| Stroke / bore | mm | 88.0 / 85.0 |  | 88.0 / 85.0 | 89.6 / 84.0 |  |
| Compression ratio | :1 | 12.0 |  | 12.0 | 10.2 |  |
| Fuel grade |  | min RON 91 |  | min RON 91 | min RON 91 |  |
| Max output | kW/hp | 150 / 204 |  | 190 / 258 | 225 / 306 |  |
| at | rpm | 6100 |  | 6600 | 5800 |  |
| Max torque | Nm/lb-ft | 270/200 |  | 310/228 | 400/295 |  |
| at | rpm | 1500-4250 |  | 2600-5000 | 1200-5000 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– | 80 (90) / luggage comp |  | 80 (90) / luggage comp | 80 (90) / luggage comp |  |
| Alternator | A/W | 210 / 2940 |  | 210 / 2940 | 210 / 2940 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive, air suspension with automatic self-levelling |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| diameter | mm | 330 x 24 / vented |  | 348 x 30 / vented | 348 x 36 / vented |  |
| Brakes, rear |  | Single-piston swing-calliper aluminium disc brakes |  |  |  |  |
| diameter | mm | 330 x 20 / vented |  | 330 x 20 / vented | 345 x 24 / vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant optional: Dynamic Damper Control (BMW 523i, BMW 528i), Adaptive Drive (BMW 535i) |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 17.1 |  | 17.1 | 17.1 |  |
| Tyres, front/rear |  | 225/55 R17 97W |  | 225/55 R17 97Y | 225/55 R17 97Y |  |
| Rims, front/rear |  | 8J x 17 light-alloy |  | 8J x 17 light-alloy | 8J x 17 light-alloy |  |

Specifications.
The New BMW 5 Series Touring.
523i, 528i, 535i.

|  |  |  | BMW 523i Touring |  |  | BMW 528i Touring | BMW 535i Touring |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: internet, BMW Online incl Park Info, National Info, Google Directory Search, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming, and BMW Routes |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  |  | Six-speed manual transmission (optional: eight-speed automatic with Steptronic) |  |  |  |  |  |
| Gear ratios I |  |  |  | 4.323 (4.714) |  | 3.498 (4.714) |  |  | 4.110 (4.714) |
| II |  |  |  | 2.456 (3.143) |  | 2.005 (3.143) |  |  | 2.315 (3.143) |
| III |  |  |  | 1.659 (2.106) |  | 1.313 (2.106) |  |  | 1.542 (2.106) |
| IV |  |  |  | 1.230 (1.667) |  | 1.000 (1.667) |  |  | 1.179 (1.667) |
| V |  |  |  | 1.000 (1.285) |  | 0.809 (1.285) |  |  | 1.000 (1.285) |
| VI |  |  |  | 0.848 (1.000) |  | 0.701 (1.000) |  |  | 0.846 (1.000) |
| VII |  |  |  | – (0.839) |  | – (0.839) |  |  | – (0.839) |
| VIII |  |  |  | – (0.667) |  | – (0.667) |  |  | – (0.667) |
| R |  |  |  | 3.938 (3.295) |  | 3.187 (3.295) |  |  | 3.727 (3.295) |
| Final drive |  |  |  | 3.385 (3.462) |  | 4.100 (3.385) |  |  | 3.231 (3.077) |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  | 11.4 (11.5) |  | 9.1 (9.2) |  | 7.8 (7.9) |  |
| Output per litre | kW/hp |  |  | 50.1/68.1 |  | 63.4/86.2 |  | 75.5/102.7 |  |
| Acceleration 0–100 km/h | sec |  |  | 8.2 (8.4) |  | 6.9 (6.9) |  | 6.0 (6.1) |  |
| Standing-start km | sec |  |  | 29.0 (29.5) |  | 27.0 (27.0) |  | 25.4 (25.8) |  |
| In 4th gear 80–120 km/h | sec |  |  | 9.1 (–) |  | 7.3 (–) |  | 6.1 (–) |  |
| Top speed | km/h |  |  | 231 (227) |  | 247 (246) |  | 250 (250) |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration with recuperation display, Auto Start Stop, gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), tyres with reduced roll resistance |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  | 10.9 (10.8) |  | 10.8 (10.6) |  | 11.9 (11.9) |  |
| Extra-urban | ltr/100 km |  |  | 6.2 (6.1) |  | 6.4 (6.2) |  | 6.7 (6.5) |  |
| Combined | ltr/100 km |  |  | 7.9 (7.8) |  | 8.0 (7.8) |  | 8.6 (8.5) |  |
| CO2 | g/km |  |  | 185 (182) |  | 188 (182) |  | 201 (197) |  |
| Emission rating |  |  |  | EU5 |  | EU5 |  | EU5 |  |

Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)

Figures in brackets apply to automatic transmission models

1) With roof aerial: 1,488 mm

2)  Oil change