Specifications.
The new BMW 5 Series Sedan. 
530d, 535d.

|  |  | BMW 530d |  |  | BMW 535d |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 |  |  | 4 / 5 |  |
| Length/width/height (unladen) | mm | 4899 / 1860 / 1464 |  |  | 4899 / 1860 / 1464 |  |
| Wheelbase | mm | 2968 |  |  | 2968 |  |
| Track, front/rear | mm | 1600 / 1627 |  |  | 1600 / 1627 |  |
| Ground clearance | mm | 141 |  |  | 141 |  |
| Turning circle | m | 11.95 |  |  | 11.95 |  |
| Tank capacity | app ltr | 70 |  |  | 70 |  |
| Cooling system incl heater | ltr | 8.0 |  |  | 8.0 |  |
| Engine oil1) | ltr | 7.2 |  |  | 7.2 |  |
| Weight, unladen, to DIN/EU | kg | 1715 / 1790 (1720 /1795) |  |  | (1750 /1825) |  |
| Max load to DIN | kg | 610 |  |  | 610 |  |
| Max permissible | kg | 2325 (2330) |  |  | (2360) |  |
| Max axle load, front/rear | kg | 1125 / 1295 |  |  | 1140 / 1310 |  |
| Max trailer load (12%), braked/unbraked | kg | 2000 / 750 |  |  | 2000 / 750 |  |
| Max roofload/max towbar dwnl | kg | 100/90 |  |  | 100/90 |  |
| Luggage comp capacity | ltr | 520 |  |  | 520 |  |
| Air resistance | Cd x A | 0.28 x 2.35 |  |  | 0.29 x 2.35 |  |
| Power Unit |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-six / 4 |  |  | Straight-six / 4 |  |
| Engine technology |  | Turbocharger with variable intake geometry, common-rail direct injection with piezo injectors (max pressure 1800 bar) |  |  |  | BMW TwinPower Turbo (variable Twin Turbo), common-rail direct injection, piezo injectors (max pressure 2000 bar) |
| Capacity, effective | cc | 2993 |  |  | 2993 |  |
| Stroke / bore | mm | 90.0 / 84.0 |  |  | 90.0 / 84.0 |  |
| Compression ratio | :1 | 16.5 |  |  | 16.5 |  |
| Fuel grade |  | Diesel |  |  | Diesel |  |
| Max output | kW/hp | 180 / 245 |  |  | 220 / 300 |  |
| at | rpm | 4000 |  |  | 4400 |  |
| Max torque | Nm/lb-ft | 540/398 |  |  | 600/442 |  |
| at | rpm | 1750-3000 |  |  | 1750 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– | 90 / luggage comp |  |  | 90 / luggage comp |  |
| Alternator | A/W | 180 / 2520 |  |  | 180 / 2520 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle, aluminium, with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| Diameter | mm | 330 x 24 / inner-vented |  |  | 348 x 36 / inner-vented |  |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |  |
| Diameter | mm | 330 x 20 / inner-vented |  |  | 345 x 24 / inner-vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Adaptive Drive |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 17.1 |  |  | 17.1 |  |
| Tyres, front/rear |  | 225/55 R17 97W |  |  | 225/55 R17 97W |  |
| Rims, front/rear |  | 8J x 17 light-alloy |  |  | 8J x 17 light-alloy |  |

Specifications.
The new BMW 5 Series Sedan. 
530d, 535d.

|  |  |  | BMW 530d |  |  |  |  |  | BMW 535d |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: internet, BMW Online incl Park Info, National Info, Google Directory Search, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming, and BMW Routes |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  |  | Six-speed manual transmission (optional: eight-speed automatic with Steptronic) |  |  |  |  | Eight-speed automatic with Steptronic |
| Gear ratios I |  | :1 |  | 5.080 (4.714) |  |  |  |  | 4.714 |
| II |  | :1 |  | 2.804 (3.143) |  |  |  |  | 3.143 |
| III |  | :1 |  | 1.783 (2.106) |  |  |  |  | 2.106 |
| IV |  | :1 |  | 1.260 (1.667) |  |  |  |  | 1.667 |
| V |  | :1 |  | 1.000 (1.285) |  |  |  |  | 1.285 |
| VI |  | :1 |  | 0.835 (1.000) |  |  |  |  | 1.000 |
| VII |  | :1 |  | – (0.839) |  |  |  |  | 0.839 |
| VIII |  | :1 |  | – (0.667) |  |  |  |  | 0.667 |
| R |  | :1 |  | 4.607 (3.317) |  |  |  |  | 3.317 |
| Final drive |  | :1 |  | 2.563 (2.471) |  |  |  |  | 2.647 |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  | 9.5 (9.6) |  |  |  |  | 8.0 |
| Output per litre | kW/hp |  |  | 60.1/81.7 |  |  |  |  | 73.5/100.0 |
| Acceleration 0–100 km/h | sec |  |  | 6.3 (6.3) |  |  |  |  | 5.7 |
| Standing-start km | sec |  |  | 26.5 (26.5) |  |  |  |  | 25.0 |
| In 4th gear 80–120 km/h | sec |  |  | 5.0 (–) |  |  |  |  | – |
| Top speed | km/h |  |  | 250 (250) |  |  |  |  | 250 |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration with recuperation display, Auto Start Stop (BMW 520d with manual gearbox) gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  | 8.0 (7.7) |  |  |  |  | 7.9 |
| Extra-urban | ltr/100 km |  |  | 5.3 (5.2) |  |  |  |  | 5.1 |
| Combined | ltr/100 km |  |  | 6.3 (6.1) |  |  |  |  | 6.1 |
| CO2 | g/km |  |  | 166 (160) |  |  |  |  | 162 |
| Emission rating |  |  |  | EU52) |  |  |  |  | EU52) |

Specifications applicable to ACEA markets. data relevant to homologation applicable in part only to Germany (weight)

Figures in brackets apply to automatic transmission models

1)  Height with roof fin: 1475 mm

2)  Oil change

3)  EU6 with optional BMW BluePerformance technology