[image: image1.png]


BMW
Media
Information

9/2010

Valid from 
9/2010
BMW
Media
Information

9/2010

Valid from 9/2010

Specifications BMW 3 Series Saloon.
316d, 318d, 320d

	
	
	BMW 316d 
Saloon
	BMW 318d
Saloon
	BMW 320d 
Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	4 / 5
	4 / 5

	Length/width/height (unladen)
	mm
	4531 / 1817 / 1421
	4531 / 1817 / 1421
	4531 / 1817 / 1421

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1506 / 1535
	1506 / 1535
	1506 / 1535

	Ground clearance
	
	144
	144
	144

	Turning circle
	m
	11.0
	11.0
	11.0

	Tank capacity
	appr ltr
	61
	61
	61

	Cooling system incl heater
	ltr
	7.5
	7.5 (7.9)
	7.5 (7.9)

	Engine oil1)
	ltr
	5.2
	5.2
	5.2

	Weight, unladen, to DIN/EU
	kg
	1400 / 1475


	1415 / 1490

(1430 / 1505)
	1420 / 1495

(1430 / 1505)

	Max load to DIN
	kg
	520
	520
	520

	Max permissible
	kg
	1920
	1935 (1950)
	1940 (1950)

	Max axle load, front / rear
	kg
	900 / 1080
	925 / 1080
	925 / 1080

	Max trailer load (12%),

braked / unbraked
	kg
	1600 / 695
	1600 / 745
	1600 / 745

	Max roofload / max download
	kg
	75 / 75
	75 / 75
	75 / 75 

	Luggage comp
	ltr
	460
	460
	460

	Air drag
	cd x A
	0.27 x 2.17
	0.27 x 2.17
	0.27 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves Zylinder/Ventile


	
	Straight-4 / 4
	Straight-4 / 4
	Straight-4 / 4

	Engine technology
	
	Common-rail direct fuel injection, turbocharger

with variable intake geometry
	Common-rail direct fuel injection, turbocharger with variable intake geometry
	Common-rail direct fuel injection, turbocharger 

with variable intake geometry

	Capacity, effective
	cc
	1995
	1995
	1995

	Stroke / bore
	mm
	90.0 / 84.0
	90.0 / 84.0
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	16.5
	16.5 

	Fuel
	
	Diesel
	Diesel
	Diesel

	Max output
	kW/hp
	85 / 115
	105 / 143
	135 / 184

	at
	rpm
	4000
	4000
	4000

	Max torque
	Nm/lb-ft
	260/192
	320/236
	380/280

	at 
	rpm
	1750-2500
	1750-2500
	1750-2750

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	90 / luggage compartment
	90 (80) / luggage comp
	90 (80) / luggage comp

	Alternator
	A/W
	180 / 2520
	180 / 2520
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 24 / vented
	300 x 24 / vented
	312 x 24 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	296 x 11 / vented
	300 x 20 / vented
	300 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Electric power steering                                     Hydraulic rack-and-pinion steering

	Steering ratio, overall
	:1
	16.0
	16.0
	16.0

	Tyres, front/rear
	
	205/55 R16 91H
	205/55 R16 91H
	205/55 R16 91V

	Wheels, front/rear
	
	7J x 16 steel
	7J x 16 steel
	7J x 16 steel


Specifications BMW 3 Series Saloon.
	
	
	BMW 316d
Saloon
	BMW 318d
Saloon
	BMW 320d
Saloon

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, BMW Routes, office functions,
Bluetooth audio streaming and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual
	Six-speed manual
(optional: six-speed automatic)

	Gear ratios
I
	:1
	4.002
	4.002 (4.171)
	4.110 (4.171) 

	                                  
II
	:1
	2.108
	2.108 (2.340)
	2.248 (2.340)

	                                  
III
	:1
	1.380
	1.380 (1.521)
	1.403 (1.521)

	                                  
IV
	:1
	1.000
	1.000 (1.143)
	1.000 (1.143)

	                                  
V
	:1
	0.780
	0.780 (0.867)
	0.802 (0.867)

	                                   
VI
	:1
	0.645
	0.645 (0.691)
	0.659 (0.691)

	                                   
R
	:1
	3.187
	3.187 (3.403)
	3.727 (3.403)

	Final drive
	:1
	3.071
	3.071 (3.231)
	3.154 (3.154)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
(DIN-Leergew.)
	kg/kW
	16.5
	13.5 (13.6)
	10.5 (10.6)

	Output per litre
	kW/hp
	42.6/57.9
	52.6/71.5
	67.7/92.1

	Acceleration
0–100 km/h
	sec
	10.9
	9.1 (9.3)
	7.5 (7.6)

	
standing-start km km
	sec


	32.2


	30.2 (30.5)
	28.1 (28.2)

	In 4th/5th gear
80–120 km/h km/h km/h km/h km/h
	sec
	9.5 / 13.1 (-)
	7.8 / 10.4 (-)
	5.9 / 7.5 (-) 

	Top speed
	km/h
	202
	210
	235 (233)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Electric Power Steering (BMW 316d, BMW 318d),
Auto Start Stop and gearshift point indicator (manual gearbox models), on-demand management of ancillary units, intelligent lightweight construction, 
a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	5.4 
	5.4 (6.8) 
	5.9 (6.8)

	Extra-urban
	ltr/100km
	4.0
	4.0 (4.4)
	4.0 (4.4) 

	Combined
	ltr/100km
	4.5
	4.5 (5.3)
	4.7 (5.3) 

	CO2
	g/km
	118
	119 (140)
	125 (140)

	Emission rating
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change





















































