[image: image1.png]


BMW
Media
Information

9/2010

Valid from 9/2010
BMW
Media
Information

9/2010

Valid from 9/2010

Specifications BMW 3 Series Saloon.
325i xDrive, 330i xDrive, 335i xDrive
	
	
	BMW 325i xDrive
Saloon
	BMW 330i xDrive
Saloon
	BMW 335i xDrive
Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	4 / 5
	4 / 5

	Length/width/height (unladen)
	mm
	4531 / 1817 / 1421
	4531 / 1817 / 1421
	4531 / 1817 / 1421

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1506 / 1535
	1500 / 1529
	1500 / 1529

	Ground clearance
	
	140
	140
	140

	Turning circle
	m
	11.8
	11.8
	11.8

	Tank capacity
   appr ltr
	63
	63
	63

	Cooling system incl heater
	ltr
	8.2 (8.5)
	8.2 (8.5)
	8.2 (8.5)

	Engine oil1)
	ltr
	6.5
	6.5
	6.5

	Weight, unladen, to DIN/EU
	kg
	1540 / 1615

(1560 / 1635)
	1575 / 1650

(1590 / 1665)
	1615 / 1690

(1630 / 1705)

	Max load to DIN
	kg
	520
	520
	520

	Max permissible
	kg
	2060 (2080)
	2100 (2115)
	2135 (2150)

	Max axle load, front / rear
	kg
	1020 / 1105
	1035 (1110)
	1090 (1135)

	Max trailer load (12%),

braked / unbraked
	kg
	1700 / 750
	1800 / 750
	1800 / 750

	Max roofload / max download
	kg
	75 / 75
	75 / 75
	75 / 75 

	Luggage comp
	ltr
	460
	460
	450

	Air drag
	cd x A
	0.30 x 2.17
	0.31 x 2.17
	0.32 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	Straight-6 / 4
	Straight-6 / 4

	Engine technology
	
	Second-generation
direct fuel injection 

(High Precision Injection) with lean burn
	Second-generation
direct fuel injection 

(High Precision Injection) with lean burn
	BMW TwinPower Turbo, second-generation 

direct fuel injection
(High Precision Injection), fully variable valve management (VALVETRONIC)

	Capacity, effective
	cc
	2996
	2996
	2979

	Stroke / bore
	mm
	88.0 / 85.0
	88.0 / 85.0
	89.6 / 84.0

	Compression ratio
	:1
	12.0
	12.0
	10.2 

	Fuel
	
	min 91 RON
	min 91 RON
	min 91 RON

	Max output
	kW/hp
	160 / 218
	200 / 272
	225 / 306

	at
	rpm
	6100
	6700
	5800

	Max torque
	Nm/lb-ft
	270 / 200
	320 / 236
	400 / 295

	at 
	rpm
	2400-4200
	2750-3000
	1200-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	80 / luggage compartment
	80 / luggage comp
	80 / luggage comp

	Alternator
	A/W
	155 / 2170 (180 / 2520)
	155 / 2170 (180 / 2520)
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	312 x 24 / vented
	330 x 24 / vented
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	336 x 22 / vented
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant, Dry Braking, Fading Compensation, Start-Off Assistant, DSC and xDrive networked through Integrated Chassis Management (ICM)

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Hydraulic rack-and-pinion power steering

	Steering ratio, overall
	:1
	18.2
	18.2
	18.2

	Tyres, front/rear
	
	205/55 R16 91W
	225/45 R17 91W RSC
	225/45 R17 91W RSC

	Wheels, front/rear
	
	7J x 16 light-alloy
	8J x 17 light-alloy
	8J x 17 light-alloy


Specifications BMW 3 Series Saloon xDrive.
	
	
	BMW 325i xDrive
Saloon
	BMW 330i xDrive
Saloon
	BMW 335i xDrive
Saloon

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, BMW Routes, office functions, 
Bluetooth audio streaming and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual (optional: six-speed automatic)

	Gear ratios
I
	:1
	4.350 (4.171)
	4.350 (4.171)
	4.110 (4.171) 

	                                  
II
	:1
	2.496 (2.340)
	2.496 (2.340)
	2.315 (2.340)

	                                  
III
	:1
	1.665 (1.521)
	1.665 (1.521)
	1.542 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	1.230 (1.143)
	1.179 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	                                   
VI
	:1
	0.851 (0.691)
	0.851 (0.691)
	0.846 (0.691)

	                                   
R
	:1
	3.926 (3.403)
	3.926 (3.403)
	3.727 (3.403)

	Final drive
	:1
	3.071 (3.462)
	3.154 (3.636)
	3.077 (3.462)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	9.6 (9.8)
	7.9 (8.0)
	7.2

	Output per litre
	kW/hp
	53.4 / 72.6
	66.8 / 90.8
	75.5 / 102.7

	Acceleration
0–100 km/h
	sec
	7.3 (7.7)
	6.2 (6.4)
	5.4 (5.5)

	
standing-start
	sec


	27.8 (28.1)
	26.1 (26.2)
	25.0

	In 4th/5th gear
80–120 km/h 
	sec
	8.3 / 10.2 (-)
	7.0 / 9.0 (-)
	5.9 / 7.1 (-) 

	Top speed
	km/h
	244 (242)
	250
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual gearbox models),

on-demand management of ancillary units, intelligent lightweight construction,

tyres with reduced roll resistance (BMW 330i and BMW 335i),

oil pump with map control, a/c compressor detachable


	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	11.0 (11.1) 
	11.1 (11.2) 
	12.4 (13.1)

	Extra-urban
	ltr/100km
	6.4 (6.5)
	6.5 (6.6)
	6.7 (6.8) 

	Combined
	ltr/100km
	8.1 (8.2)
	8.2 (8.3)
	8.8 (9.1) 

	CO2
	g/km
	188 (192)
	191 (193)
	205 (212)

	Emission rating
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change





















































