[image: image1.png]


BMW
Media
Information

9/2010

Valid from 9/2010
BMW
Media
Information

9/2010

Valid from 9/2010


Specifications BMW 1 Series Three-Door.
116i, 118i
	
	
	BMW 116i
	BMW 116i
	BMW 118i

	
	
	(other EU markets incl. Germany and NL)
	(IT, GR, NL, PT, HU, JP, Baltic States)
	

	Body
	
	
	
	

	No of doors/seats
	
	3 / 4
	3 / 4
	3 / 4

	Length/width/height (unladen)
	mm
	4 239 / 1748 / 1421
	4 239 / 1748 / 1421
	4 239 / 1748 / 1421

	Wheelbase
	mm
	2 660
	2 660
	2 660

	Track, front/rear
	mm
	1484 / 1497
	1484 / 1497
	1484 / 1497

	Ground clearance
	mm
	145
	145
	145

	Turning circle
	m
	10.7
	10.7
	10.7

	Tank capacity
	app ltr
	53
	53
	53

	Cooling system incl heater
	ltr
	7.0 (7.6)
	7.0 (7.6)
	7.2 (7.9)

	Engine oil1)
	ltr
	4.3
	4.3
	4.3

	Weight, unladen, to DIN/EU
	kg
	1245/1320 

(1275/1350)
	1245/1320 

(1275/1350)
	1265/1340

(1290/1365)

	Max load to DIN
	kg
	490
	500
	500

	Max permissible
	kg
	1755 (1780)
	1755 (1780)
	1765 (1790)

	Max axle load, front/rear
	kg
	855 / 1020 
	855 / 1020 
	855 / 1020 

	Max trailer load (12%),

braked/unbraked
	kg
	1200 / 650
	1200 / 650
	1200 / 670

	Max roofload/max towbar downl
	kg
	75 / 75
	75 / 75
	75 / 75

	Luggage comp capacity
	ltr
	330-1150
	330-1150
	330-1150

	Air drag
	Cd x A
	0.29 x 2.09
	0.29 x 2.09
	0.30 x 2.09

	
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-four / 4
	Straight-four / 4
	Straight-four / 4

	Engine technology
	 
	Second-generation direct fuel injection

(High Precision Injection) with lean burn 

	Capacity, effective
	Cc
	1995
	1599
	1995

	Stroke / bore
	Mm
	90 / 84
	75.7 / 82
	90 / 84

	Compression ratio
	:1
	12.0 
	12.0 
	12.0 

	Fuel 
	
	RON 91-98
	RON 91-98
	RON 91-98

	Max output
	kW/hp
	902) / 122
	90 / 122
	105 / 143

	at 
	Rpm
	6 000
	6 000
	6000

	Max torque
	Nm/lb-ft
	185/136
	160/118
	190/140

	at
	Rpm
	3000 – 4250
	4250
	4250

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	80 (70) / luggage comp 
	80 (70) / luggage comp
	80 (70) / luggage comp

	Alternator
	A/W
	180 / 2520
	180 / 2520
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint tiebar spring strut axle, aluminium

	Suspension, rear
	
	Five-arm axle, lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	Mm
	284 x 22 / vented
	284 x 22 / vented
	292 x 22 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	Mm
	280 x 10
	280 x 10
	296 x 11

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control

	Safety equipment
	  
	Standard: airbags for driver and front passenger (two-stage), side airbags 
for driver and front passenger, head airbags front and rear, three-point 
inertia-reel seat belts on all seats, with belt retainer, belt latch tensioner 
and belt force limiter at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion steering; 3.0 turns; EPS Electronic Power Steering

	Steering transmission, overall
	:1
	16.0
	16.0
	16.0

	Tyres, front/rear
	
	195/55 R16 87H
	195/55 R16 87H
	195/55 R16 87H

	Rims, front/rear
	
	6.5J x 16 steel
	6.5J x 16 steel
	6.5J x 16 steel


Specifications BMW 1 Series Three-Door.


	
	
	BMW 116i
	BMW 116i
	BMW 118i

	
	
	(Germany, Romania)
	(Other EU markets)
	

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including features such as enquiry service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including features such as Parkinfo, Nationalinfo, Google directory service, News, realtime weather, 
BMW Routes, Office functions, Bluetooth audio streaming

 and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, 
Park Distance Control, cruise control, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual gearbox (optional: six-speed automatic transmission)

	Gear ratios
I
	
	4.323 (4.171)
	4.323 (4.171)
	4.323 (4.171)

	                                  
II
	
	2.456 (2.340)
	2.456 (2.340)
	2.456 (2.340)

	                                  
III
	
	1.659 (1.521)
	1.659 (1.521)
	1.659 (1.521)

	                                  
IV
	
	1.230 (1.143)
	1.230 (1.143)
	1.230 (1.143)

	                                  
V
	
	1.000 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	                                   
VI
	
	0.848 (0.691)
	0.848 (0.691)
	0.848 (0.691)

	                                   
Reverse
	
	3.938 (3.403)
	3.938 (3.403)
	3.938 (3.403)

	Final drive
	
	3.385 (3.909)
	3.636 (4.100)
	3.385 (3.909)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	13.8 (14.3)
	13.8 (14.2)
	12.0 (12.3)

	Output per litre
	kW/hp
	45.1/61.3
	56.3/76.6
	52.6/71.5

	Acceleration       0–100 km/h
	sec
	9.8 (10.7)
	10.1 (11.1)
	8.7 (9.2)



	                      Standing-start km
	sec
	31.2 (31.9)
	31.3 (32.2)
	29.7 (30.1)

	In 4th/5th gear    80–120 km/h 
	sec
	9.1/11.7
	9.9/13.2
	8.9/11.5

	Top speed
	km/h
	204 (202)
	204 (202)
	210

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Brake Energy Regeneration, Electric Power Steering,
Auto Start Stop and Gearshift Point Indicator (manual gearbox models), intelligent lightweight technology, map-controlled oil pump, a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	7.9 (8.7)
	7.9 (8.9)
	7.9 (8.7)

	Extra-urban
	ltr/100 km
	5.1 (5.4)
	5.1 (5.5)
	5.1 (5.4)

	Combined
	ltr/100 km
	6.1 (6.6)
	6.1 (6.8)
	6.1 (6.6)

	CO2
	g/km
	143 (154)
	143 (158)
	143 (154)

	Emission rating
	
	EU5 
	EU5 
	EU5 


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight).

Figures in brackets apply to automatic transmission models.

1)  Oil change

2)  BMW 116i in Austria: 89 kW/121 hp




















































