Specifications.
The New BMW 5 Series Sedan.
535i xDrive.

|  |  |  |  |  | BMW 535i xDrive |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  |  |  |  | 4 / 5 |  |
| Length/width/height 1) (unladen) | mm |  |  |  | 4899 / 1860 / 1464 |  |
| Wheelbase | mm |  |  |  | 2968 |  |
| Track, front/rear | mm |  |  |  | 1600 / 1627 |  |
| Ground clearance |  |  |  |  | 141 |  |
| Turning circle | m |  |  |  | 11.95 |  |
| Tank capacity | app ltr |  |  |  | 70 |  |
| Cooling system incl heater | ltr |  |  |  | 9.3 |  |
| Engine oil2) | ltr |  |  |  | 6.5 |  |
| Weight, unladen, to DIN/EU | Kg |  |  |  | 1765 / 1840 |  |
| Max load to DIN | Kg |  |  |  | 600 |  |
| Max permissible | kg |  |  |  | 2380 |  |
| Max axle load, front/rear | kg |  |  |  | 1170 / 1310 |  |
| Max trailer load (12%), braked/unbraked | kg |  |  |  | 2000 / 750 |  |
| Max roofload/max towbar dwnl | kg |  |  |  | 100 / 90 |  |
| Luggage comp capacity | ltr |  |  |  | 520 |  |
| Air resistance | cd x A |  |  |  | 0.29 x 2.35 |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  |  |  |  | straight-six / 4 |  |
| Engine technology |  |  |  |  |  | BMW TwinPower Turbo, direct fuel injection (High Precision Injection), fully-variable valve control (VALVETRONIC) |
| Capacity, effective | cc |  |  |  | 2979 |  |
| Stroke / bore | mm |  |  |  | 88.3 x 89.0 |  |
| Compression ratio | :1 |  |  |  | 10.0 |  |
| Fuel grade |  |  |  |  | min RON 91 |  |
| Max output | kW/hp |  |  |  | 225 / 306 |  |
| at | rpm |  |  |  | 5800 |  |
| Max torque | Nm/lb-ft |  |  |  | 400/295 |  |
| at | rpm |  |  |  | 1200-5000 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– |  |  |  | 90 / luggage comp |  |
| Alternator | A/W |  |  |  | 210 / 2940 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double wishbone axle with separate lower steering level in aluminium, smaller steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle in aluminium, steering function, with anti-squat and anti-dive, double acoustic separation |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| Diameter | mm |  |  |  | 348 x 36 / inner-vented |  |
| Brakes, rear |  | Single-piston aluminium swing-calliper disc brakes |  |  |  |  |
| Diameter | mm |  |  |  | 345 x 24 / inner-vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, networked with xDrive all-wheel-drive system in ICM Integrated Chassis Management; optional: Adaptive Drive |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, belt stoppers, belt tighteners and belt force limiters at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Hydraulic rack-and-pinion power steering with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 |  |  |  | 17.1 |  |
| Tyres, front/rear |  |  |  |  | 225/55 R17 97W |  |
| Rims, front/rear |  |  |  |  | 8J x 17 light-alloy |  |

Specifications.
The New BMW 5 Series Sedan.
535i xDrive.

|  |  |  |  |  |  |  | BMW 535i xDrive |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |
| Infotainment |  |  |  | Optional: internet, BMW Online incl Park Info, National Info, Google Directory Search, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming, and BMW Routes |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View incl Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, collision warning with brake function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  |  | Eight-speed automatic with Steptronic |  |  |  |  |
| Gear ratios I |  |  |  |  |  |  |  | 4.714 |
| II |  |  |  |  |  |  |  | 3.143 |
| III |  |  |  |  |  |  |  | 2.106 |
| IV |  |  |  |  |  |  |  | 1.667 |
| V |  |  |  |  |  |  |  | 1.285 |
| VI |  |  |  |  |  |  |  | 1.000 |
| VII |  |  |  |  |  |  |  | 0.839 |
| VIII |  |  |  |  |  |  |  | 0.667 |
| R |  |  |  |  |  |  |  | 3.295 |
| Final drive |  |  |  |  |  |  |  | 3.077 |
| Performance |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  |  |  |  | 7.8 |  |
| Output per litre | kW/hp |  |  |  |  |  | 75.5/102.7 |  |
| Acceleration 0–100 km/h | sec |  |  |  |  |  | 5.9 |  |
| Standing-start km | sec |  |  |  |  |  | 25.6 |  |
| In 4th gear 80–120 km/h | sec |  |  |  |  |  | – |  |
| Top speed | km/h |  |  |  |  |  | 250 |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration with recuperation display, Auto Start Stop, intelligent lightweight construction, on-demand operation of ancillary units, tyres with reduced roll resistance |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  |  |  |  | 10.9 |  |
| Extra-urban | ltr/100 km |  |  |  |  |  | 6.5 |  |
| Combined | ltr/100 km |  |  |  |  |  | 8.1 |  |
| CO2 | g/km |  |  |  |  |  | 189 |  |
| Emission rating |  |  |  |  |  |  | EU5 |  |

Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)

1) Height with roof aerial: 1,475 mm

2)  Oil change