Specifications.
The new BMW 5 Series Sedan. 
523i, 528i.

|  |  | BMW 523i |  |  | BMW 528i |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 |  |  | 4 / 5 |  |
| Length/width/height (unladen) | mm | 4899 / 1860 / 1464 |  |  | 4899 / 1860 / 1464 |  |
| Wheelbase | mm | 2968 |  |  | 2968 |  |
| Track, front/rear | mm | 1600 / 1627 |  |  | 1600 / 1627 |  |
| Ground clearance | mm | 141 |  |  | 141 |  |
| Turning circle | m | 11.95 |  |  | 11.95 |  |
| Tank capacity | app ltr | 70 |  |  | 70 |  |
| Cooling system incl heater | ltr | 9.0 |  |  | 9.0 |  |
| Engine oil1) | ltr | 6.5 |  |  | 6.5 |  |
| Weight, unladen, to DIN/EU | kg | 1625 / 1700 (1650 / 1725) |  |  | 1635 / 1710 (1655 / 1730) |  |
| Max load to DIN | kg | 610 |  |  | 610 |  |
| Max permissible | kg | 2235 (2260) |  |  | 2245 (2265) |  |
| Max axle load, front/rear | kg | 1070 / 1265 (1070 /1270) |  |  | 1070 / 1265 (1070 / 1270) |  |
| Max trailer load (12%), braked/unbraked | kg | 1900 / 750 (2000 / 750) |  |  | 2000 / 750 |  |
| Max roofload/max towbar dwnl | kg | 100/90 |  |  | 100/90 |  |
| Luggage comp capacity | ltr | 520 |  |  | 520 |  |
| Air resistance | Cd x A | 0.28 x 2.35 |  |  | 0.28 x 2.35 |  |
| Power Unit |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-six / 4 |  |  | Straight-six / 4 |  |
| Engine technology |  | Direct fuel injection (High Precision Injection), lean-burn |  |  |  | Direct fuel injection (High Precision Injection), lean-burn |
| Capacity, effective | cc | 2996 |  |  | 2996 |  |
| Stroke / bore | mm | 88.0 / 85.0 |  |  | 88.0 / 85.0 |  |
| Compression ratio | :1 | 12.0 |  |  | 12.0 |  |
| Fuel grade |  | min RON 91 |  |  | min RON 91 |  |
| Max output | kW/hp | 150 / 204 |  |  | 190 / 258 |  |
| at | rpm | 6100 |  |  | 6600 |  |
| Max torque | Nm/lb-ft | 270/199 |  |  | 310/228 |  |
| At | rpm | 1500-4250 |  |  | 2600-5000 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– | 80 (90) / luggage comp |  |  | 80 (90) / luggage comp |  |
| Alternator | A/W | 210 / 2940 |  |  | 210 / 2940 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle, aluminium, with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| Diameter | mm | 330 x 24 / inner-vented |  |  | 348 x 30 / inner-vented |  |
| Brakes, rear |  | Single-piston aluminium swing-calliper disc brakes |  |  |  |  |
| Diameter | mm | 330 x 20 / inner-vented |  |  | 330 x 20 / inner-vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant; optional: Dynamic Damper Control |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 17.1 |  |  | 17.1 |  |
| Tyres, front/rear |  | 225/55 R17 97W |  |  | 225/55 R17 97W |  |
| Rims, front/rear |  | 8J x 17 light-alloy |  |  | 8J x 17 light-alloy |  |

Specifications.
The new BMW 5 Series Sedan. 
523i, 528i.

|  |  |  |  | BMW 523i |  |  |  | BMW 528i |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |
| Infotainment |  |  |  |  | Opt: internet, BMW Online incl Parkinfo, National Info, Google Direct Serv, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming, BMW Routes |  |  |  |  |
| Safety |  |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  | Six-speed manual (optional: eight-speed automatic with Steptronic) |  |  |  |  |  |  |
| Gear ratios I |  | :1 | 4.323 (4.714) |  |  |  |  |  | 3.498 (4.714) |
| II |  | :1 | 2.456 (3.143) |  |  |  |  |  | 2.005 (3.143) |
| III |  | :1 | 1.659 (2.106) |  |  |  |  |  | 1.313 (2.106) |
| IV |  | :1 | 1.230 (1.667) |  |  |  |  |  | 1.000 (1.667) |
| V |  | :1 | 1.000 (1.285) |  |  |  |  |  | 0.809 (1.285) |
| VI |  | :1 | 0.848 (1.000) |  |  |  |  |  | 0.701 (1.000) |
| VII |  | :1 | – (0.839) |  |  |  |  |  | – (0.839) |
| VIII |  | :1 | – (0.667) |  |  |  |  |  | – (0.667) |
| R |  | :1 | 3.938 (3.295) |  |  |  |  |  | 3.187 (3.295) |
| Final drive |  | :1 | 3.231 (3.385) |  |  |  |  |  | 4.100 (3.385) |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  | 10.8 (11.0) |  |  |  |  | 8.6 (8.7) |  |
| Output per litre | kW/hp |  | 50.1/68.1 |  |  |  |  | 63.4/86.2 |  |
| Acceleration 0–100 km/h | sec |  | 7.9 (8.2) |  |  |  |  | 6.6 (6.7) |  |
| Standing-start km | sec |  | 28.5 (29.0) |  |  |  |  | 26.5 (26.6) |  |
| In 4th gear 80–120 km/h | sec |  | 9.0 (–) |  |  |  |  | 7.0 (–) |  |
| Top speed | km/h |  | 238 (234) |  |  |  |  | 250 |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  | Brake Energy Regeneration with recuperation display, gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance |  |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  | 10.5 (10.5) |  |  |  |  | 10.4 (10.4) |  |
| Extra-urban | ltr/100 km |  | 5.9 (5.9) |  |  |  |  | 6.3 (6.0) |  |
| Combined | ltr/100 km |  | 7.6 (7.6) |  |  |  |  | 7.8 (7.6) |  |
| CO2 | g/km |  | 177 (178) |  |  |  |  | 182 (178) |  |
| Emission rating |  |  | EU5 |  |  |  |  | EU5 |  |

Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)

Figures in brackets apply to automatic transmission models

1)  Oil change