PAGE  
BMW
Press

10/2003
Page 77

主要諸元：BMW 645Ci.

	Body


	
	645Ci

	
	
	

	No. of doors/seats

L
	
	2/2 + 2
	
	
	

	Length/width/height (unladen) 

)
	mm
	4,820/1,855/1,373
	
	
	

	Wheelbase
	mm
	2,780
	
	
	

	Track, front/rear


	mm
	1,558/1,592
	
	
	

	Turning circle
	m
	11.4
	
	
	

	Tank capacity
	approx ltr
	70
	
	
	

	Cooling system incl heater
	ltr
	14.2
	
	
	

	Engine oil


	ltr
	8.0
	
	
	

	Transmission fluid
	ltr
	1.8
	
	
	

	Final drive fluid
	ltr
	1.2
	
	
	

	Weight, unladen, to EU1
	kg
	1,690 (1,695)
	
	
	

	Max. load to DIN
	kg
	450 
	
	
	

	Max. permissible weight to DIN
	kg
	2,065
	
	
	

	Max. axle load front/rear
	kg
	1,030/1,140
	
	
	

	Max. trailer load 
	
	
	
	
	

	braked (12%); unbraked


	kg
	–
	
	
	

	Max. roofload/max towbar download
	kg
	–
	
	
	

	Luggage comp to VDA
	ltr
	450
	
	
	

	Air resistance
	cX x A
	0.29 x 2.15
	
	
	

	Engine
	
	
	
	
	

	Configuration/No. of cyls/valves 


	
	V 90/8/4
	
	
	

	Engine management
	
	Bosch ME 9.2.1
	
	
	

	Capacity
	cc
	4,398
	
	
	

	Bore/stroke
	mm
	92.0/82.7
	
	
	

	Compression ratio
	:1
	10.0 
	
	
	

	Fuel grade
	RON
	91–98
	
	
	

	Max. output
	kW/bhp
	245/333
	
	
	

	at speed
	rpm
	6,100
	
	
	

	Torque
	Nm/lb-ft
	450/332
	
	
	

	at speed
	Rpm
	3,600
	
	
	

	Electrical system
	
	
	
	
	

	Battery/installation
	Ah/–
	110/luggage comp
	
	
	

	Alternator
	A/W
	170/2,380
	
	
	

	Chassis and suspension
	
	
	
	
	

	Suspension, front
	
	Double-joint spring strut axle with track control arms and tiebars

	Suspension, rear
	
	Integral axle

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348, vented
	
	
	

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345, vented
	
	
	

	Driving stability systems
	
	ABS, DSC, CBC
	
	
	

	Steering
	
	Rack-and-pinion power steering
	

	Transmission ratio, steering, overall
	:1
	14.4
	
	
	

	Type of transmission
	
	6-speed manual (6-speed Steptronic automatic)

	Gear ratios 
	1st
	:1
	4.055 (4.17)
	
	
	

	                                  
	2nd
	:1
	2.396 (2.34)
	
	
	

	                                  
	3rd
	:1
	1.582 (1.52)
	
	
	

	                                  
	4th
	:1
	1.192 (1.14)
	
	
	

	                                  
	5th
	:1
	1.0 (0.87)
	
	
	

	                                  
	6th
	:1
	0.872 (0.69)
	
	
	

	                                   
	Rev
	:1
	3.677 (3.40)
	
	
	

	Final drive ratio
	:1
	3.23 (3.46)
	
	
	

	Tyres, front
	
	 245/45 R 18 96W RSC
	
	
	

	Tyres, rear
	
	 245/45 R 18 96W RSC
	
	
	

	Rims, front
	
	8 J x 18 alu
	
	
	

	Rims, rear 
	
	 8 J x 18 alu
	
	
	

	Performance
	
	
	
	
	

	Power-to-weight ratio (DIN)
	kg/kW
	6.6
	
	
	

	Output per litre
	kW/ltr
	55.7
	
	
	

	Acceleration  0–100 km/h
	sec
	5.6 (5.8)
	
	
	

	                           
	
	
	
	
	

	in 4th/5th gear      80–120 km/h
	sec
	5.1 / 6.7
	
	
	

	Top speed
	km/h / mph
	250/1552
	
	
	

	Fuel consumption, EU cycle
	
	
	
	
	

	Urban
	ltr/100 km
	17.2 (16.1)
	
	
	

	Extra-urban
	ltr/100 km
	8.6 (8.0)
	
	
	

	Composite
	ltr/100 km
	11.7 (10.9)
	
	
	

	CO2 
	g/km
	283 (264)
	
	
	

	Miscellaneous
	
	
	
	
	

	Emission category
	
	EU4
	
	
	



 Weight of the car in road trim (DIN) plus 75 kg for the driver and luggage.



2 Electronically limited.

[image: image1.png]Torgue [Nm]

480

440

400

360

320

280

240

200

160

120

80

40

450 Nm @ 3¢J00min-1

245 kW @

' 6100min-

/7 \

260,0

+ 240,0

+ 220,0

N\

+ 200,0

+ 180,0

+ 160,0

+ 140,0

+ 120,0

+ 100,0

+ 80,0

+ 60,0

+ 40,0

+ 20,0

0 1000 2000
Engine speed [min-1]

3000

4000

5000

6000

0,0
7000

Output [kW]



15.
エンジン性能曲線図


[image: image2.png]


16. インテリアおよびエクステリア寸法
[image: image3.png]JO—_

1592
1855

O\

A

|
}

=

SO

y/

1558
2036

<

O WMDY L8/

=

‘\

=

%AA)
G2 Ll
_w/
O I covl
0 AN
4 /

——









































