BMW
Media Information
01/04/2004

valid from:

April 2004

Specifications BMW X5

3.0i; 4.4i; 3.0d; 4.8is

	Body
	
	3.0i
	4.4i
	3,0d
	4,8is

	No. of doors / seats
	
	5 / 5
	5 / 5
	5 / 5
	5 / 5

	Length / width / height (unladen)
	mm
	4667 / 1872 / 1715
	4667 / 1872 / 1707
	4667 / 1872 / 1715
	4667 / 1925 / 1688

	Wheelbase
	mm
	2820
	2820
	2820
	2820

	Track, front / rear
	mm
	1576 / 1576
	1576 / 1576
	1576 / 1576
	1566 / 1596

	Turning circle
	m
	12,1
	12,1
	12,1
	12,1

	Fuel tank capacity
	approx. ltr
	93
	93
	93
	93

	Cooling system incl. heating
	ltr
	11,0
	14,4
	10,2
	14,4

	Engine oil
	ltr
	7,5
	8,0
	7,0
	8,0

	Transmission fluid
	ltr
	Permanent filling
	Permanent filling
	Permanent filling
	              Permanent filling

	Final drive fluid
	ltr
	Permanent filling
	Permanent filling
	Permanent filling
	              Permanent filling

	Weight, unladen (EU) 

	kg
	2070 (2095)
	2195
	2170 (2180)
	2275

	Max load (DIN)
	kg
	600
	580
	590
	500

	Max. permissible weight (DIN)
	kg
	2595 (2620)
	2700
	2685 (2695)
	2700

	Permissible axle load front/rear
	kg
	1165 / 1475
	1255 / 1485
	1240 / 1480
	1265 / 1485

	Max. trailer load 

	
	
	
	
	

	braked (12%) / unbraked
	kg
	2300 / 750
	2300 / 750
	2300 / 750
	2300 / 750

	Max. roof load/trailer nose weight
	kg
	100 / 120
	100 / 120
	100 / 120  
	100 / 120

	Luggage comp. capacity (VDA)
	ltr
	465-1550
	465-1550
	465-1550  
	465 – 1550

	Drag coefficient
	cX x A
	0,35 x 2,70
	0,36 x 2,70
	0,35 x 2,70
	0,38 x 2,70

	Engine
	
	
	
	
	

	Layout / No. of cylinders / Valves
	
	Inline / 6 / 4
	V 90 / 8 / 4
	Inline 6 / 4
	V 90 / 8 / 4

	Engine management
	
	Siemens MS 43
	Motronic ME 9.2.1
	DDE 5.06
	Motronic ME 9.2.1

	Displacement
	cc
	2979
	4398
	2993
	4799

	Bore / Stroke
	mm
	84,0 / 89,6
	92,0 / 82,7
	84,0 / 90, 0
	93,0 / 88,3

	Compression ratio
	:1
	10,2
	10,0
	18,0 
	11,0

	Fuel grade
	RON
	91-98
	98
	Diesel
	98

	Max. output
	kW / bhp
	170 / 231
	235 / 320
	160 / 218
	265 / 360

	At engine speed
	rpm
	5900
	6100
	4000
	6200

	Torque
	Nm
	300
	440
	500
	500

	At engine speed
	rpm
	3500
	3700
	2000-2750
	3500

	Electrics
	
	
	
	
	

	Battery / Location
	Ah / -
	70 / boot
	90 / boot
	90 / boot
	90 / boot

	Alternator
	A / W
	140 /1960 
	170 / 2380
	150 / 2100
	170 / 2380

	Chassis
	
	
	
	
	

	Front suspension
	
	Double-joint thrust-rod spring-strut suspension with displaced camber; small

	
	
	negative steering scrub radius; traverse force compensation; anti-dive

	Rear suspension
	
	Integral axle, multi-dimensional suspension with anti-squat and anti-dive

	Brakes, front
	
	Double-piston floating-calliper disc brakes 

	Diameter
	mm
	332, vented


	332, vented


	332, vented
	356, vented

	Brakes, rear
	
	Single-piston floating-calliper disc brakes



	Diameter
	mm
	324,
	324, vented
	324
	324, vented

	Driving Stability Systems
	
	DSC III (HDC, DBC, ABS, ASC-X, ADB-X, Trailer-Stability-Control)

	Steering
	
	Rack-and- pinion, power-assisted

	Overall ratio
	:1
	21.1
	21.1
	21.1
	21.1

	Type of transmission
	
	6-gear manual (6-gear Steptronic automatic transmission standard on 4.4i, 4.8is)

	Transmission ratio
I
	:1
	5,14 (3,42)
	4,17
	5,08 (4,17)
	4,171

	
II
	:1
	2,83 (2,22)
	2,34
	2,80 (2,34)
	2,340

	
III
	:1
	1,78 (1,60)
	1,52
	1,78 (1,52)
	1,521

	
IV
	:1
	1,26 (1,00)
	1,14
	1,26 (1,14)
	1,143

	
V
	:1
	1,00 (0,75)
	0,87
	1,00 (0,87)
	0,867

	
R
	:1
	0,83 (-)
	0,69
	0,84 (0,69)
	0,691

	Final drive ratio
	:1
	4,64 (3,03)
	3,40
	4,61 (3,40)
	3,403

	Tyres front (rear)
	
	235/65 R 17 104H
	255/55 R 18 104 H
	235/65R17 104H
	 275/40(315/35)R 20 W

	Wheels front / rear


	
	                      7,5 J x 17 Alu
	7,5 J x 18 
	7,5 J x 17 
	9,5 J x 20 / 10,5 J x 20 

	Performance
	
	
	
	
	

	Power to weight ratio (DIN)
	kg/kW
	11,7 (11,9)
	9,0
	13,1 (13,2)
	8,3

	Output per litre
	kW/ltr
	57,1
	53,4
	53,5
	55,2

	Acceleration        0 - 62 mph
	sec
	8,3 (8,8)
	7,0
	8,3 (8,8)
	6,1

	                             0 -1000 m
	sec
	29,4 (30,2)
	27,3
	29,2 (29,9)
	26,2

	50 – 75 mph in 4th gear
	sec
	7,6 (-)
	-
	6,4 (-)
	-

	Top speed
	km/h
	210 (2153)
	210 (2403)
	210 (2103)
	246

	Fuel consumption (EU cycle)
	
	
	
	
	

	In town
	ltr/100km 
	17,8 (18,1)
	18,2
	11,4 (12,0)
	18,7

	Out of town
	ltr/100km 
	9,7 (9,9)
	10,2
	7,0 (8,0)
	10,5

	Overall
	ltr/100km 
	12,7 (12,9)
	13,1
	8,6 (9,4)
	13,5

	CO2 
	g/km
	307 (312)
	317
	229 (250)
	324

	Miscellaneous
	
	
	
	
	

	Emission classification
	
	EU3 N1 G1
	EU3 N1 G1
	EU3 N1 G3
	EU3 N1 G1

	Wading depth (at 7 km/h)
	mm
	500
	500
	500
	500

	Sloping angle front/rear
	°
	25 / 24
	26 / 23
	25 / 24
	26 /22

	Ramp angle
	°
	19
	18
	17
	19

	Ground clearance between axles
	mm
	204
	203
	176
	2074

	Perm. all-wheel drive distr. of power  f:r
	
	variable
	variable
	variable
	variable


Technical specifications apply to Europe (ECE) / weight specifications relevant for homologation may apply for Germany only. 

The values given in parenthesis apply to the automatic transmission.   

1) Weight of the car in road trim (DIN) plus 75 kg for driver and luggage
3)  With Sport package

2) Enhancement possible under certain conditions    
4)  In Off road mode









