[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 5 Series Saloon.
520d Special Edition, 520d
	
	
	BMW 520d
Special Edition
Saloon
	
	BMW 520d
Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4841 / 1846 / 1468
	
	4841 / 1846 / 1468

	Wheelbase
	mm
	2888
	
	2888

	Track, front/rear
	mm
	1558 / 1581
	
	1558 / 1581

	Ground clearance
	mm
	142
	
	142

	Turning circle
	m
	11.4
	
	11.4

	Tank capacity
	approx ltr
	70
	
	70

	Cooling system incl heater
	ltr
	
	
	

	Engine oil1)
	ltr
	5.5
	
	5.5

	Weight, unladen, to DIN/EU
	kg
	1510 / 1585

(1520 / 1595)
	
	1510 / 1585

(1520 / 1595)

	Max load to DIN
	kg
	540
	
	540

	Max permissible
	kg
	2050 (2060)
	
	2050 (2060)

	Max axle load, front/rear
	kg
	990 / 1170
	
	990 / 1170

	Max trailer load (12%),
braked/unbraked
	kg
	1800 / 1800 

(1800) / 750


	
	1800 / 1800 

(1800) / 750



	Max roofload/towbar download
	kg
	100/90
	
	100/90

	Luggage comp capacity
	ltr
	520
	
	520

	Air resistance
	cD x A
	0.28 (0.27) x 2.26
	
	0.28 (0.27) x 2.26

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-4 / 4
	
	Straight-4 / 4

	Engine technology
	 
	Common-rail direct injection, turbocharger with variable intake geometry
	
	Common-rail direct injection, turbocharger
 with variable intake geometry

	Capacity, effective
	cc
	1995
	
	1995

	Stroke/bore
	mm
	90 / 84
	
	90 / 84

	Compression ratio
	:1
	16.0
	
	16.0

	Fuel grade
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	120 / 163
	
	130 / 177

	at 
	rpm
	4000
	
	4000

	Max torque
	Nm / lb-ft
	350 / 258
	
	350 / 258

	at 
	rpm
	1750-2500
	
	1750-3000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	     80 / luggage compartm compartment
	
	80 / luggage compartment

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint tiebar spring strut axle, aluminium, 

compensation of lateral forces, anti-dive

	Suspension, rear
	
	Integral-IV multi-arm axle, aluminium, multi-dimensional suspension
with anti-squat and anti-dive, air suspension with
automatic self-levelling

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	324 x 30 / vented
	
	324 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 20 / vented
	
	320 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, 
Fading Compensation, Start-Off Assistant

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator, optional: side airbags and belt latch tensioners at the rear

	Steering
	
	Rack-and-pinion steering with Servotronic; optional: Active Steering

	Transmission ratio, overall
	:1
	13.7
	
	13.7

	Tyres, front/rear
	
	225/55 R16 95W
	
	225/55 R16 95W

	Rims, front/rear
	
	7J x 16 light-alloy
	
	7J x 16 light-alloy


Specifications BMW 5 Series Saloon.
	
	
	BMW 520d
Special Edition
Saloon
	
	BMW 520d
Saloon

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, Integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,
Park Distance Control, BMW Night Vision, Head-Up Display,
Active Cruise Control with Stop & Go, Lane Departure Warning, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic with Steptronic)

	Gear ratios
I
	:1
	5.140 (4.171)
	
	5.140 (4.171)

	                                  
II
	:1
	2.830 (2.340)
	
	2.830 (2.340)

	                                  
III
	:1
	1.804 (1.521)
	
	1.804 (1.521)

	                                  
IV
	:1
	1.257 (1.143)
	
	1.257 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (0.867)

	                                   
VI
	:1
	0.831 (0.691)
	
	0.831 (0.691)

	                                   
R
	:1
	4.638 (3.403)
	
	4.638 (3.403)

	Final drive ratio
	:1
	2.563 (3.231)
	
	2.563 (3.231)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	12.6 (12.7)
	
	11.6 (11.7)

	Output per litre
	kW/hp
	60.2 / 81.9
	
	65.2 / 88.7

	Acceleration
    0–100 km/h
	sec
	8.5 (8.6)
	
	8.3 (8.4)

	Standing-start km
	sec
	29.6 (29.7)
	
	29.1 (29.2)

	In 4th/5th gear         80–120 km/h
	sec
	7.2 / 9.0 (-)
	
	7.1 (-)

	Top speed
	km/h
	227 (225)
	
	231 (229)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), tyres with reduced roll resistance, intelligent lightweight construction, on-demand control of ancillary units, air flap control, Varioserv steering assistance pump,
a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	6.5 (7.5)
	
	6.5 (7.5)

	Extra-urban
	ltr/100km
	4.3 (4.6)
	
	4.3 (4.6)

	Combined
	ltr/100km
	5.1 (5.6)
	
	5.1 (5.6)

	CO2
	g/km
	136 (149)
	
	136 (149)

	Emission rating
	
	EU5  
	
	EU5  


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































