[image: image1.png]


BMW
Media
Information

8/2009

Valid from 9/2009
BMW
Media
Information

8/2009

Valid from 9/2009

Specifications BMW 5 Series Saloon.
520i, 523i, 525i
	
	
	BMW 520i Saloon
	BMW 523i Saloon
	BMW 525i Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	4 / 5
	4 / 5

	Length/width/height (unladen)
	mm
	4841 / 1846 / 1468
	4841 / 1846 / 1468
	4841 / 1846 / 1468

	Wheelbase
	mm
	2888
	2888
	2888

	Track, front/rear
	mm
	1558 / 1581
	1558 / 1581
	1558 / 1581

	Ground clearance
	mm
	142
	142
	142

	Turning circle
	m
	11.4
	11.4
	11.4

	Tank capacity
	approx ltr
	70
	70
	70

	Cooling system incl heater
	ltr
	
	10.0 (10.5)
	10.0 (10.5)

	Engine oil1)
	ltr
	4.25
	6.5
	6.5

	Weight, unladen, to DIN/EU
	kg
	1460 / 1535

(1490 / 1565)
	1500 / 1575

(1525 / 1600)
	1510 / 1585

(1530 / 1605)

	Max load to DIN
	kg
	540
	560
	560

	Max permissible
	kg
	2000 (2030)
	2060 (2085)
	2070 (2090)

	Max axle load, front/rear
	kg
	980 / 1150
	980 / 1170
	980 / 1180

	Max trailer load (12%),
braked/unbraked
	kg
	1600 / 1800 

(1800) / 750


	1600 / 1800 

(1800) / 750


	1800 / 2000 

(2000) / 750

	Max roofload/towbar download
	kg
	100/90
	100/90
	100/90

	Luggage comp capacity
	ltr
	520
	520
	520

	Air resistance
	cD x A
	0.27 (0.26) x 2.26
	0.27 (0.26) x 2.26
	0.27 (0.26) x 2.26

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-4 / 4
	Straight-6 / 4
	Straight-6 / 4

	Engine technology
	 
	Second-generation
direct injection (High Precision  Injection)
with lean burn
	Second-generation
direct injection (High Precision  Injection)
with lean burn
	Second-generation
direct injection (High Precision  Injection)
with lean burn

	Capacity, effective
	cc
	1995
	2497
	2996

	Stroke/bore
	mm
	90 / 84
	78.8 / 82.0
	88.0 / 85.0

	Compression ratio
	:1
	12.0
	12.0
	12.0

	Fuel grade
	
	min RON 91
	min RON 91
	min RON 91

	Max output
	kW/hp
	125 / 170
	140 / 190
	160 / 218

	at 
	rpm
	5700 (6700)
	6100
	6100

	Max torque
	Nm / lb-ft
	210 / 155
	240 / 177
	270 / 199

	at 
	rpm
	4250
	3500-5000
	2400-4200

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	80 / luggage compartm
	80 / luggage compartm
	80 / luggage compartm

	Alternator
	A/W
	180 / 2520
	155 / 2170
	155 / 2170

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint tiebar spring strut axle, aluminium, 

compensation of lateral forces, anti-dive

	Suspension, rear
	
	Integral-IV multi-arm axle, aluminium, multi-dimensional suspension
with anti-squat and anti-dive, air suspension with
automatic self-levelling

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	310 x 24 / vented
	324 x 30 / vented
	324 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	320 x 20 / vented
	320 x 20 / vented
	320 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, 
Fading Compensation, Start-Off Assistant, optional: Dynamic Drive (BMW 525i)

	Safety features
	  
	Standard: airbags and side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator, optional: side airbags and belt latch tensioners at the rear, optional: side airbags and belt latch tensioners at the rear

	Steering
	
	Rack-and-pinion steering with Servotronic;
optional: Active Steering (BMW 523i, BMW 525i)

	Transmission ratio, overall
	:1
	13.7
	13.7
	13.7

	Tyres, front/rear
	
	225/55 R16 95V
	225/55 R16 95V
	225/55 R16 95W

	Rims, front/rear
	
	7J x 16 light-alloy
	7J x 16 light-alloy
	7J x 16 light-alloy


Specifications BMW 5 Series Saloon.
	
	
	BMW 520i Saloon
	BMW 523i Saloon
	BMW 525i Saloon

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, Integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,
Park Distance Control, BMW Night Vision, Head-Up Display,
Active Cruise Control with Stop & Go, Lane Departure Warning, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic with Steptronic)

	Gear ratios
I
	:1
	4.323 (4.171)
	4.323 (4.171)
	4.323 (4.171)

	                                  
II
	:1
	2.456 (2.340)
	2.456 (2.340)
	2.456 (2.340)

	                                  
III
	:1
	1.659 (1.521)
	1.659 (1.521)
	1.659 (1.521)

	                                  
IV
	:1
	1.230 (1.143)
	1.230 (1.143)
	1.230 (1.143)

	                                  
V
	:1
	1.000 (0.867)
	1.000 (0.867)
	1.000 (0.867)

	                                   
VI
	:1
	0.648 (0.691)
	0.648 (0.691)
	0.648 (0.691)

	                                   
R
	:1
	3.938 (3.403)
	3.938 (3.403)
	3.938 (3.403)

	Final drive ratio
	:1
	3.636 (4.100)
	3.231 (3.727)
	3.154 (3.636)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	11.7 (11.9)
	10.7 (10.9)
	9.4 (9.6)

	Output per litre
	kW/hp
	62.7 / 85.3
	56.1 / 76.3
	53.4 / 72.6

	Acceleration
    0–100 km/h
	sec
	8.7 (9.6)
	8.2 (8.7)
	7.1 (7.7)

	Standing-start km
	sec
	29.4 (30.0)
	28.6 (29.2)
	27.2 (27.8)

	In 4th/5th gear         80–120 km/h
	sec
	9.1 (-)
	9.0 (-)
	7.8 (-)

	Top speed
	km/h
	224
	237 (236)
	248 (246)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), tyres with reduced roll resistance, intelligent lightweight construction, on-demand control of ancillary units, air flap control, Varioserv steering assistance pump,
a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	9.2 (9.4)
	10.4 (10.6)
	10.6 (10.7)

	Extra-urban
	ltr/100km
	5.4
	5.8 (6.0)
	5.8 (6.0)

	Combined
	ltr/100km
	6.7 (6.9)
	7.5 (7.7)
	7.6 (7.7)

	CO2
	g/km
	162 (164)
	177 (181)
	179 (182)

	Emission rating
	
	EU4  
	EU4  
	EU4  


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































