[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

	Body
	
	MINI John Cooper Works
	MINI John Cooper Works Clubman
	

	No. of doors/seats
	 
	3/4
	5/5 (optionally 4)
	

	Length/width/height (unladen)
	mm
	3,714/1,683/1,407
	3,958/1,683/1,432
	

	Wheelbase
	mm
	2,467
	2547
	

	Track width front/rear
	mm
	1,453/1,461
	1,453/1,461
	

	Turning circle
	m
	10.7
	11.0
	

	Fuel tank capacity
	approx. l
	50
	50
	

	Cooling system incl. heating
	l
	5.2
	5.2
	

	Engine oil
	l
	4.2
	4.2
	

	Gearbox oil inc. final drive
	l
	lifetime
	lifetime
	

	Unladen weight to DIN/EU1)
	kg
	1,130/1,205
	1,205/1,280
	

	Payload to DIN
	kg
	450
	485
	

	Max. authorised weight to DIN
	kg
	1,580
	1,690
	

	Permitted axle load front/rear
	kg
	885/755
	865/855
	

	Permitted trailer weight2)
	
	
	
	

	braked (12%)/unbraked
	kg
	not possible
	not possible
	

	Permitted roof load/permitted vertical load
	kg
	75/–
	75/–
	

	Luggage compartment capacity to DIN
	l
	160–680


	260–930


	

	Aerodynamic drag
	cX/ A/ cX x A
	0.36/1.97/0.71
	0.35/2.02/0.71
	

	Engine
	
	
	
	

	Type/no. of cylinders/valves
	
	in-line 4/4
	in-line/4/4
	

	Fuel preparation
	
	MED 17.2
	MED 17.2
	

	Capacity
	cc
	1,598
	1,598
	

	Bore/stroke
	mm
	77/ 85.8
	77/ 85.8
	

	Compression ratio
	:1 
	10.0 : 1
	10.0 : 1
	

	Fuel
	RON
	91–98
	91–98
	

	Output
	kW/PS
	155/ 211
	155/ 211
	

	at engine speed
	rpm 
	6,000
	6,000
	

	Torque (with overboost)
	Nm
	260 (280)
	260 (280)
	

	at engine speed (with overboost)
	rpm
	1,850-5,600 (2,000-5,300)
	1,850-5,600 (2,000-5,300)
	

	Electrical system
	
	
	
	

	Battery/installation
	Ah/–
	46/ engine comp.
	46/ engine comp.
	

	Alternator
	A/W
	120/ 1,680
	120/ 1,680
	

	Chassis
	
	
	
	

	Suspension, front
	
	Single-joint Mc Pherson spring strut with brake squat compensation
	

	Suspension, rear
	
	Longitudinal control arm with centrally mounted transverse links, z axle
	

	Brakes, front
	
	disc, vented
	

	Diameter
	mm
	316 x 22
	

	Brakes, rear
	
	disc 
	

	Diameter
	mm
	280 x 10
	

	Driving stability systems
	
	Hydraulic 2-circle brake system with antilock system (ABS), Electronic Brakeforce distribution (EBD), Cornering Brake Control (CBC), traction assistance (ASC+T), Driving Stability Control (DSC), Dynamic Stability Control (DTC) and Electronic Differential Lock Control (EDLC). Handbrake acts mechanically on rear wheels.
	

	Steering
	
	Rank and pinion steering with electrical steering support (EPS)
	

	Steering transmission, overall
	:1 
	14.1
	14.1
	

	Type of gearbox
	
	6-speed manual transmission
	6-speed manual transmission
	

	Gear ratios
I
	:1 
	3.308
	3.308
	

	
II
	:1 
	2.130
	2.130
	

	
III
	:1
	1.483
	1.483
	

	
IV
	:1 
	1.139
	1.139
	

	
V
	:1 
	0.949
	0.949
	

	
VI
	:1 
	0.816
	0.816
	

	
R
	:1 
	3.231

	3.231

	

	Final drive
	:1
	3.647
	3.647
	

	Tyres
	
	205/45 R17 84W RSC
	205/45 R17 84W RSC
	

	Rims
	
	7J x 17LM
	7J x 17LM
	

	Performance
	
	
	
	

	Power-to-weight ratio to DIN
	kg/kW
	7.3
	7.8
	

	Output per litre
	kW/l
	97.0
	97.0
	

	Acceleration
0–100 km/h
	s
	6.5
	6.8
	

	
0–1000 m
	s
	26.3
	26.7
	

	in 4th/5th gear
80–120 km/h
	s
	5.2/ 6.2
	5.4/ 6.6
	

	Top speed
	km/h
	238
	238
	

	Fuel consumption (EU cycle)
	
	
	
	

	Urban
	l/100 km
	9.2
	9.3
	

	Extra-urban
	l/100 km
	5.6
	5.7
	

	Composite
	l/100 km
	6.9
	7.0
	

	CO2 
	g/km
	165
	167
	

	Miscellaneous
	
	
	
	

	Emission rating
	
	EU4
	EU4
	

	Ground clearance
	mm
	130
	135
	


Specifications 
MINI John Cooper Works and MINI John Cooper Works Clubman. (February 2008)






1 Weight of car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain conditions.


[image: image1.png][image: image2.png]