[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications 
MINI Cooper Convertible, MINI Cooper S Convertible. (November 2008)
	
	
	MINI Cooper Convertible 
	MINI Cooper S Convertible 

	Body 
	
	
	

	No of doors/seats 
	  
	2/4 
	2/4 

	Length/width/height (unladen) 
	mm 
	3,699/1,683/1,414 
	3,714/1,683/1,414 

	Wheelbase 
	mm 
	2467 
	2,467 

	Track, front/rear 
	mm 
	1,459/1,467 
	1,453/1,461 

	Turning circle 
	m 
	10.7 
	10.7 

	Tank capacity 
	approx ltr 
	40 
	50 

	Cooling system incl heater 
	ltr 
	5.2 
	5.2 

	Engine oil 
	ltr 
	4.2 
	4.2 

	Transmission fluid incl final drive 
	ltr 
	Lifetime 
	Lifetime  

	Weight, unladen, to DIN/EU1  
	kg 
	1,165/1,240 (1,205/1,280) 
	1,230/1,305 (1,255/1,330) 

	Max load to DIN 
	kg 
	430 
	430 

	Max permissible weight 
	kg 
	1,595 (1,635) 
	1,660 (1,685) 

	Max permissible axle load front/rear 
	kg 
	840/780 (880/780) 
	880/795 (905/795) 

	Max permissible trailer load2 
	kg 
	– 
	– 

	braked (12%)/unbraked 
	kg 
	– 
	– 

	Max roof load/max towbar download 
	kg 
	– 
	– 

	Luggage comp capacity 
	Ltr 
	125/170/660 
	125/170/660 

	Air drag cd/A/cd x A 
	–/m2/m2 
	0.35/1.97/0.69 
	0.37/ 1.99/0.74 

	
	
	

	Power Unit 
	
	

	Configuration/No of cyls/valves 
	
	Straight/4/4 
	Straight/4/4 

	Engine management 
	
	MEV 17.2 
	MED 17.2 

	Capacity 
	cc 
	1,598 
	1,598 

	Bore/stroke 
	mm 
	77.0/85.8 
	77.0/ 85.8 

	Compression ratio 
	:1  
	11.0 
	10.5 

	Fuel grade 
	RON 
	91–98 
	91–98 

	Max output 
	kW/hp 
	88/120 
	128/175 

	at 
	rpm 
	6,000 
	5,500 

	Max torque (with Overboost) 
	Nm/lb-ft 
	160/118 
	240/177 (260/192) 

	at 
	rpm 
	4,250 
	1,600–5,000 

	
	
	

	Electrical System 
	
	

	Battery/installation 
	Ah/– 
	46/front 
	46/front 

	Alternator 
	A/W 
	120/1,680 
	120/1,680 

	
	
	

	Chassis and Suspension 
	
	

	Suspension, front 
	
	  Single-joint Mc Pherson spring strut axle with anti-dive 

	Suspension, rear 
	
	  Longitudinal arm axle with centrally mounted track arms, z-axle 

	Brakes, front 
	
	Discs, vented 

	Diameter 
	mm 
	280 x 22  
	294 x 22 

	Brakes, rear 
	
	Discs 

	Diameter 
	mm 
	259 x 10 
	259 x 10 

	Driving stability systems 
	
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD) and Cornering Brake Control (CBC), traction control (ASC+T), Dynamic Stability Control (DSC), optional Dynamic Traction Control (DTC) and Electronic Differential Lock Control (EDLC). Handbrake acting mechanically on rear wheels 

	
	
	

	Steering 
	
	Electric Power Steering (EPS); 2,4 turns from lock to lock 

	Steering transmission ratio, overall 
	:1  
	14.1 
	14.1 

	Transmission 
	
	Six-speed manual (six-speed automatic)  
	

	Gear ratios
I 
	:1  
	3.214 (4.148) 
	3.308 (4.044) 

	
II 
	:1  
	1.792 (2.370) 
	2.130 (2.371) 

	
III 
	:1  
	1.194 (1.556) 
	1,483 (1,556) 

	
IV 
	:1  
	0.914 (1.155) 
	1.139 (1.159) 

	
V 
	:1  
	0.784 (0.859) 
	0.949 (0.852) 

	
VI 
	:1  
	0.683 (0.686) 
	0.816 (0.672) 

	Reverse 
	:1 
	4.353 (4.103) 
	3.647 (3.683) 

	Tyres  
	  
	175/65 R15 84H 
	195/55 R16 87V RSC 

	Rims  
	  
	5.5J x 15 light alloy 
	6.5J x 16 light alloy 

	
	
	

	Performance 
	
	

	Power-to-weight ratio to DIN 
	kg/kW 
	13.2 (13.7) 
	9.6 (9.8) 

	Output per litre 
	kW/hp 
	55.1/74.9 
	80.1/108.9 

	Acceleration
0–100 km/h 
	sec 
	9.8 (11.1) 
	7.4 (7.7) 

	
Standing-start km 
	sec 
	31.0 (32.1) 
	28.1 (28.4) 

	In 4th/5th gear
80–120 km/h 
	sec 
	10.5/13.3 
	6.2/7.9 

	Top speed 
	km/h 
	198 (191) 
	222 (217) 

	
	
	

	Fuel Consumption in EU Cycle 
	
	

	Urban 
	ltr/100 km 
	7.4 (9.3) 
	8.1 (9.9) 

	Extra-urban 
	   ltr/100 km 
	4.7 (5.2) 
	5.4 (5.4) 

	Combined 
	ltr/100 km 
	5.7 (6.7) 
	6.4 (7.1) 

	CO2  
	g/km 
	137 (161) 
	153 (170) 

	
	
	

	Miscellaneous 
	
	  

	Emission rating 
	– 
	EU 4 
	EU 4 

	Ground clearance (unladen) 
	mm 
	139 
	130 

	Max speed when opening/
closing roof roof 
	km/h 
	30 
	30 

	Max speed when opening/
closing sliding roof 
	km/h 
	120 
	120 


(Status: November 2008. Figures in brackets apply to models with automatic transmission)

1 Weight of the ready for use vehicle (DIN) plus 75 kg for the driver and luggage.
2 Under certain conditions increases may be possible.







1 Weight of car in road trim (DIN) plus 75 kg for driver and luggage.

2 Deviations are possible under certain conditions.


[image: image1.png][image: image2.png]