[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 3 Series Coupé.
330d, 335d 
	
	
	BMW 330d Coupé
	
	BMW 335d Coupé

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	
	2 / 4

	Length/width/height (unladen)
	mm
	4580 / 1782 / 1395
	
	4580 / 1782 / 1375

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1500 / 1513
	
	1500 / 1507

	Ground clearance
	mm
	143
	
	143

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
	approx ltr
	61
	
	61

	Cooling system incl heater
	ltr
	7.9
	
	8.2

	Engine oil1)
	ltr
	7.2
	
	7.5

	Weight, unladen, to DIN/EU
	kg
	1525 / 1600

(1540 / 1615)
	
	1570 / 1645



	Max load to DIN
	kg
	480
	
	480

	Max permissible
	kg
	2005 (2020)
	
	2050

	Max axle load, front/rear
	kg
	970 / 1080
	
	990 / 1090

	Max trailer load (12%),
braked/unbraked
	kg
	1800 / 750
	
	1800 / 750

	Max roofload/towbar download
	kg
	75 / 75
	
	75 / 75 

	Luggage comp capacity
	ltr
	440
	
	430

	Air resistance
	cD x A
	0.28 x 2.08
	
	0.30 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	
	Straight-6 / 4

	Engine technology
	 
	Common-rail 

direct injection, turbocharger with variable intake geometry
	
	Common-rail 

direct injection, 

Variable Twin Turbo

	Capacity, effective
	cc
	2993
	
	2993

	Stroke/bore
	mm
	90.0 / 84.0
	
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	
	17.0 

	Fuel grade
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	180 / 245
	
	210 / 286

	at 
	rpm
	4000
	
	4400

	Max torque
	Nm / lb-ft
	520 / 383
	
	580 / 427

	at 
	rpm
	1750-3000
	
	1750-2250

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	90 / luggage compartment
	
	90 / luggage compartment

	Alternator
	A/W
	180 / 2520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	Double-joint spring strut axle, aluminium, displaced castor, 

small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel construction

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 30 / vented
	
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	336 x 22 / vented
	
	336 x 22 / vented 

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), 

CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, with belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator

	Steering
	
	Rack-and-pinion power steering

	Transmission ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	225/45 R17 91 W RSC
	
	225/45 R17 91W RSC / 255/40 R17 94 W RSC

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J8.5J x 17 light-alloy


Specifications BMW 3 Series Coupé.
	
	
	BMW 330d Coupé
	
	BMW 335d Coupé

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, 

High-Beam Assistant, Park Distance Control, Cruise Control, 

Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic)
	
	Six-speed automatic

	Gear ratios
I
	:1
	5.080 (4.171)
	
	4.171 

	                                  
II
	:1
	2.804 (2.340)
	
	2.340

	                                  
III
	:1
	1.783 (1.521)
	
	1.521

	                                  
IV
	:1
	1.260 (1.143)
	
	1.143

	                                  
V
	:1
	1.000 (0.867)
	
	0.867

	                                   
VI
	:1
	0.835 (0.691)
	
	0.691

	                                   
R
	:1
	4.607 (3.403)
	
	3.403

	Final drive ratio
	:1
	2.353 (2.813)
	
	2.813

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	8.5 (8.6)
	
	7.5

	Output per litre
	kW/hp
	60.1 / 81.7
	
	70.2 / 95.5

	Acceleration
 0–100 km/h
	sec
	6.0 (6.1)
	
	5.9

	Standing-start km
	sec
	25.7 (25.9)
	
	25.1

	In 4th/5th gear        80–120 km/h
	sec
	4.7 / 6.0 (-)
	
	- 

	Top speed
	km/h
	250
	
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (BMW 330d manual model), on-demand control of ancillary units, intelligent lightweight construction, active air flap control (BMW 330d), tyres with reduced roll resistance, oil pump with map control, a/c compressor detachable

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	7.3 (8.0) 
	
	9.1

	Extra-urban
	ltr/100km
	4.8 (5.2)
	
	5.3 

	Combined
	ltr/100km
	5.7 (6.2)
	
	6.7 

	CO2
	g/km
	152 (164)
	
	177

	Emission rating
	
	EU5
	
	EU4  

	
	
	
	
	


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































