[image: image1.png]


BMW
Media
Information

8/2009

Valid from
9/2009
BMW
Media
Information

8/2009

Valid from
9/2009

Specifications BMW 3 Series Coupé.
330i, 335i
	
	
	BMW 330i Coupé
	
	BMW 335i Coupé

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	2 / 4
	
	2 / 4

	Length/width/height (unladen)
	mm
	4580 / 1782 / 1395
	
	4580 / 1782 / 1395

	Wheelbase
	mm
	2760
	
	2760

	Track, front/rear
	mm
	1500 / 1513
	
	1500 / 1507

	Ground clearance
	mm
	143
	
	143

	Turning circle
	m
	11.0
	
	11.0

	Tank capacity
	approx ltr
	63
	
	63

	Cooling system incl heater
	ltr
	8.2 (8.5)
	
	8.2

	Engine oil1)
	ltr
	6.5
	
	6.5

	Weight, unladen, to DIN/EU
	kg
	1465 / 1540

(1800 / 1555)
	
	1525 / 1600

(1540 / 1615)

	Max load to DIN
	kg
	480
	
	480

	Max permissible
	kg
	1950 (1965)
	
	2005 (2020)

	Max axle load, front/rear
	kg
	930 / 1070
	
	960 / 1080

	Max trailer load (12%),
braked/unbraked
	kg
	1700 / 750
	
	1700 / 750

	Max roofload/towbar download
	kg
	75 / 75
	
	75 / 75 

	Luggage comp capacity
	ltr
	440
	
	440

	Air resistance
	cD x A
	0.27 x 2.08
	
	0.30 x 2.08

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves


	
	Straight-6 / 4
	
	Straight-6 / 4

	Engine technology
	 
	Second-generation 

direct injection (High Precision Injection), 

lean burn
	
	BMW Twin Turbo,

second-generation

direct injection (High Precision Injection)

	Capacity, effective
	cc
	2996
	
	2979

	Stroke/bore
	mm
	88.0 / 85.0
	
	89.6 / 84.0

	Compression ratio
	:1
	12.0
	
	10.2 

	Fuel grade
	
	min RON 91
	
	min RON 95

	Max output
	kW/hp
	200 / 272
	
	225 / 306

	at 
	rpm
	6700
	
	5800

	Max torque
	Nm / lb-ft
	320 / 236
	
	400 / 295

	at 
	Rpm
	2750-3000
	
	1300-5000

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	80 / luggage compartment
	
	80 / luggage compartment

	Alternator
	A/W
	155 / 2170 (180 / 2520)
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety 
	

	Suspension, front
	
	                Double-joint spring strut axle, aluminium, displaced castor, 

    small positive steering roll radius, compensation of lateral forces, anti-dive

Five-arm axle in lightweight steel construction


	Suspension, rear
	
	Five-arm axle in lightweight steel construction

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	Mm
	348 x 30 / vented
	
	348 x 30 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	Mm
	336 x 22 / vented
	
	336 x 22 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking,

 Fading Compensation, Start-Off Assistant

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel seat belts on all seats, belt restrainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, tyre defect indicator

	Steering
	
	Rack-and-pinion power steering

	Transmission ratio, overall
	:1
	16.0
	
	16.0

	Tyres, front/rear
	
	225/45 R17 91W RSC
	
	225/45 R17 91W RSC / 

255/40 R17 94W RSC

	Rims, front/rear
	
	8J x 17 light-alloy
	
	8J8.5J x 17 light-alloy


Specifications BMW 3 Series Coupé.
	
	
	BMW 330i Coupé
	
	BMW 335i Coupé

	

	BMW ConnectedDrive
	
	
	
	

	Comfort 
	
	Optional: BMW Assist incl Enquiry Service, remote-control functions and
V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google Directory Service, News, real-time weather, office functions, BMW Routes and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,
Park Distance Control, Active Cruise Control, 

Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type 
	
	Six-speed manual (optional: six-speed automatic,

BMW 335i: seven-speed double-clutch)

	Gear ratios
I
	:1
	4.350 (4.171)
	
	4.055 (4.780) 

	                                  
II
	:1
	2.496 (2.340)
	
	2.396 (3.056)

	                                  
III
	:1
	1.665 (1.521)
	
	1.582 (2.153)

	                                  
IV
	:1
	1.230 (1.143)
	
	1.192 (1.678)

	                                  
V
	:1
	1.000 (0.867)
	
	1.000 (1.390)

	                                   
VI
	:1
	0.851 (0.691)
	
	0.872 (1.203)

	                                   
VII
	:1
	
	
	 (1.000)

	                                         R 
	:1
	3.926 (3.403)
	
	3.677 (4.454)

	Final drive ratio
	
	3.154 (3.636)
	
	3.077 (2.563)

	Performance
	
	
	
	

	Power-to-weight ratio 
	kg/kW
	
	
	

	Output per litre
	kW/hp
	7.3 (7.4)
	
	6.8

	Acceleration
   0–100 km/h
	sec
	66.8 / 90.8
	
	75.5 / 102.7

	Standing-start km
	sec
	6.0 (6.2)
	
	5.5 (5.4)

	In 4th/5th gear         80–120 km/h
	sec
	25.4 (25.6)
	
	24.5 (24.4)

	Top speed
	km/h
	6.1 / 7.6 (-)
	
	5.2 / 6.2 (-) 

	
	
	250
	
	250

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, gearshift point indicator (manual model), on-demand control of ancillary units, intelligent lightweight construction, tyres with reduced roll resistance, oil pump with map control, a/c compressor detachable

 

	
	
	
	
	

	Fuel Con in EU Cycle
	
	
	

	Urban
	ltr/100km
	10.0 (10.2) 
	
	13.2 (12.5)

	Extra-urban
	ltr/100km
	5.9
	
	6.7 

	Combined
	ltr/100km
	7.4 (7.5)
	
	9.1 (8.8) 

	CO2
	g/km
	173 (175)
	
	218 (210)

	Emission rating
	
	EU5
	
	EU4  

	
	
	
	
	


Specifications apply to ACEA markets/homologation data apply in part only to Germany (weight).

Figures in brackets apply to models with automatic transmission.

1)  Oil change




















































