Specifications.
The new BMW 5 Series Sedan. 
535i, 550i.

|  |  | BMW 535i |  |  | BMW 550i |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 |  |  | 4 / 5 |  |
| Length/width/height (unladen) | mm | 4899 / 1860 / 1464 |  |  | 4899 / 1860 / 1464 |  |
| Wheelbase | mm | 2968 |  |  | 2968 |  |
| Track, front/rear | mm | 1600 / 1627 |  |  | 1600 / 1627 |  |
| Ground clearance | mm | 141 |  |  | 141 |  |
| Turning circle | m | 11.95 |  |  | 11.95 |  |
| Tank capacity | app ltr | 70 |  |  | 70 |  |
| Cooling system incl heater | ltr | 9.3 |  |  | 11,4 |  |
| Engine oil1) | ltr | 6.5 |  |  | 8,5 |  |
| Weight, unladen, to DIN/EU | kg | 1685 / 1760 (1700 / 1775) |  |  | 1830 / 1905 |  |
| Max load to DIN | kg | 610 |  |  | 600 |  |
| Max permissible | kg | 2295 (2310) |  |  | 2430 |  |
| Max axle load, front/rear | kg | 1100 / 1275 |  |  | 1195 / 1310 |  |
| Max trailer load (12%), braked/unbraked | kg | 2000 / 750 |  |  | 2000 / 750 |  |
| Max roofload/max towbar dwnl | kg | 100/90 |  |  | 100/90 |  |
| Luggage comp capacity | ltr | 520 |  |  | 520 |  |
| Air resistance | Cd x A | 0.29 x 2.35 |  |  | 0.30 x 2.35 |  |
| Power Unit |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-six / 4 |  |  | V90 / 8 / 4 |  |
| Engine technology |  | BMW TwinPower Turbo, direct fuel injection (High Precision Injection), fully-variable valve control (VALVETRONIC) |  |  |  | BMW TwinPower Turbo, direct fuel injection (High Precision Injection) |
| Capacity, effective | cc | 2979 |  |  | 4395 |  |
| Stroke / bore | mm | 89.6 / 84.0 |  |  | 88,3 / 89,0 |  |
| Compression ratio | :1 | 10.2 |  |  | 10.0 |  |
| Fuel grade |  | Min RON 91 |  |  | min RON 91 |  |
| Max output | kW/hp | 225 / 306 |  |  | 300 / 407 |  |
| at | rpm | 5800 |  |  | 5500-6400 |  |
| Max torque | Nm/lb-ft | 400/295 |  |  | 600/442 |  |
| at | rpm | 1200-5000 |  |  | 1750-4500 |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– | 80 (90) / luggage comp |  |  | 90 / luggage comp |  |
| Alternator | A/W | 170 / 2380 |  |  | 210 / 2940 |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle, aluminium, with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| Diameter | mm | 348 x 36 / inner-vented |  |  | 374 x 36 / inner-vented |  |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |  |
| Diameter | mm | 345 x 24 / inner-vented |  |  | 345 x 24 / inner-vented |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Adaptive Drive |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 17.1 |  |  | 17.1 |  |
| Tyres, front/rear |  | 225/55 R17 97W |  |  | 245/45 R18 96Y RSC |  |
| Rims, front/rear |  | 8J x 17 light-alloy |  |  | 8J x 18 light-alloy |  |

Specifications.
The new BMW 5 Series Sedan. 
535i, 550i.

|  |  |  | BMW 535i |  |  |  | BMW 550i |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Optional: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Optional: access to the internet, BMW Online including Parkinfo, National Info, Google Directory Service, News, Weather Realtime and BMW Routes |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  |  | Six-speed manual (optional: eight-speed automatic with Steptronic) |  |  |  |  | Eight-speed automatic with Steptronic |
| Gear ratios I |  | :1 |  | 4.110 (4.714) |  |  |  | 4.714 |  |
| II |  | :1 |  | 2.315 (3.143) |  |  |  | 3.143 |  |
| III |  | :1 |  | 1.542 (2.106) |  |  |  | 2.106 |  |
| IV |  | :1 |  | 1.179 (1.667) |  |  |  | 1.667 |  |
| V |  | :1 |  | 1.000 (1.285) |  |  |  | 1.285 |  |
| VI |  | :1 |  | 0.846 (1.000) |  |  |  | 1.000 |  |
| VII |  | :1 |  | – (0.839) |  |  |  | 0.839 |  |
| VIII |  | :1 |  | – (0.667) |  |  |  | 0.667 |  |
| R |  | :1 |  | 3.727 (3.295) |  |  |  | 3.317 |  |
| Final drive |  | :1 |  | 3.231 (3.077) |  |  |  | 2.813 |  |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  | 7.5 (7.6) |  |  | 6.1 |  |  |
| Output per litre | kW/hp |  |  | 75.5/102.7 |  |  | 68.3/92.9 |  |  |
| Acceleration 0–100 km/h | sec |  |  | 6.0 (6.1) |  |  | 5.0 |  |  |
| Standing-start km | sec |  |  | 25.2 (25.6) |  |  | 23.2 |  |  |
| In 4th gear 80–120 km/h | sec |  |  | 5.9 (–) |  |  | – |  |  |
| Top speed | km/h |  |  | 250 |  |  | 250 |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration with recuperation display, gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control (BMW 535i), Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance (BMW 535i) |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  | 11.8 (11.9) |  |  | 15.4 |  |  |
| Extra-urban | ltr/100 km |  |  | 6.6 (6.4) |  |  | 7.5 |  |  |
| Combined | ltr/100 km |  |  | 8.5 (8.4) |  |  | 10.4 |  |  |
| CO2 | g/km |  |  | 199 (195) |  |  | 243 |  |  |
| Emission rating |  |  |  | EU5 |  |  | EU5 |  |  |

Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)

Figures in brackets apply to automatic transmission models

1)  Oil change