[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI One. (Stand: January 2010)

	Body
Anzahl Türen / s
	
	MINI One (72 KW) 
	MINI One automatik
	MINI One (55 KW) 
	

	No of doors/seats
	 
	3/4
	3/4
	3 / 4


	

	Length/width/heigth (unladen)
	mm
	3699 / 1683 / 1407
	3699 / 1683 / 1407 
	3699 / 1683 / 1407
	

	Wheelbase
	mm
	2 467
	2 467
	2467
	

	Track front/rear
	mm
	1459 / 1467
	1459 / 1467
	1459 / 1467
	

	Turning circle
	m
	10,7
	10,7
	10,7
	

	Fuel tank capacity
	ca. l
	40
	40
	40
	

	Cooling system including heater
	l
	5,2
	5,2
	5,2
	

	Engine oil
	l
	4,2
	4,2
	4,2
	

	Transmission fluid incl. Final drive
	l
	Permanent filling
	Permanent filling
	Permanent filling
	

	Weight, unladen, according to DIN/EU1
	kg
	1060 / 1135
	1100 / 1175
	1060 / 1135
	

	Max load according to DIN
	kg
	450
	450
	450
	

	Max permissible weight to DIN
	kg
	1510
	1550
	1510
	

	Max permissible axle load front/rear
	kg
	815 / 730
	855 / 730
	810 / 730
	

	Max permissible trailer load
	
	
	
	
	

	Braked (12%)/unbraked
	kg
	
	
	
	

	Max roof load/max towbar download
	kg
	
	
	
	

	Luggage compartment according to DIN
	l
	160-680
	160-680
	160-680
	

	Air restistance
	cX x A
	0,33 x 1,97
	0,33 x 1,97
	0,33 x 1,97
	

	Engine
	
	
	
	
	

	Configuration/No of cylinders/salves

	
	Inline /4/ 4
	Inline/ 4/ 4
	Inline/ 4/ 4
	

	Enginge management
	
	MEV 17.2
	MEV 17.2
	MEV 17.2
	

	Capacity
	cm³
	1598
	1598
	1598
	

	Bore / stroke
	mm
	77 / 85,8
	77 / 85,8
	77 / 85,8
	

	Compression ratio
	:1 
	11:1
	11:1
	11:1
	

	Fuel
	ROZ
	91-98
	91-98
	91-98
	

	Max output
	kW/PS
	72/98
	72/98
	55/75
	

	At
	min-1
	6000
	6000
	5000
	

	Max torque
	Nm
	153
	153
	140
	

	At
	min-1
	3000
	3000
	2250
	

	Elektrical system
	
	
	
	
	

	Battery/installation
	Ah/–
	55 / Motorraum
	55 / Motorraum
	55 / Motorraum
	

	Alternator
	A/W
	120/1680
	120/1680
	120/1680
	

	Chassis and suspension
	
	
	
	
	

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	
	
	

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	
	
	

	Brakes, front
	
	Vented disc
	Vented disc
	Vented disc
	

	Diameter
	mm
	280
	280
	280
	

	Brake, rear
	
	Disc brake
	Disc brake
	Disc brake
	

	Diameter
	mm
	259
	259
	259
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	
	

	Steering
	Electric power steering (EPS)
	
	

	Overall transmission ratio
	:1 
	14,1
	14,1
	14,1
	

	Type of transmission
	
	6-gear manual
	6-gear automatic 
	6-gear manual
	

	Gear ratios
I


	:1 
	3,214
	4,148
	3,214
	

	
II
	:1 
	1,792
	2,370
	1,792
	

	
III
	:1 
	1,194
	1,556
	1,194
	

	
IV
	:1 
	0,914
	1,155
	0,914
	

	
V
	:1 
	0,784
	0,859
	0,784
	

	
VI
	:1
	0,683
	0,686
	0,683
	

	
R
	:1 
	3,143
	3,394
	3,143
	

	Final drive ratio
	:1
	4,353
	4.103
	4,353
	

	Tyres
	 
	175 / 65 R15 84H
	175 / 65 R15 84H
	175 / 65 R15 84H
	

	Wheels
	 
	5,5Jx15 Stahl
	5,5Jx15 Stahl
	5,5Jx15 Stahl
	

	Performance
	
	
	
	
	

	Power to weight ratio, DIN
	kg/kW
	15,1
	15,7
	19,3
	

	Output per litre
	kW/l
	50,1
	50,1
	39,4
	

	Acceleration
0–100 km/h
	s
	10,5
	12,3
	13,2
	

	
0–1000 m
	s
	31,7
	34,0
	-
	

	in 4./5. gear
80–120 km/h
	s
	12,1 / 15,3
	-
	13,5 / 16,7
	

	Top speed
	km/h
	186
	181
	175
	

	Consumption in the EU cycle
	
	
	
	
	

	Urban
	l/100 km
	7,2
	8,7
	7,2
	

	Extra- urban
	l/100 km
	4,4
	5,1
	4,4
	

	Combined
	l/100 km
	5,4
	6,4

	5,4
	

	CO2 
	g/km
	127
	150
	127
	

	Miscellaneous
	
	 
	
	
	

	Emission category
	–
	EU5
	EU5
	EU5
	

	Insurance ratings Germany
	HPF/VK/TK
	14 / 16 / 19
	14 / 16 / 19
	14 / 16 / 19
	

	Ground clearance
	mm
	139
	139
	139
	


1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]