[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications 
MINI John Cooper Works. (Stand: January. 2010)
	Body

L
	
	MINI John Cooper Works 
	
	

	No of doors/seats

L
	 
	3/4
	
	

	Length/width/height (unladen) 

)
	mm
	3714/ 1683/ 1407
	
	

	Wheelbase
	mm
	2467
	
	

	Track front/rear

wgsdfg


	mm
	1453/ 1461
	
	

	Turning circle
	m
	10,7
	
	

	Fuel tank capacity
	ca. l
	50
	
	

	Cooling system including heater

l
	l
	5,2
	
	

	Engine oil

Getriebeöl


	l
	4,2
	
	

	Transmission fluid incl final drive
	l
	permanent filling
	
	

	Weight, unladen, according to DIN/EU1 
	kg
	1130/ 1205
	
	

	Max load according to DIN
	kg
	450
	
	

	Max permissible weight to DIN
	kg
	1580
	
	

	Max permissible axle load front/rear
	kg
	860/ 750
	
	

	Max permissible trailer load2
	
	
	
	

	braked (12%)/unbraked
	kg
	nicht möglich
	
	

	Max roof load / max towbar download
	kg
	75 / -
	
	

	Luggage compartment according to DIN
	l
	160-680


	
	

	Air resistance
	cx/ A/ cX x A
	0,36/ 1,97/ 0,71
	
	

	Engine
	
	
	
	

	Configuration/No of cylinders/valves Zylinder/Ventile


	
	Inline/ 4 / 4
	
	

	Engine management
	
	MED 17.2
	
	

	Capacity
	cm3
	1598
	
	

	Bore/stroke
	mm
	77 / 85,8
	
	

	Compression ratio


	:1 
	10,0:1
	
	

	Fuel
	ROZ
	91-98
	
	

	Max output
	kW/PS
	155/ 211
	
	

	at
	min-1 
	6.000
	
	

	Max torque
	Nm
	260 (280)
	
	

	at
	min-1
	1.850-5.600 (2.000-5.300)
	
	

	Electrical system
	
	
	
	
	
	

	Battery/installation
	Ah/–
	46 / Motorraum
	
	

	Alternator
	A/W
	120 / 1680
	
	

	Chassis and suspension
	
	
	
	

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	

	Brake, front
	
	Disc, vented
	

	Diameter
	mm
	316 x 22
	

	Brake, rear
	
	Disc, brake
	

	Diameter
	mm
	280 x 10
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	

	Steering
	Electric power steering (EPS)
	

	Overall transmission ratio
	:1 
	14,1
	
	

	Type of transmission
	
	6- gear-manual
	
	

	Gear ratios
I


	:1 
	3,308
	
	

	
II
	:1 
	2,130
	
	

	
III
	:1 
	1,483
	
	

	
IV
	:1 
	1,139
	
	

	
V
	:1 
	0,949
	
	

	
VI
	:1 
	0,816
	
	

	
R
	:1 
	3,231

	
	

	Final drive ratio
	:1
	3,647
	
	

	Tyres
	 
	205/ 45 R17 84W RSC
	
	

	Wheels
	 
	7J x 17LM
	
	

	Performance
	
	
	
	

	Power-to-weight ratio, DIN
	kg/kW
	7,3
	
	

	Output per litre
	kW/l
	97,0
	
	

	Acceleration
0–62 mph
	s
	6,5
	
	

	
0–1000 m
	s
	26,3
	
	

	in 4th/5th gear
50–75 mph
	s
	5,2/ 6,2
	
	

	Top speed
	km/h
	238
	
	

	Consumption in the EU cycle
	
	
	
	

	Urban
	l/100 km
	9,4
	
	

	Extra-urban
	l/100 km
	5,8
	
	

	Combined
	l/100 km
	7,1
	
	

	CO2 
	g/km
	165
	
	

	Miscellaneous
	
	 
	
	

	Emission category
	–
	EU5
	
	

	Insurance ratings Germany
	HPF/VK/TK
	17/ 24/ 25
	
	

	Ground clearance
	mm
	130
	135
	








1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]