[image: image1.png]



MINI
Corporate Communications

Specifications MINI Cooper Cabrio / MINI Cooper S Cabrio. 
(Stand: January 2010)
	Body

L
	
	MINI Cooper Cabrio
	MINI Cooper S Cabrio
	

	No of doors/seats

L
	 
	2 / 4
	2 / 4
	

	Length/width/height (unladen) 

)
	mm
	3699 / 1683 / 1414
	3714 / 1683 / 1414
	

	Wheelbase
	mm
	2467
	2467
	

	Track front/rear

wgsdfg


	mm
	1459 / 1467
	1453 / 1461
	

	Turning circle
	m
	10,7
	10,7
	

	Fuel tank capacity
	           approx. l
	40
	50
	

	Cooling system including heater

l
	l
	5,2
	5,2
	

	Engine oil

Getriebeöl


	l
	4,2
	4,2
	

	Transmission fluid incl final drive
	l
	permanent filling
	permanent filling
	

	Weight, unladen, according to DIN/EU1 
	kg
	1165 / 1240 (1205 / 1280)
	1230 / 1305 (1255 / 1330)
	

	Max load according to DIN
	kg
	430
	430
	

	Max permissible weight to DIN
	kg
	1595 (1635)
	1660 (1685)
	

	Max permissible axle load front/rear
	kg
	845 / 775 (880 / 780)
	885 / 795 (905 / 795)
	

	Max permissible trailer load2
	kg
	-
	-
	

	braked (12%)/unbraked
	kg
	-
	-
	

	Max roof load / max towbar download
	kg
	-
	-
	

	Luggage compartment according to DIN
	l
	125 / 170 / 660
	125 / 170 / 660
	

	Air resistance
	- / m2 / m2
	0,35 / 1,97 / 0,69
	0,37/ 1,99 / 0,74
	

	Engine
	
	
	

	Configuration/No of cylinders/valves Zylinder/Ventile


	
	Inline / 4 / 4
	Inline / 4 / 4
	

	Engine management
	
	MEV 17,2
	MED 17,2
	

	Capacity
	cm³
	1598
	1598
	

	Bore/stroke
	mm
	77,0 / 85,8
	77/ 85,8
	

	Compression ratio


	:1 
	11,0
	10,5
	

	Fuel
	ROZ
	91-98
	91-98
	

	Max output
	kW / PS
	88 / 120
	128 / 175
	

	at
	min–1
	6 000
	5 500
	

	Max torque
	Nm
	160
	240 (260)
	

	at
	min–1
	4 250
	1 600 – 5 000
	

	Electrical system
	
	
	

	Battery/installation
	Ah / –
	46 / Motorraum
	46 / Motorraum
	

	Alternator
	A / W
	120 / 1680
	120 / 1680
	

	Chassis and suspension
	
	
	

	Front suspension
	
	Singe-joint Mc Pherson spring strut axle with anti-drive
	

	Rear suspension
	
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	

	Brake, front
	
	Disc, vented
	

	Diameter
	mm
	280 x 22 
	294 x 22
	

	Brake, rear
	
	Disc  brake 
	

	Diameter
	mm
	259 x 10
	259 x 10
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	

	Steering
	Electric power steering (EPS))
	

	Overall transmission ratio
	:1 
	14,1
	14,1
	

	Type of transmission
	
	6-Gear-manual (6-Stufen-automatic) 
	
	

	Gear ratios
I


	:1 
	3,214 (4,148)
	3,308 (4,044)
	

	
II
	:1 
	1,792 (2,370)
	2,130 (2,371)
	

	
III
	:1 
	1,194 (1,556)
	1,483 (1,556)
	

	
IV
	:1 
	0,914 (1,155)
	1,139 (1,159)
	

	
V
	:1 
	0,784 (0,859)
	0,949 (0,852)
	

	
VI
	:1 
	0,683 (0,686)
	0,816 (0,672)
	

	
R
	:1
	3,143 (3,394)
	3,231 (3,193)
	

	Final drive ratio
	:1
	4,353 (4,103)
	3,706 (3,683)
	

	Tyres
	 
	175/65 R15 84H
	195/55 R16 87V RSC
	

	Wheels
	 
	5,5J x 15 LM
	6,5J x 16 LM
	

	Performance
	
	
	

	Power-to-weight ratio, DIN
	kg / kW
	12,9 (13,4)
	9,1 (9,3)
	

	Output per litre
	kW / l
	56,3
	84,5
	

	Acceleration
0–62 mph
	s
	9,8 (11,1)
	7,3 (7,6)
	

	
0–1000 m
	s
	31,0 (32,1)
	27,7 (28,0)
	

	in 4th/5th gear
50–75 mph
	s
	10,5 / 13,3
	6,2 / 7,9
	

	Top speed
	Km / h
	198 (191)
	225 (220)
	

	Consumption in the EU cycle
	
	
	

	Urban
	l / 100 km
	7,2 (8,9)
	7,5 (9,1)
	

	Extra-urban
	l / 100 km
	4,9 (5,3)
	5,1 (5,1)
	

	Combined
	l / 100 km
	5,7 (6,6)
	6,0 (6,6)
	

	CO2 
	g / km
	133 (154)
	139 (153)
	

	Miscellaneous
	
	 
	

	Emission category
	–
	EU5
	EU5
	

	Insurance ratings Germany
	HPF/VK/TK
	-
	-
	

	Ground clearance
	mm
	139
	130
	


1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png]