[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI Cooper Clubman. (Stand: January 2010)
	Body

L
	
	MINI Cooper Clubman
	MINI Cooper Clubman automatik
	

	No of doors/seats

L
	 
	5 / 5 (optional 4 Sitzplätze)
	5 / 5 (optional 4 Sitzplätze)
	

	Length/width/height (unladen) 

)
	mm
	3937 / 1683 / 1426
	3937 / 1683 / 1426 
	

	Wheelbase
	mm
	2 547
	2 547
	

	Track front/rear

wgsdfg


	mm
	1459 / 1467
	1459 / 1467
	

	Turning circle
	m
	11,0
	11,0
	

	Fuel tank capacity
	approx. l
	40
	40
	

	Cooling system including heater

l
	l
	5,2
	5,2
	

	Engine oil

Getriebeöl


	l
	4,2
	4,2
	

	Transmission fluid incl final drive
	l
	permanent filling
	permanent filling
	

	Weight, unladen, according to DIN/EU1 
	kg
	1145 / 1220
	1175 / 1250
	

	Max load according to DIN
	kg
	500
	500
	

	Max permissible weight to DIN
	kg
	1645
	1675
	

	Max permissible axle load front/rear
	kg
	840 / 840
	870 / 850
	

	Max permissible trailer load2
	
	
	
	

	braked (12%)/unbraked
	kg
	750 / 500
	750 / 500
	

	Max roof load / max towbar download
	kg
	75 / 50
	75 / 50
	

	Luggage compartment according to DIN
	l
	260-930
	260-930
	

	Air resistance
	cX x A
	0,34 x 2,01
	0,34 x 2,01
	

	Engine
	
	
	
	

	Configuration/No of cylinders/valves Zylinder/Ventile


	
	Inline / 4/ 4
	Inline / 4/ 4
	

	Engine management
	
	MEV 17.2
	MEV 17.2
	

	Capacity
	cm³
	1598
	1598
	

	Bore/stroke
	mm
	77 / 85,8
	77 / 85,8
	

	Compression ratio


	:1 
	11,0
	11,0
	

	Fuel
	ROZ
	91-98
	91-98
	

	Max output
	kW/PS
	90 / 122
	90 / 122
	

	at
	min-1
	6 000
	6 000
	

	Max torque
	Nm
	160
	160
	

	at
	min-1
	4 250
	4 250
	

	Electrical system
	
	
	
	

	Battery/installation
	Ah/–
	46 / vorn
	46 / vorn
	

	Alternator
	A/W
	120 / 1680
	120 / 1680
	

	Chassis and suspension
	
	
	
	

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	

	Brake, front
	
	Vented disc
	Vented disc
	

	Diameter
	mm
	280 
	280
	

	Brake, rear
	
	Disc brake
	Disc brake
	

	Diameter
	mm
	259
	259
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	

	Steering
	Electric power steering (EPS))
	

	Overall transmission ratio
	:1 
	14,1
	14,1
	

	Type of transmission
	
	6-gear-manual
	6-gear-automatic
	

	Gear ratios
I


	:1 
	3,214
	4,148
	

	
II
	:1 
	1,792
	2,370
	

	
III
	:1 
	1,194
	1,556
	

	
IV
	:1 
	0,914
	1,155
	

	
V
	:1 
	0,784
	0,859
	

	
VI
	:1 
	0,683
	0,686
	

	
R
	:1 
	3,143
	3,394
	

	Final drive ratio
	:1
	4,353
	4,103
	

	Tyres
	 
	175 / 65 R15 84H
	175 / 65 R15 84H
	

	Wheels
	 
	5,5Jx15 LM
	5,5Jx15 LM
	

	Performance
	
	
	
	

	Power-to-weight ratio, DIN
	kg/kW
	12,7
	13,5
	

	Output per litre
	kW/l
	56,3
	56,3
	

	Acceleration
0–62 mph
	s
	9,8
	10,9
	

	
0–1000 m
	s
	30,9
	32,0
	

	in 4th/5th gear
50–75 mph
	s
	10,2 / 12,7
	-
	

	Top speed
	km/h
	201
	195
	

	Consumption in the EU cycle
	
	
	
	

	Urban
	l/100 km
	7,0
	8,8
	

	Extra-urban
	l/100 km
	4,7
	5,2
	

	Combined
	l/100 km
	5,5
	6,5
	

	CO2 
	g/km
	129
	152
	

	Miscellaneous
	
	 
	
	

	Emission category
	–
	EU5
	EU5
	

	Insurance ratings Germany
	HPF/VK/TK
	13 / 17 / 19
	13 / 17 / 19
	

	Ground clearance
	mm
	138
	138
	

	
	
	
	
	


1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]