[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI Cooper S Clubman. (Stand: January 2010)
	Body
	
	MINI Cooper S Clubman
	MINI Cooper S Clubman automatic
	

	No of doors/seats

L
	 
	5/5 (optional 4 seats)
	5/5 (optional 4 seats)
	

	Length/width/height (unladen) 

)
	mm
	3958 / 1683 / 1432 
	3958 / 1683 / 1432
	

	Wheelbase
	mm
	2 547
	2 547
	

	Track front/rear

wgsdfg


	mm
	1453 / 1461
	1453 / 1461
	

	Turning circle
	m
	11,0
	11,0
	

	Fuel tank capacity
	                   approx. l
	50
	50
	

	Cooling system including heater

l
	l
	5,2
	5,2
	

	Engine oil

Getriebeöl


	l
	4,2
	4,2
	

	Transmission fluid incl final drive
	l
	permanent filling
	permanent filling
	

	Weight, unladen, according to DIN/EU1 
	kg
	1205 / 1280
	1230 / 1305
	

	Max load according to DIN
	kg
	485
	485
	

	Max permissible weight to DIN
	kg
	1690
	1715
	

	Max permissible axle load front/rear
	kg
	870 / 850
	900 / 850
	

	Max permissible trailer load2
	kg
	
	
	

	braked (12%)/unbraked
	kg
	
	
	

	Max roof load / max towbar download
	kg
	75 / -
	75 / -
	

	Luggage compartment according to DIN
	 
	260-930


	260-930


	

	Air resistance
	mm
	0,35 x 2,02
	0,35 x 2,02
	

	Engine
	
	
	
	

	Configuration/No of cylinders/valves Zylinder/Ventile


	
	Inline / 4 / 4
	Inline / 4 / 4
	

	Engine management
	
	MEVD 17.2
	MEVD 17.2
	

	Capacity
	cm3
	1598
	1598
	

	Bore/stroke
	mm
	77 / 85,8
	77 / 85,8
	

	Compression ratio


	:1 
	10,5:1
	10,0:1
	

	Fuel
	ROZ
	91-98
	91-98
	

	Max output
	kW/PS
	135 / 184
 / 170
	135 / 184
	

	at
	min-1 
	5500
	5500
	

	Max torque
	Nm
	240 (260)
	240 (260)
	

	at
	min-1
	1600-5000
	1600-5000
	

	Electrical system
	
	
	
	
	
	

	Battery/installation
	Ah/–
	46 / vorn
	46 / vorn
	

	Alternator
	A/W
	120 / 1680
	120 / 1680
	

	Chassis and suspension
	
	
	
	

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	

	Brake, front
	
	Disc, vented
	Disc, vented
	

	Diameter
	mm
	294
	294
	

	Brake, rear
	
	Disc, brake
	Disc, brake
	

	Diameter
	mm
	259
	259
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	

	Steering
	Electric power steering (EPS))
	

	Overall transmission ratio
	:1 
	14,1
	14,1
	

	Type of transmission
	
	6-gear-manuel
	6-gear-automatic
	

	Gear ratios
I


	:1 
	3,308
	4,044
	

	
II
	:1 
	2,130
	2,371
	

	
III
	:1 
	1,483
	1,556
	

	
IV
	:1 
	1,139
	1,159
	

	
V
	:1 
	0,949
	0,852
	

	
VI
	:1 
	0,816
	0,672
	

	
R
	:1 
	3,231
	3,193

	

	Final drive ratio
	:1
	3,706
	3,683 
	

	Tyres
	 
	195 / 55 R16 87V RSC
	195 / 55 R16 87V RSC
	

	Wheels
	 
	6,5Jx16 LM
	6,5Jx16 LM
	

	Performance
	
	
	
	

	Power-to-weight ratio, DIN
	kg/kW
	9,4
	9,6
	

	Output per litre
	kW/l
	80,1
	80,1
	

	Acceleration
0–62 mph
	s
	7,5
	7,7
	

	
0–1000 m
	s
	28,0
	27,8
	

	in 4th/5th gear
50–75 mph
	s
	5,9 / 7,6  
	- / -
	

	Top speed
	km/h
	227 
	222
	

	Consumption in the EU cycle
	
	
	
	

	Urban
	l/100 km
	7,4
	8,9
	

	Extra-urban
	l/100 km
	5,0
	5,0
	

	Combined
	l/100 km
	5,9
	6,4
	

	CO2 
	g/km
	137
	150
	

	Miscellaneous
	
	 
	
	

	Emission category
	–
	EU5
	EU5
	

	Insurance ratings Germany
	HPF/VK/TK
	14 / 20 / 23
	14 / 20 / 23
	

	Ground clearance
	mm
	135
	135
	








1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]