[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI One Clubman. (Stand: January 2010 )
	Body
Anzahl Türen / s
	· 
	· MINI One Clubman
	· MINI One Clubman automatic

	No of doors/seats
	·  
	5 / 5 (optional 4 seats) 
	· 5 / 5 (optional 4 seats)

	Length/width/heigth (unladen)
	· mm
	· 3937 / 1683 / 1426
	· 3937 / 1683 / 1426 

	Wheelbase
	· mm
	· 2 547
	· 2 547

	Track front/rear
	· mm
	· 1459 / 1467
	· 1459 / 1467

	Turning circle
	· m
	· 11,0
	· 11,0

	Fuel tank capacity
	· ca. l
	· 40
	· 40

	Cooling system including heater
	· l
	· 5,2
	· 5,2

	Engine oil
	· l
	· 4,2
	· 4,2

	Transmission fluid incl. Final drive
	· l
	· permanent filling
	· permanent filling

	Weight, unladen, according to DIN/EU1
	· kg
	· 1140 / 1215
	· 1170 / 1245

	Max load according to DIN
	· kg
	· 500
	· 500

	Max permissible weight to DIN
	· kg
	· 1640
	· 1670

	Max permissible axle load front/rear
	· kg
	· 835 / 840
	· 870 / 845

	Max permissible trailer load
	· 
	· 
	· 

	Braked (12%)/unbraked
	· kg
	· 650 / 500
	· 650 / 500

	Max roof load/max towbar download
	· kg
	· 75 / 50
	· 75 / 50

	Luggage compartment according to DIN
	· l
	· 260-930
	· 260-930

	Air restistance
	· cX x A
	· 0,33 x 2,01
	· 0,33 x 2,01

	Engine
	· 
	· 
	· 

	Configuration/No of cylinders/salves

	· 
	· Inline/ 4/ 4
	· Inline/ 4/ 4

	Enginge management
	· 
	· MEV 17.2
	· MEV 17.2

	Capacity
	· cm³
	· 1598
	· 1598

	Bore / stroke
	· mm
	· 77 / 85,8
	· 77 / 85,8

	Compression ratio
	· :1 
	· 11,0
	· 11,0

	fuel
	ROZ
	91-98
	91-98

	Max output
	· kW/PS
	· 72 / 98
	· 72 / 98

	At
	· min-1
	· 6 000
	· 6 000

	Max torque
	· Nm
	· 153
	· 153

	At
	· min-1
	· 3 000
	· 3 000

	Elektrical system
	· 
	· 
	· 

	Battery/installation
	· Ah/–
	· 46 / vorn
	· 46 / vorn

	Alternator
	· A/W
	· 120 / 1680
	· 120 / 1680

	Chassis and suspension
	· 
	· 
	· 

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	· 

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	· 

	Brakes, front
	· 
	· Vented Disc
	· Vented Disc

	Diameter
	· mm
	· 280 
	· 280

	Brake, rear
	· 
	· Disc brake
	· Disc brake

	Diameter
	· mm
	· 259
	· 259

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	

	Steering
	· Electric power steering (EPS)
	· 

	Overall transmission ratio
	· :1 
	· 14,1
	· 14,1

	Type of transmission
	· 
	· 6-gear-manuall 
	· 6-Gear automatic

	Gear ratios
I


	· :1 
	· 3,214
	· 4,148

	
II
	· :1 
	· 1,792
	· 2,370

	
III
	· :1 
	· 1,194
	· 1,556

	
IV
	· :1 
	· 0,914
	· 1,155

	
V
	· :1 
	· 0,784
	· 0,859

	
VI
	· :1 
	· 0,683
	· 0,686

	
R
	· :1 
	· 3,143
	· 3,394

	Final drive ratio
	· :1
	· 4,353
	· 4,103

	Tyres
	·  
	· 175 / 65 R15 84H
	· 175 / 65 R15 84H

	Wheels
	·  
	· 5,5Jx15 ST
	· 5,5Jx15 ST

	Performance
	· 
	· 
	· 

	Power to weight ratio, DIN
	· kg/kW
	· 15,8
	· 16,3

	Output per litre
	· kW/l
	· 45,1
	· 45,1

	Acceleration
0–100 km/h
	· s
	· 11,1
	· 12,8

	
0–1000 m
	· s
	· 32,3
	· 34,5

	in 4./5. gear
80–120 km/h
	· s
	· 12,9 / 16,4
	· -

	Top speed
	· km/h
	· 185
	· 179

	Consumption in the EU cycle
	· 
	· 
	· 

	Urban
	· l/100 km
	· 7,3
	· 8,8

	Extra- urban
	· l/100 km
	· 4,5
	· 5,2

	Combined
	· l/100 km
	· 5,5
	· 6,5

	CO2 
	· g/km
	· 129
	· 152

	Miscellaneous
	· 
	·  
	· 

	Emission category
	· –
	· EU5
	· EU5

	Insurance ratings Germany
	· HPF/VK/TK
	· 13 / 17 / 19
	· 13 / 17 / 19

	Ground clearance
	· mm
	· 138
	· 138


1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]