[image: image2.png]



MINI
Corporate Communications
[image: image1.png]



MINI
Presse- und Öffentlichkeitsarbeit

Specifications MINI One MINIMALISM Line (72 kW and 55 kW). (Stand: January 2010)

	Body
Anzahl Türen / s
	
	MINI One MINIMALISM Line 
	
	MINI One MINIMALISM Line (55 KW) 
	

	No of doors/seats
	 
	3/4
	
	3 / 4


	

	Length/width/heigth (unladen)
	mm
	3699 / 1683 / 1407
	
	3699 / 1683 / 1407
	

	Wheelbase
	mm
	2 467
	
	2467
	

	Track front/rear
	mm
	1459 / 1467
	
	1459 / 1467
	

	Turning circle
	m
	10,7
	
	10,7
	

	Fuel tank capacity
	ca. l
	40
	
	40
	

	Cooling system including heater
	l
	5,2
	
	5,2
	

	Engine oil
	l
	4,2
	
	4,2
	

	Transmission fluid incl. Final drive
	l
	Permanent filling
	
	Permanent filling
	

	Weight, unladen, according to DIN/EU1
	kg
	1060 / 1135
	
	1060 / 1135
	

	Max load according to DIN
	kg
	450
	
	450
	

	Max permissible weight to DIN
	kg
	1510
	
	1510
	

	Max permissible axle load front/rear
	kg
	815 / 730
	
	810 / 730
	

	Max permissible trailer load
	
	
	
	
	

	Braked (12%)/unbraked
	kg
	
	
	
	

	Max roof load/max towbar download
	kg
	75 / - 
	
	
	

	Luggage compartment according to DIN
	l
	160-680
	
	160-680
	

	Air restistance
	cX x A
	0,33 x 1,97
	
	0,33 x 1,97
	

	Engine
	
	
	
	
	

	Configuration/No of cylinders/salves

	
	Inline /4/ 4
	
	Inline/ 4/ 4
	

	Enginge management
	
	MEV 17.2
	
	MEV 17.2
	

	Capacity
	cm³
	1598
	
	1598
	

	Bore / stroke
	mm
	77 / 85,8
	
	77 / 85,8
	

	Compression ratio
	:1 
	11:1
	
	11:1
	

	Fuel
	ROZ
	91-98
	
	91-98
	

	Max output
	kW/PS
	72/98
	
	55/75
	

	At
	min-1
	4750
	
	5000
	

	Max torque
	Nm
	153
	
	140
	

	At
	min-1
	3000
	
	2250
	

	Elektrical system
	
	
	
	
	

	Battery/installation
	Ah/–
	55 / Motorraum
	
	55 / Motorraum
	

	Alternator
	A/W
	120/1680
	
	120/1680
	

	Chassis and suspension
	
	
	
	
	

	Front suspension
	Singe-joint Mc Pherson spring strut axle with anti-drive
	
	

	Rear suspension
	Longitudinal struts with centrally-pivoted control arms, Z-axle
	
	

	Brakes, front
	
	Vented disc
	
	Vented disc
	

	Diameter
	mm
	280
	
	280
	

	Brake, rear
	
	Disc brake
	
	Disc brake
	

	Diameter
	mm
	259
	
	259
	

	Driving stability systems
	Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant.

Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, 
Limited-slip differential.
	
	

	Steering
	Electric power steering (EPS)
	
	

	Overall transmission ratio
	:1 
	14,1
	
	14,1
	

	Type of transmission
	
	6-gear manual
	
	               6-gear manual
	

	Gear ratios
I


	:1 
	3,214
	
	3,214
	

	
II
	:1 
	1,792
	
	1,792
	

	
III
	:1 
	1,194
	
	1,194
	

	
IV
	:1 
	0,914
	
	0,914
	

	
V
	:1 
	0,784
	
	0,784
	

	
VI
	:1
	0,683
	
	0,683
	

	
R
	:1 
	3,143
	
	3,143
	

	Final drive ratio
	:1
	3,706
	
	4,353
	

	Tyres
	 
	175 / 65 R15 84H
	
	175 / 65 R15 84H
	

	Wheels
	 
	5,5Jx15 Stahl
	
	5,5Jx15 Stahl
	

	Performance
	
	
	
	
	

	Power to weight ratio, DIN
	kg/kW
	11,8
	
	19,3
	

	Output per litre
	kW/l
	56,3
	
	39,4
	

	Acceleration
0–100 km/h
	s
	10,5
	
	13,2
	

	
0–1000 m
	s
	31,7
	
	-
	

	in 4./5. gear
80–120 km/h
	s
	12,1 / 15,3
	
	13,5 / 16,7
	

	Top speed
	km/h
	186
	
	175
	

	Consumption in the EU cycle
	
	
	
	
	

	Urban
	l/100 km
	6,5
	
	6,5
	

	Extra- urban
	l/100 km
	4,3
	
	4,3
	

	Combined
	l/100 km
	5,1
	
	5,1
	

	CO2 
	g/km
	119
	
	119
	

	Miscellaneous
	
	 
	
	
	

	Emission category
	–
	EU5
	
	EU5
	

	Insurance ratings Germany
	HPF/VK/TK
	14 / 16 / 19
	
	14 / 16 / 19
	

	Ground clearance
	mm
	139
	
	139
	


1 Gewicht des betriebsfertigen Fahrzeugs (DIN) plus 75 kg für Fahrer und Gepäck.

2 Erhöhungen sind unter bestimmten Voraussetzungen möglich.


[image: image1.png][image: image2.png]