[image: image1.png]


BMW
Media
Information

1/2010

Valid from 
3/2010
BMW
Media
Information

1/2010

Valid from 3/2010

Specifications BMW 3 Series Saloon.
316d, 318d, 320d

	
	
	BMW 316d 
Saloon
	BMW 318d
Saloon
	BMW 320d 
Saloon

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	4 / 5
	4 / 5
	4 / 5

	Length/width/height (unladen)
	mm
	4531 / 1817 / 1421
	4531 / 1817 / 1421
	4531 / 1817 / 1421

	Wheelbase
	mm
	2760
	2760
	2760

	Track, front/rear
	mm
	1506 / 1535
	1506 / 1535
	1506 / 1535

	Ground clearance
	
	144
	144
	144

	Turning circle
	m
	11.0
	11.0
	11.0

	Tank capacity
	appr ltr
	61
	61
	61

	Cooling system incl heater
	ltr
	7.5
	7.5 (7.9)
	7.5 (7.9)

	Engine oil1)
	ltr
	5.2
	5.2
	5.2

	Weight, unladen, to DIN/EU
	kg
	1400 / 1475


	1415 / 1490

(1445 / 1520)
	1420 / 1495

(1430 / 1505)

	Max load to DIN
	kg
	520
	520
	520

	Max permissible
	kg
	1920
	1935 (1950)
	1940 (1950)

	Max axle load, front / rear
	kg
	900 / 1080
	925 / 1080
	925 / 1080

	Max trailer load (12%),

braked / unbraked
	kg
	1600 / 695
	1600 / 745
	1600 / 745

	Max roofload / max download
	kg
	75 / 75
	75 / 75
	75 / 75 

	Luggage comp
	ltr
	460
	460
	460

	Air drag
	cd x A
	0.27 x 2.17
	0.27 x 2.17
	0.27 x 2.17

	
	
	
	
	

	Power Unit
	
	
	
	

	Configuration/No of cyls/valves Zylinder/Ventile


	
	Straight-4 / 4
	Straight-4 / 4
	Straight-4 / 4

	Engine technology
	
	Common-rail direct fuel injection, turbocharger

with variable intake geometry
	Common-rail direct fuel injection, turbocharger with variable intake geometry
	Common-rail direct fuel injection, turbocharger 

with variable intake geometry

	Capacity, effective
	cc
	1995
	1995
	1995

	Stroke / bore
	mm
	90.0 / 84.0
	90.0 / 84.0
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	16.5
	16.5 

	Fuel
	
	Diesel
	Diesel
	Diesel

	Max output
	kW/hp
	85 / 115
	105 / 143
	135 / 184

	at
	rpm
	4000
	4000
	4000

	Max torque
	Nm/lb-ft
	260/192
	320/236
	380/280

	at 
	rpm
	1750-2500
	1750-2500
	1900-2750

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery / installation
	Ah/–
	90 / luggage compartment
	90 (80) / luggage comp
	90 (80) / luggage comp

	Alternator
	A/W
	180 / 2520
	180 / 2520
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double-joint spring strut axle in aluminium with displaced castor, small positive steering roll radius, compensation of lateral forces, anti-dive

	Suspension, rear
	
	Five-arm axle in lightweight steel

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	292 x 22 / vented
	300 x 24 / vented
	312 x 24 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	300 x 20 / vented
	300 x 20 / vented
	300 x 20 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, 

CBC Cornering Brake Control, DBC Brake Assistant

	Safety
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, three-point inertia-reel belts on all seats, with belt retainer, belt latch tensioner and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Hydraulic rack-and-pinion steering

	Steering ratio, overall
	:1
	16.0
	16.0
	16.0

	Tyres, front/rear
	
	205/55 R16 91H
	205/55 R16 91H
	205/55 R16 91V

	Wheels, front/rear
	
	7J x 16 steel
	7J x 16 steel
	7J x 16 steel


Specifications BMW 3 Series Saloon.
	
	
	BMW 316d
Saloon
	BMW 318d
Saloon
	BMW 320d
Saloon

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-

Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, 

Google directory search, news, realtime weather, office functions, BMW Routes

and music title update online

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant,

Park Distance Control, cruise control, automatic/enhanced emergency call function 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed manual
	Six-speed manual
(optional: six-speed automatic)

	Gear ratios
I
	:1
	4.002
	4.002 (4.171)
	4.110 (4.171) 

	                                  
II
	:1
	2.108
	2.108 (2.340)
	2.248 (2.340)

	                                  
III
	:1
	1.380
	1.380 (1.521)
	1.403 (1.521)

	                                  
IV
	:1
	1.000
	1.000 (1.143)
	1.000 (1.143)

	                                  
V
	:1
	0.780
	0.780 (0.867)
	0.802 (0.867)

	                                   
VI
	:1
	0.645
	0.645 (0.691)
	0.659 (0.691)

	                                   
R
	:1
	3.187
	3.187 (3.403)
	3.727 (3.403)

	Final drive
	:1
	3.071
	3.071 (3.231)
	3.154 (3.154)

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 
(DIN-Leergew.)
	kg/kW
	16.5
	13.5 (13.8)
	10.5 (10.6)

	Output per litre
	kW/hp
	42.6/57.9
	52.6/71.5
	67.7/92.1

	Acceleration
0–100 km/h
	sec
	10.9
	9.1 (9.3)
	7.5 (7.6)

	
standing-start km km
	sec


	32.2


	30.2 (30.5)
	28.1 (28.2)

	In 4th/5th gear
80–120 km/h km/h km/h km/h km/h
	sec
	9.5 / 13.1 (-)
	7.8 / 10.4 (-)
	5.9 / 7.5 (-) 

	Top speed
	km/h
	202
	210
	235 (233)

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, Electric Power Steering,

Auto Start Stop and gearshift point indicator (manual gearbox models), on-demand management of ancillary units, intelligent lightweight construction, 
a/c compressor detachable

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100km
	5.4 
	5.4 (6.8) 
	5.9 (6.8)

	Extra-urban
	ltr/100km
	4.0
	4.0 (4.4)
	4.0 (4.4) 

	Combined
	ltr/100km
	4.5
	4.5 (5.3)
	4.7 (5.3) 

	CO2
	g/km
	118
	119 (140)
	125 (140)

	Emission rating
	
	EU5
	EU5
	EU5  

	
	
	
	
	


Specifications apply to ACEA markets/data relevant to homologation apply in part only to Germany (weight) 

Figures in brackets apply to models with automatic transmission

1)  Oil change




















































