[image: image1.png]


BMW
Media
Information

7/2010

Valid from 9/2010

BMW
Media
Information

7/2010

Valid from 9/2010


Specifications BMW 5 Series Gran Turismo.
530d xDrive, 535d xDrive.
	
	
	BMW 530d xDrive 
Gran Turismo
	
	BMW 535d xDrive 
Gran Turismo

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4998 / 1901 / 1559
	
	4998 / 1901 / 1559

	Wheelbase
	mm
	3070
	
	3070

	Track, front/rear
	mm
	1611 / 1654
	
	1611 / 1654

	Ground clearance
	mm
	145
	
	145

	Turning circle
	m
	12.5
	
	12.5

	Tank capacity
	app ltr
	70
	
	70

	Cooling system incl heater
	ltr
	8.9
	
	8.9

	Engine oil1)
	ltr
	7.2
	
	7.2

	Weight, unladen, to DIN/EU
	kg
	2015 / 2090
	
	2035 / 2110



	Max load to DIN
	kg
	630
	
	600

	Max permissible
	kg
	2645
	
	2635

	Max axle load, front/rear
	kg
	1220 / 1480
	
	1235 / 1480

	Max trailer load (12%),

braked/unbraked
	kg
	2100 / 750
	
	2100 / 750



	Max roofload/max towbar downl
	kg
	75 / 100
	
	75 / 100

	Luggage comp capacity
	ltr
	440 / 590 /1700
	
	440 / 590 / 1700

	Air drag
	Cd  x A
	0.30 x 2.57
	
	0.31 x 2.57

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-six / 4
	
	Straight-six / 4

	Engine technology
	
	Turbocharger with variable intake geometry, common-rail direct fuel injection with piezo-injectors (max injection pressure 1800 bar)
	
	BMW TwinPower Turbo (Variable Twin Turbo), common-rail direct fuel injection with piezo-injectors (max injection pressure 2000 bar)

	Capacity, effective
	cc
	2993
	
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	
	16.5

	Fuel 
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	180 / 245
	
	220 / 300

	at 
	rpm
	4000
	
	4400

	Max torque
	Nm/lb-ft
	540/398
	
	600/442

	at
	rpm
	1750–3000
	
	1500-2500

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	90 / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	180 / 2 520
	
	220 / 3080

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double wishbone axle in aluminium construction, steel springs

	Suspension, rear
	
	Integral-V multi-arm axle in aluminium construction, air suspension with automatic self-levelling, optional Integral Active Steering

(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 36 / vented
	
	348 x 36 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345 x 24 / vented
	
	345 x 24 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, networked with xDrive all-wheel-drive system in ICM Integrated Chassis Management; optional:Adaptive Drive incl Dynamic Damping Control and Dynamic Drive anti-roll stability management

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags at the rear on the outer seats, three-point inertia-reel seat belts on all seats, belt limiters and belt latch tensioners at the front, crash sensors, crash-activated headrests at the front, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion power steering with speed-related steering assistance (Servotronic), 3.1 turns from-lock-to lock,  Integral Active Steering

	Steering transmission, overall
	:1
	19.1
	
	19.1

	Tyres, front/rear
	
	245/50 R18 100Y  RSC
	             
	245/50 R18 100Y RSC



	Rims, front/rear
	
	8J x 18 light-alloy
	
	8J x 18 light-alloy


Specifications BMW 5 Series Gran Turismo.

	
	
	BMW 530d xDrive 
Gran Turismo
	
	BMW 535d xDrive
Gran Turismo

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including features such as Parkinfo, National Info, Google directory service, News, realtime weather, office functions, BMW Routes, Bluetooth Audio Streaming and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and Adaptive Headlight Range Control, High-Beam Assistant, Park Distance Control, back-up camera with Top View, Surround View incl Top View and Side View, BMW Night Vision with detection of persons, Head-Up Display, Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Eight-speed automatic transmission with Steptronic function

	Gear ratios
I
	:1
	4.714
	
	4.714

	                                  
II
	:1
	3.143
	
	3.143

	                                  
III
	:1
	2.106
	
	2.106

	                                  
IV
	:1
	1.667
	
	1.667

	                                  
V
	:1
	1.285
	
	1.285

	                                   
VI
	:1
	1.000
	
	1.000

	                                   
VII
	:1
	0.839
	
	0.839

	                                   
VIII
	:1
	0.667
	
	0.667

	                                   
R
	:1
	3.317
	
	3.317

	Final drive
	:1
	2.563
	
	2.647

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	11.2
	
	9.3

	Output per litre
	kW/hp
	60.1/81.7
	
	73.5/100.0

	Acceleration       0–100 km/h
	sec
	6.9
	
	6.0

	                      Standing-start km
	sec
	27.9
	
	26.3

	Top speed
	km/h
	235
	
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Brake Energy Regeneration, BMW EfficientDynamics Display, Varioserv steering assistance pump, intelligent lightweight construction, active air flap control, on-demand operation of ancillary units, final drive with optimised thermal management

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	8.5
	
	8.9

	Extra-urban
	ltr/100 km
	6.0
	
	6.1

	Combined
	ltr/100 km
	6.9
	
	                                              7.1

	CO2
	g/km
	183
	
	187

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1)  Oil change




















































