[image: image1.png]


BMW
Media
Information

7/210
Valid from 9/2010

Seite 1
BMW
Media
Information

7/2010
Valid from 9/2010


4/2009
Seite 2

Specifications BMW 5 Series Gran Turismo.
535i, 550i.

	
	
	BMW 535i Gran Turismo
	Turismo
	BMW 550i GranTurismo TurTurTTurismo

	
	
	
	
	

	Body
	
	
	
	

	No of doors/seats
	
	4 / 5
	
	4 / 5

	Length/width/height (unladen)
	mm
	4998 / 1901 / 1559
	
	4998 / 1901 / 1559

	Wheelbase
	mm
	3070
	
	3070

	Track, front/rear
	mm
	1611 / 1654
	
	1621 / 1636

	Ground clearance
	mm
	145
	
	145

	Turning circle
	m
	12.2
	
	12.2

	Tank capacity
	app ltr
	70
	
	70

	Cooling system incl heater
	ltr
	9.3
	
	11.4

	Engine oil1)
	ltr
	6.5
	
	8.5

	Weight, unladen, to DIN/EU
	kg
	1940 / 2015
	
	2060 / 2135



	Max load to DIN
	kg
	630
	
	630

	Max permissible
	kg
	2570
	
	2690

	Max axle load, front/rear
	kg
	1165 / 1470
	
	1245 / 1500

	Max trailer load (12%),

braked/unbraked
	kg
	2100 / 750
	
	2100 / 750



	Max roofload/max towbar downl
	kg
	75 / 100
	
	75 / 100

	Luggage comp capacity
	ltr
	440 / 590 /1700
	
	440 / 590 / 1700

	Air drag
	Cd  x A
	0.31 x 2.57
	
	0.32 x 2.57

	Power Unit
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves 
	
	Straight-six / 4
	
	V8 / 4

	Engine technology
	
	BMW TwinPower Turbo, High Precision Injection, fully variable intake valve management (VALVETRONIC)
	
	BMW TwinPower Turbo, High Precision Injection

	Capacity, effective
	cc
	2979
	
	4395

	Stroke / bore
	mm
	89.6 / 84.0
	
	88.3 / 89.0

	Compression ratio
	:1
	10.2
	
	10.0

	Fuel 
	
	min RON 91
	
	min RON 91

	Max output
	kW/hp
	235 / 306
	
	300 / 407

	at 
	rpm
	5800
	
	5500 – 6400

	Max torque
	Nm/lb-ft
	400/295
	
	600/442

	At
	rpm
	1200–5000
	
	1750-4500

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location
	Ah/–
	90 / luggage comp
	
	90 / luggage comp

	Alternator
	A/W
	180 / 2 520
	
	180 / 2520

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double wishbone axle in aluminium construction, steel springs

	Suspension, rear
	
	Integral-V multi-arm axle in aluminium construction, air suspension with automatic self-levelling, optional Integral Active Steering

(air suspension with self-levelling standard on long-wheelbase version)

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	348 x 36 / vented
	
	374 x 36 / vented

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter
	mm
	345 x 24 / vented
	
	370 x 24 / vented

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, Fading Compensation, Start-Off Assistant, optional: Adaptive Drive incl Dynamic Damper Control and Dynamic Drive anti-roll stability management

	Safety equipment
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags at the rear on the outer seats, three-point inertia-reel seat belts on all seats, belt limiters and belt latch tensioners at the front, crash sensors, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion power steering with speed-related steering assistance (Servotronic), 3.1 turns from-lock-to lock,  Integral Active Steering

	Steering transmission, overall
	:1
	19.1
	
	19.1

	Tyres, front/rear
	
	245/55 R17 100Y  RSC
	
	245/45 R19 98Y RSC/

275/40 R 19 101 Y RSC



	Rims, front/rear
	
	8J x 18 light-alloy
	
	8.5J x 19 light-alloy/

9.5Jx19 LM

):%Jx!9




Specifications BMW 5 Series Gran Turismo

	
	
	BMW 535i Gran Turismo
	
	BMW 550i Gran Turismo

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including Enquiry Service, remote-control function and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including features such as Parkinfo, National Info, Google directory service, News, Realtime Weather, Office Functions, BMW Routes, Bluetooth Audio Streaming and online music title update

	Safety
	
	Optional: Adaptive Headlights with Bending Lights, variable light distribution and Adaptive Headlight Range Control, High-Beam Assistant, Park Distance Control, back-up camera with Top View, Surround View incl Top View and Side View,, BMW Night Vision with detection of persons, Head-Up Display, Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Eight-speed automatic transmission with Steptronic function

	Gear ratios
I
	:1
	4.714
	
	4.714

	                                  
II
	:1
	3.143
	
	3.143

	                                  
III
	:1
	2.106
	
	2.106

	                                  
IV
	:1
	1.667
	
	1.667

	                                  
V
	:1
	1.285
	
	1.285

	                                   
VI
                                        VII                

                                        VIII                                      
	:1
:1
:1              
	1.000
0.839
0.667
	
	1.000
0.839
0.667



	                                   
Reverse
	:1
	3.295
	
	3.317

	Final drive
	:1
	3.077
	
	3.077

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	8.6
	
	6.9

	Output per litre
	kW/hp
	75.5/102.7
	
	68.3/92.9

	Acceleration       0–100 km/h
	sec
	6.3
	
	5.5

	                      Standing-start km
	sec
	26.2
	
	24.4

	Top speed
	km/h
	250
	
	250

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features
	
	Brake Energy Regeneration, BMW EfficientDynamics Display, Varioserv steering assistance pump, intelligent lightweight construction, on-demand operation of ancillary units, final drive with optimised thermal management

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban
	ltr/100 km
	12.3
	
	16.2

	Extra-urban
	ltr/100 km
	5.9
	
	8.3

	Combined
	ltr/100 km
	8.9
	
	                                            11.2

	CO2
	g/km
	209
	
	263

	Emission rating
	
	EU5  
	
	EU5  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications applicable to ACEA markets/homologation data applicable in part only to Germany (weight),

1)  Oil change




















































