Specifications.
The new BMW 5 Series Sedan. 
520d, 525d.

|  |  | BMW 520d | BMW 525d |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Body |  |  |  |  |  |  |
| No of doors/seats |  | 4 / 5 | 4 / 5 |  |  |  |
| Length/width/height (unladen) | mm | 4899 / 1860 / 1464 | 4899 / 1860 / 1464 |  |  |  |
| Wheelbase | mm | 2968 | 2968 |  |  |  |
| Track, front/rear | mm | 1600 / 1627 | 1600 / 1627 |  |  |  |
| Ground clearance | mm | 141 | 141 |  |  |  |
| Turning circle | m | 11.95 | 11.95 |  |  |  |
| Tank capacity | app ltr | 70 | 70 |  |  |  |
| Cooling system incl heater | ltr | 7.0 | 8.0 |  |  |  |
| Engine oil1) | ltr | 5.2 | 7.2 |  |  |  |
| Weight, unladen, to DIN/EU | kg | 1620 / 1695 (1625 / 1700) | 1690 / 1765 (1705 / 1780) |  |  |  |
| Max load to DIN | kg | 610 | 610 |  |  |  |
| Max permissible | kg | 2230 (2235) | 2300 (2315) |  |  |  |
| Max axle load, front/rear | kg | 1060 / 1275 | 1100 / 1280 (1100 / 1285) |  |  |  |
| Max trailer load (12%), braked/unbraked | kg | 2000 / 750 | 2000 / 750 |  |  |  |
| Max roofload/max towbar dwnl | kg | 100/90 | 100/90 |  |  |  |
| Luggage comp capacity | ltr | 520 | 520 |  |  |  |
| Air resistance | Cd x A | 0.28 x 2.35 | 0.28 x 2.35 |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Power Unit |  |  |  |  |  |  |
| Config/No of cyls/valves |  | Straight-four / 4 | Straight-six / 4 |  |  |  |
| Engine technology |  | Turbo with vari intake geom, common-rail direct fuel injec, piezo-injectors (1800 max pressure) injection pressue | Turbo with variable intake geom, common-rail direct fuel injec, piezo-injectors (1800 bar max pressure) |  |  |  |
| Capacity, effective | cc | 1995 | 2993 |  |  |  |
| Stroke / bore | mm | 90.0 / 84.0 | 90.0 / 84.0 |  |  |  |
| Compression ratio | :1 | 16.5 | 16.5 |  |  |  |
| Fuel grade |  | Diesel | Diesel |  |  |  |
| Max output | kW/hp | 135 / 184 | 150 / 204 |  |  |  |
| at | rpm | 4000 | 4000 (3750) |  |  |  |
| Max torque | Nm/lb-ft | 380/280 | 450/332 |  |  |  |
| At | rpm | 1750-2750 | 1750-2500 |  |  |  |
| Electrical System |  |  |  |  |  |  |
| Battery/installation | Ah/– | 80 (90) / luggage comp | 90 / luggage comp |  |  |  |
| Alternator | A/W | 180 / 2520 | 180 / 2520 |  |  |  |
| Driving Dynamics and Safety |  |  |  |  |  |  |
| Suspension, front |  | Double track control arm axle with separate lower track arm level, aluminium, small steering roll radius, anti-dive |  |  |  |  |
| Suspension, rear |  | Integral-V multi-arm axle, aluminium, with steering function, anti-squat and anti-dive, double acoustic separation (air suspension with self-levelling standard on long-wheelbase version) |  |  |  |  |
| Brakes, front |  | Single-piston aluminium swing-calliper disc brakes in frame structure |  |  |  |  |
| Diameter | mm | 330 x 24 / inner-vented |  | 330 x 24 / inner-vented |  |  |
| Brakes, rear |  | Single-piston swing-calliper disc brakes |  |  |  |  |
| Diameter | mm | 330 x 20 / inner-vented |  | 330 x 20 / inner-vented |  |  |
| Driving stability systems |  | Standard: DSC incl ABS and DTC Dynamic Traction Control, CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking function, Fading Compensation, Start-Off Assistant; optional: Dynamic Damper Control |  |  |  |  |
| Safety equipment |  | Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags for front and rear seats, three-point inertia-reel belts on all seats with belt stopper, belt tightener and belt force limiter at the front, crash-activated headrests at the front, crash sensors, Tyre Defect Indicator |  |  |  |  |
| Steering |  | Electric Power Steering (EPS) with Servotronic; optional: Integral Active Steering |  |  |  |  |
| Steering transmission, overall | :1 | 17.1 | 17.1 |  |  |  |
| Tyres, front/rear |  | 225/55 R17 97W | 225/55 R17 97W |  |  |  |
| Rims, front/rear |  | 8J x 17 light-alloy | 8J x 17 light-alloy |  |  |  |

Specifications.
The new BMW 5 Series Sedan. 
520d, 525d.

|  |  |  | BMW 520d |  |  | BMW 525d |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMW ConnectedDrive |  |  |  |  |  |  |  |  |  |
| Comfort |  |  |  |  | Opt: BMW Assist including Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals |  |  |  |  |
| Infotainment |  |  |  | Opt: internet, BMW Online incl Parkinfo, National Info, Google Dir Service, News, Realtime Weather, Office Functions, Bluetooth Audio Streaming, BMW Routes Routes |  |  |  |  |  |
| Safety |  |  |  | Optional: Adaptive Headlights with Bending Lights, variable light distribution and adaptive headlight range control, High-Beam Assistant, Park Distance Control, back-up camera, Surround View including Side View and Top View, BMW Night Vision with detection of persons, Head-Up Display, Parking Assistant, collision warning with brake activation function in combination with Active Cruise Control with Stop & Go, Lane Change Warning, Lane Departure Warning, Speed Limit Info, Speed Limit Device, Automatic/Enhanced Emergency Call |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Type of gearbox |  |  |  | Six-speed manual (optional: eight-speed automatic with Steptronic) |  |  |  |  |  |
| Gear ratios I |  | :1 |  | 4.110 (4.714) |  | 4.110 (4.714) |  |  |  |
| II |  | :1 |  | 2.248 (3.143) |  | 2.248 (3.143) |  |  |  |
| III |  | :1 |  | 1.403 (2.106) |  | 1.403 (2.106) |  |  |  |
| IV |  | :1 |  | 1.000 (1.667) |  | 1.000 (1.667) |  |  |  |
| V |  | :1 |  | 0.802 (1.285) |  | 0.802 (1.285) |  |  |  |
| VI |  | :1 |  | 0.659 (1.000) |  | 0.659 (1.000) |  |  |  |
| VII |  | :1 |  | – (0.839) |  | – (0.839) |  |  |  |
| VIII |  | :1 |  | – (0.667) |  | – (0.667) |  |  |  |
| R |  | :1 |  | 3.727 (3.295) |  | 3.727 (3.295) |  |  |  |
| Final drive |  | :1 |  | 3.385 (2.929) |  | 3.154 (2.471) |  |  |  |
| Performance |  |  |  |  |  |  |  |  |  |
| Power-to-weight ratio | kg/kW |  |  | 12.1 (12.2) |  | 11.3 (11.4) |  |  |  |
| Output per litre | kW/hp |  |  | 67.7/92.1 |  | 50.1/68.1 |  |  |  |
| Acceleration 0–100 km/h | sec |  |  | 8.1 (8.1)2) |  | 7.2 (7.2) |  |  |  |
| Standing-start km | sec |  |  | 29.0 (29.0)2) |  | 27.7 (27.8) |  |  |  |
| In 4th gear 80–120 km/h | sec |  |  | 6.9 (–)2) |  | 6.3 (–) |  |  |  |
| Top speed | km/h |  |  | 227 (225)2) |  | 236 (236) |  |  |  |
| BMW EfficientDynamics |  |  |  |  |  |  |  |  |  |
| BMW EfficientDynamics, standard features |  |  |  | Brake Energy Regeneration with recuperation display, Auto Start Stop (BMW 520d with manual gearbox) gearshift point indicator (on manual gearbox models), intelligent lightweight construction, on-demand operation of ancillary units, air flap control, Electric Power Steering (EPS), detachable a/c compressor, tyres with reduced roll resistance |  |  |  |  |  |
| Fuel Consump in EU Cycle |  |  |  |  |  |  |  |  |  |
| Urban | ltr/100 km |  |  | 5.9 (6.4) |  | 8.1 (7.7) |  |  |  |
| Extra-urban | ltr/100 km |  |  | 4.3 (4.5) |  | 5.1 (5.1) |  |  |  |
| Combined | ltr/100 km |  |  | 4.9 (5.2) |  | 6.2 (6.1) |  |  |  |
| CO2 | g/km |  |  | 129 (137) |  | 162 (160) |  |  |  |
| Emission rating |  |  |  | EU5 |  | EU5 |  |  |  |

Specifications applicable to ACEA markets, data relevant to homologation applicable in part only to Germany (weight)

Figures in brackets apply to automatic transmission models

1)  Oil change

2)  EU6 with optional BMW BluePerformance technology