[image: image1.png]


BMW
Media
Information

7/2010

Valid from 10/2010

BMW
Media
Information

7/2010

Valid from 10/2010


Specifications BMW X5.
X5 xDrive30d, X5 xDrive40d
	
	
	BMW X5 xDrive30d
	
	BMW X5 xDrive40d

	
	
	
	
	

	Body      
	
	
	
	

	No of doors/seats
	
	5 / 5 (7) 1)
	
	5 / 5 (7) 1)

	Length/width/height (unladen)
	mm
	4857 / 1933 / 1776 2)
	
	4857 / 1933 / 1776 2)

	Wheelbase
	mm
	2933
	
	2933

	Track, front/rear     
	mm
	1644 / 1650
	
	1644 / 1650

	Turning circle
	m
	12.8
	
	12.8

	Tank capacity
	app ltr
	85
	
	85

	Cooling system incl heater  
	ltr
	10.4
	
	10.4

	Engine oil3)
	ltr
	7.5
	
	7.5

	Weight, unladen, to DIN/EU
	kg
	2075 / 2150


	
	2110 / 2185



	Max load to DIN
	kg
	680 (890) 1)
	
	680 (890) 1)

	Max permissible  
	kg
	2755 (2965) 1)
	
	2790 (3000) 1)

	Max axle load, front/rear  
	kg
	1275 / 1545

(1280 / 1745) 1)
	
	1300 / 1555
(1305 / 1755) 1)


	Max trailer load (12%)
braked/unbraked
	kg
	3500 / 750


	
	3500 / 750



	Max roofload/max towbar downl
	kg
	100/140
	
	100/140

	Luggage comp capacity 
	ltr
	620-1750
	
	620-1750

	Air drag     
	cd x A
	0.34 x 2.87
	
	0.34 x 2.87

	
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves     Zylinder/Ventile


	
	Straight-6 / 4
	
	Straight-6 / 4

	Engine technology 
	 
	Turbocharger with variable intake geometry, common-rail direct injection with piezo-injectors (max. injection pressure: 1800 bar)
	
	BMW TwinPower Turbo (Variable Twin Turbo), common-rail direct fuel injection with piezo-injectors (max injection pressure: 2000 bar)

	Capacity, effective
	cc 
	2993
	
	2993

	Stroke / bore
	mm
	90.0 / 84.0
	
	90.0 / 84.0

	Compression ratio
	:1
	16.5
	
	16.5

	Fuel      
	
	Diesel
	
	Diesel

	Max output
	kW/hp
	180 / 245
	
	225 / 306

	at
	rpm  
	4000
	
	4400

	Max torque
	Nm/lb-ft
	540/398
	
	600/442

	at
	rpm
	1750-3000
	
	1500-2500

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/location  
	Ah/–
	90 / luggage comp
	
	90 / luggage comp

	Alternator   
	A/W
	170 / 2380
	
	170 / 2380

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front 
	
	Double wishbone axle, small, negative steering roll radius, anti-dive

	Suspension, rear    
	
	Integral-IV rear axle, multi-dimensional suspension with anti-squat and anti-dive

	Brakes, front
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	332 x 30 / vented  
	
	348 x 30 / vented  

	Brakes, rear
	
	Single-piston swing-calliper disc brakes

	Diameter   
	mm
	320 x 20 / vented  
	
	320 x 20 / vented  

	Driving stability systems
	  
	Standard: DSC incl ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, Dry Braking, 
Fading Compensation, Start-Off Assistant; DSC networked to xDrive all-wheel drive, Dynamic Brake Compensation, HDC Hill Descent Control, 
optional: Adaptive Drive

	Safety equipment      
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags on the first and second row of seats, crash-activated headrests at the front, three-point inertia-reel belts on all seats, with belt latch tensioner, belt force limiter and belt retainer at the front, crash sensors, 
roll-over sensors, Tyre Defect Indicator

	Steering
	
	Rack-and pinion power steering with Servotronic, optional: Active Steering

	Steering transmission, overall
	:1
	19.5
	
	19.5

	Tyres, front/rear  
	255/55 R18 109H XL RSC
	255/55 R18 109H XL RSC

	Rims, front/rear   
	
	8.5J x 18 light-alloy
	
	8.5J x 18 light-alloy


Specifications BMW X5.

	
	
	BMW X5 xDrive30d
	
	BMW X5 xDrive40d

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist incl Enquiry Service, remote control functions and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online incl Parkinfo, National Info, Google directory search, news, realtime weather, office functions, BMW Routes and online music title update

	Safety    
	
	Optional: Adaptive Headlights with Bending Lights, High-Beam Assistant, Park Distance Control, Side View, back-up camera with Top View, Head-Up Display, Cruise Control with Stop & Go, Lane Departure Warning, Speed Limit Info, Automatic/Enhanced Emergency Call

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Eight-speed automatic with Steptronic

	Gear ratios                       I
	:1
	4.714
	
	4.714

	                                  
II
	:1
	3.143
	
	3.143

	                                  
III
	:1
	2.105
	
	2.105

	                                  
IV
	:1
	1.667
	
	1.667

	                                  
V
	:1
	1.285
	
	1.285

	                                   
VI
	:1
	1.000
	
	1.000

	                                   
VII
	:1
	0.839
	
	0.839

	                                   
VIII
	:1
	0.667
	
	0.667

	                                   
Reverse
	:1
	3.317
	
	3.317

	Final drive         
	:1
	3.154
	
	3.154

	
	
	
	
	

	Performance   
	
	
	
	

	Power-to-weight ratio           
	kg/kW
	11.5
	
	9.4

	Output per litre
	kW/hp
	60.1/81.7
	
	75.2/102.3

	Acceleration 
0–100 km/h
	sec
	7.6
	
	6.6

	                           standing-start km
	sec
	28.5
	
	27.0

	in 4th/5th gear     80–120 km/h km/h km/h km/h km/h
	sec
	-
	
	-

	Top speed            
	km/h
	210 (222) 4)
	
	236

	
	
	
	
	

	Offroad Features
	
	
	
	

	Overhang angle,
front/rear
	
°
	
25.5/22.7
	
	
25.5/22.7

	Ramp angle
	°
	19.8
	
	19.8

	Ground clearance, unladen weight,ergewicht
	mm
	222
	
	222

	Water depth (at 7 km/h)
	mm
	500
	
	500

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics standard features    
	
	Brake Energy Regeneration, on-demand control of ancillaries, intelligent lightweight construction, a/c compressor detachable, tyres with reduced roll resistance

	
	
	
	
	

	Fuel Consump in EU Cycle
	
	
	
	

	Urban    
	l/100 km
	8.7
	
	8.8

	Extra-urban  
	l/100 km
	6.7
	
	6.8

	Combined  
	l/100 km
	7.4
	
	7.5

	CO2
	g/km
	195
	
	198

	Emission rating    
	
	EU5  
	
	EU5  

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Specifications apply to ACEA markets/homologation-relevant data applicable in part only to Germany (weight, top speed)

1) With optional third row of seats

2) Height without roof aerial: 1739 mm

3) Oil change

4) With optional high speed set-up (depending on market)




















































