[image: image1.png]


BMW
Media Information

9/2010

Valid from 9/2010

BMW
Media Information

9/2010

Valid from 9/2010


Specifications BMW X6 M.

	
	
	BMW X6 M
	
	

	
	
	
	
	

	Body
	
	
	
	

	No of doors / seats
	
	5 / 4
	
	

	Length/width/height (unladen)
	mm
	4876 / 1983 / 1684
	
	

	Wheelbase
	mm
	2933
	
	

	Track, front/rear
	mm
	1660 / 1672
	
	

	Turning circle
	m
	12.8
	
	

	Tank capacity
	approx ltr
	85
	
	

	Cooling system incl heater
	ltr
	17.0
	
	

	Engine oil1)
	ltr
	8.5
	
	

	Weight, unladen, to DIN/EU
	kg
	2305 / 2380


	
	

	Max load to DIN
	kg
	600
	
	

	Max permissible
	kg
	2905
	
	

	Max axle load, front/rear
	kg
	1400 / 1520
	
	

	Max trailer load (12%),

braked/unbraked
	kg
	3000 / 750
	
	

	Max roofload/max twbar downld download
	kg
	100/120
	
	

	Luggage comp capacity
	ltr
	570 – 1450
	
	

	Air resistance
	CD x A
	0.38 x 2.85
	
	

	
	
	
	
	

	Power Unit
	
	
	
	

	Config/No of cyls/valves


	
	V90/ 8 / 4
	
	

	Engine technology
	 
	M TwinPower Turbo with exhaust gas manifold connecting both rows of cylinders. Twin Scroll Twin Turbo technology. 
High Precision Injection. double-VANOS
	
	

	Capacity, effective
	cc
	4395
	
	

	Bore / stroke
	mm
	88.3 / 89.0
	
	

	Compression ratio
	:1
	9.3
	
	

	Fuel grade
	
	 min RON 91
	
	

	Max output
	kW/hp
	408 / 555
	
	

	at
	rpm
	6000
	
	

	Max torque
	Nm/lb-ft
	680/501
	
	

	at
	rpm
	1500 – 5650
	
	

	
	
	
	
	

	Electrical System
	
	
	
	

	Battery/installation
	Ah/–
	90 / luggage comp
	
	

	Alternator
	A/W
	210 / 2940
	
	

	
	
	
	
	

	Driving Dynamics and Safety
	

	Suspension, front
	
	Double wishbone axle with M-specific elastokinematics; small, negative steering roll radius, anti-dive 

	Suspension, rear
	
	Integral-IV rear axle with M-specific elastokinematics, multi-dimensional suspension, anti-squat and anti-dive, air suspension with automatic ride height control 

	Brakes, front
	
	Four-piston swing-calliper disc brakes
	Four-piston swing-calliper disc brakes
	

	Diameter
	mm
	395 x 36, vented 
	395 x 36, vented 
	

	Brakes, rear
	
	Single-piston swing-calliper disc brakes
	Single-piston swing-calliper disc brakes
	

	Diameter
	mm
	385 x 24, vented 
	385 x 24, vented 
	

	Driving stability systems
	  
	Standard: DSC incl ABS and M Dynamic Mode, CBC Cornering Brake Control, DBC Dynamic Brake Control, ADB-X electronic differential lock, Dry Braking, Fading Compensation, Start-Off Assistant; Dynamic Stability Control interacting with xDrive all-wheel drive system, Dynamic Performance Control, HDC Hill Descent Control, Adaptive Drive

	Safety features
	  
	Standard: airbags for driver and front passenger, side airbags for driver and front passenger, head airbags front and rear, crash-activated headrests at the front, three-point inertia-reel seat belts on all seats, with belt latch tensioners, belt force limiters and belt stoppers at the front,  crash sensors, rollover sensors, Tyre Defect Indicator

	Steering
	
	Rack-and-pinion steering, power-assisted, with M-specific Servotronic function

	Steering transmission ratio, overall
	:1
	19.5
	
	

	Tyres, front/rear
	
	275/40 R20 106Y XL RSC / 315/35 R20 110Y XL RSC
	
	

	Wheels, front/rear
	
	10J x 20 light-alloy / 11J x 20 light-alloy
	
	


Specifications BMW X6 M.

	
	
	BMW X6 M
	
	

	
	
	
	
	

	BMW ConnectedDrive
	
	
	
	

	Comfort
	
	Optional: BMW Assist including features such as Enquiry Service, remote-control functions and V-Info+, BMW TeleServices, integration of mobile terminals

	Infotainment
	
	Optional: access to the internet, BMW Online including features such as Parkinfo, National Info, Google Directory Search, news, weather in real time, office functions, BMW Routes, Bluetooth Audio Streaming  and music title update online

	Safety
	
	Standard: Adaptive Headlights with Bending Lights, Park Distance Control, Head-Up Display, cruise control with brake function; optional: back-up camera with Top View, Side View, Speed Limit Info, High-Beam Assistant, Automatic/Enhanced Emergency Call 

	
	
	
	
	

	Transmission
	
	
	
	

	Type of transmission
	
	Six-speed automatic sports gearbox with Steptronic, electronic gearshift lever and shift paddles on the steering wheel

	Gear ratios
I
	:1
	4.171
	
	

	                                  
II
	:1
	2.340
	
	

	                                  
III
	:1
	1.521
	
	

	                                  
IV
	:1
	1.143
	
	

	                                  
V
	:1
	0.867
	
	

	                                   
VI
	:1
	0.691
	
	

	                                   
R
	:1
	3.403
	
	

	Final drive
	:1
	3.909
	
	

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio 

	kg/kW
	5.6


	
	

	Output per litre
	kW/hp
	92.8/126.2
	
	

	Acceleration
0–100 km/h 
	sec
	4.7
	
	

	
0–200 km/h
	sec
	16.9
	
	

	                         standing-start km
	sec
	23.5
	
	

	In fourth/fifth gear
  80–120 km/h
	sec
	4.3 / 5.1
	
	

	Top speed
	km/h
	250 (275)2)
	
	

	
	
	
	
	

	Offroad Features
	
	
	
	

	Overhang angle, 

front/rear

	
°
	
22 / 21.5
	
	


	Ramp angle
	°
	18.6
	
	

	Ground clearance, unladen
	mm
	205
	
	

	
	
	
	
	

	BMW EfficientDynamics
	
	
	
	

	BMW EfficientDynamics, standard features
	
	Brake Energy Regeneration, on-demand control of ancillary units, intelligent lightweight construction, detachable a/c compressor, anti-roll stability control with on-demand pump action

	
	
	
	
	

	Fuel Cons in the EU Cycle
	
	
	
	

	Urban
	ltr/100km
	19.3
	
	

	Extra-urban
	ltr/100km
	10.8
	
	

	Combined
	ltr/100km
	13.9
	
	

	CO2
	g/km
	325
	
	

	Emission rating
	
	EU5  
	
	

	
	
	
	
	


Specifications apply to ACEA markets/homologation-relevant data apply in part only to Germany (weight)

1)  Oil change

2)  In conjunction with optional M Driver’s Package



















































