PAGE  
BMW
Press
Information

12/2005
 


Technical specifications.
K 1200 GT.
	
	
	K 1200 GT
	
	
	

	Engine
	
	
	
	
	

	Capacity
	cm3
	1157 
	
	
	

	Bore/stroke
	mm
	79/59
	
	
	

	Output
	kW/PS
	112/152
	
	
	

	at engine speed
	min–1
	9 500
	
	
	

	Torque
	Nm
	130
	
	
	

	at engine speed
	min–1
	7 750
	
	
	

	No. of cylinders
	
	in-line
	
	
	

	Compression/fuel
	
	4
	
	
	

	Valve/gas control
	
	13:1/S PLUS (98 octane)
	
	
	

	Valves per cylinder
	
	DOHC (double overhead camshaft)
	
	
	

	Ø intake/outlet
	
	4
	
	
	

	Throttle valve diameter
	mm
	32/27.5
	
	
	

	at engine speed
	mm
	46
	
	
	

	Carburetion
	
	BMSK
	
	
	

	
	
	
	
	
	

	Electrical system
	
	
	
	
	

	Alternator
	W
	945
	
	
	

	Battery
	V/Ah
	12/19 
	
	
	

	Headlamp
	W
	Low beam 1 x H7/55W
	
	
	

	
	
	High beam 2 x H7/55W
	
	
	

	Starter
	kW
	0.7
	
	
	

	
	
	
	
	
	

	Power transmission/gearbox
	
	
	
	
	

	Clutch
	
	Multi-plate clutch in oil bath, mechanically operated
	
	

	Gearbox
	
	Constant mesh 6-speed transmission
	
	
	

	Primary ratio
	
	1.559
	
	
	

	Gear ratio steps                         I
	
	2.521
	
	
	

	


II
	
	1.842
	
	
	

	


III
	
	1.455
	
	
	

	


IV
	
	1.287
	
	
	

	


V
	
	1.143
	
	
	

	


VI
	
	1.015
	
	
	

	Rear-wheel drive
	
	Universal 
	
	
	

	Transmission ratio
	
	2.82
	
	
	

	
	
	
	
	
	

	Chassis
	
	
	
	
	

	Frame type
	
	Bridge frame in aluminium, engine load bearing
	
	

	Wheel control, front
	
	BMW Motorrad Duolever
	
	
	

	Wheel control, rear
	
	Cast aluminium single-side swing arm with
 BMW Motorrad Paralever
	
	
	

	Spring travel, front/rear
	mm
	115/135
	
	
	

	Wheel castor offset
	mm
	112
	
	
	

	Wheelbase
	mm
	1571
	
	
	

	Steering head angle
	°
	60.6
	
	
	

	Brakes
	front
	EVO brake system with dual disc, floating brake discs, 
diameter 320 mm, 4-piston fixed caliper
	
	
	

	
	rear
	Single disc brake, diameter 294 mm, 
2-piston fixed caliper
	
	
	

	Wheels
	
	Cast aluminium wheels
	
	
	

	
	front
	3.50 x 17 
	
	
	

	
	rear
	5.50 x 17 
	
	
	

	Tyres
	front
	120/70 ZR 17
	
	
	

	
	rear
	180/55 ZR 17
	
	
	

	
	
	
	
	
	

	Dimensions and weights
	
	
	
	
	

	Total length
	mm
	2 318
	
	
	

	Total length incl. mirrors
	mm
	965
	
	
	

	Total width not incl. mirrors
	mm
	795
	
	
	

	Seat height
	mm
	820 and 840 (optional extras), low seat 800 and 820)
	
	
	

	Dry weight
	kg
	249
	
	
	

	DIN unladen weight, road ready
	kg
	282
	
	
	

	Maximum authorised weight
	kg
	520
	
	
	

	Fuel tank capacity
	l
	24
	
	
	

	
	
	
	
	
	

	Performance figures
	
	
	
	
	

	Fuel consumption
	
	
	
	
	

	90 km/h
	l/100 km
	4.8
	
	
	

	120 km/h
	l/100 km
	5.8
	
	
	

	Acceleration
	
	
	
	
	

	0–100 km/h
	s
	3.1
	
	
	

	Maximum speed
	s
	–
	
	
	

	Performance figures
	km/h
	>200
	
	
	






































