Specifications MINI Cooper S. (May 2007)

| Body L |  | MINI Cooper S |  | MINI Cooper S Automatic |  |
| --- | --- | --- | --- | --- | --- |
| No of doors/seats L |  | 3/4 |  | 3/4 |  |
| Length/width/height (unladen) ) | mm | 3714 / 1683 / 1407 |  | 3714 / 1683 / 1407 |  |
| Wheelbase | mm | 2 467 |  | 2 467 |  |
| Track front/rear wgsdfg | mm | 1453 / 1461 |  | 1453 / 1461 |  |
| Turning circle | m | 10,07 |  | 10,07 |  |
| Fuel tank capacity | approx. ltr | 50 |  | 50 |  |
| Cooling system including heater l | ltr | 5,2 |  | 5,2 |  |
| Engine oil Getriebeöl | ltr | 4,2 |  | 4,2 |  |
| Transmission fluid incl final drive | ltr | Permanent filling |  | Permanent filling |  |
| Weight, unladen, according to DIN/EU1 | kg | 1130 / 1205 |  | 1155 / 1230 |  |
| Max load according to DIN | kg | 450 |  | 450 |  |
| Max permissible weight to DIN | kg | 1580 |  | 1605 |  |
| Max permissible axle load front/rear | kg | 885 / 755 |  | 885 / 755 |  |
| Max permissible trailer load2 |  |  |  |  |  |
| braked (12%)/unbraked | kg | - / - |  | - / - |  |
| Max roof load / max towbar download | kg | 75 / - |  | 75 / - |  |
| Luggage compartment according to DIN | l | 160-680 |  | 160-680 |  |
| Air resistance | cX x A | 0,36 x 1,97 |  | 0,36 x 1,97 |  |
| Engine |  |  |  |  |  |
| Configuration/No of cylinders/valves Zylinder/Ventile |  | Inline / 4 / 4 |  | Inline / 4 / 4 |  |
| Engine management |  | MED 17.2 |  | MED 17.2 |  |
| Capacity | cc | 1598 |  | 1598 |  |
| Bore/stroke | mm | 77 / 85,8 |  | 77 / 85,8 |  |
| Compression ratio | :1 | 10,5:1 |  | 10,5:1 |  |
| Fuel | RON | From 91 ROZ |  | From 91 ROZ |  |
| Max output | kW/bhp | 128 / 175 / 170 |  | 128 / 175 |  |
| at | rpm | 5500 |  | 5500 |  |
| Max torque | Nm | 240 |  | 240 |  |
| at | rpm | 1600-5000 |  | 1600-5000 |  |
| Electrical system |  |  |  |  |  |
| Battery/installation | Ah/– | 46 / rear |  | 46 / rear |  |
| Alternator | A/W | 120 / 1680 |  | 120 / 1680 |  |
| Chassis and suspension |  |  |  |  |  |
| Front suspension | Singe-joint Mc Pherson spring strut axle with anti-drive |  |  |  |  |
| Rear suspension | Longitudinal struts with centrally-pivoted control arms, Z-axle |  |  |  |  |
| Brake, front |  | Disc, vented |  |  |  |
| Diameter | mm | 294x22 |  |  |  |
| Brake, rear |  | Disc brake |  |  |  |
| Diameter | mm | 259x10 |  |  |  |
| Driving stability systems | Hydraulic two-circuit brake system with anti-lock brakes (ABS), Electronic Brake Force Distribution (EBD), Cornering Brake Control (CBC) and Brake Assistant. Optional: ASC+T traction control, Dynamic Stability Control (DSC) with Hill Start Assistant, Limited-slip differential. |  |  |  |  |
| Steering | Electric power steering (EPS)) |  |  |  |  |
| Overall transmission ratio | :1 | 14,1 |  | 14,1 |  |
| Type of transmission |  | 6- gear manual |  | 6-gear automatic |  |
| Gear ratios I | :1 | 3,308 |  | 4,044 |  |
| II | :1 | 2,130 |  | 2,371 |  |
| III | :1 | 1,483 |  | 1,556 |  |
| IV | :1 | 1,139 |  | 1,159 |  |
| V | :1 | 0,949 |  | 0,852 |  |
| VI | :1 | 0,816 |  | 0,672 |  |
| R | :1 | 3,231 |  | 3,193 |  |
| Final drive ratio | :1 | 3,647 |  | 3,683 |  |
| Tyres |  | 195 / 55 R16 87V RSC |  | 195 / 55 R16 87V RSC |  |
| Wheels |  | 6,5Jx16 Alloy |  | 6,5Jx16 Alloy |  |
| Performance |  |  |  |  |  |
| Power-to-weight ratio, DIN |  | 8,8 |  | 9,0 |  |
| Output per litre | kg/kW | 80,1 |  | 80,1 |  |
| Acceleration 0–62 mph | kW/ltr | 7,1 |  | 7,3 |  |
| 0–1000 m | s | 27,6 |  | 27,9 |  |
| in 4th/5th gear 50–75 mph | s | 5,6 / 7,0 |  | - / - |  |
| Top speed | s | 225 |  | 220 |  |
| Consumption in the EU cycle | km/h |  |  |  |  |
| Urban |  | 7,9 |  | 9,7 |  |
| Extra-urban | ltr/100 km | 5,2 |  | 5,3 |  |
| Combined | ltr/100 km | 6,2 |  | 6,9 |  |
| CO2 | ltr/100 km | 149 |  | 165 |  |
| Miscellaneous | g/km |  |  |  |  |
| Emission category |  | EU4 |  | EU4 |  |
| Ground clearance | TPL/semi/full | 142 |  | 142 |  |