	
	
	Media Information

	Date
	
	8 July 2003

	Subject
	
	 REF \* CHARFORMAT Thema  \* MERGEFORMAT 

	Page
	
	2

	
	
	



[image: image1.png]


Media Information
July 2003


Company

Bayerische

Motoren Werke

Aktiengesellschaft

Postal Address

BMW AG

80788 München

Telephone

089-382-24697
Internet

www.bmwgroup.com

500 layers against the heat – BMW introduces new heat-reflecting laminated glass technology



Munich. In a world first, BMW has introduced a pioneering heat-reflecting laminated glass technology into the new BMW 5 and 7 Series. The rear windscreen featuring more than 500 ultra-thin plastic embedded layers can be specially requested as part of the climate comfort laminated glass option. “With more than 500 layers of plastic, the laminated glass pane reduces heat build-up in the interior of the car. Each of the layers is only around 0.00001 millimetres thick, which is some fifty thousand times thinner than a human hair,” says BMW project manager Gerald Buck.

The result: enhanced passenger comfort on hot days thanks to glazing which reflects the heat-generating infrared component of the sun’s rays and thus noticeably reduces heat build-up in the cabin even at temperatures of over 30 degrees Celsius. 

Direct sunlight, especially if the car is stationary, causes a powerful build-up of heat in the interior. Compared to a car with standard glazing, the new climate comfort glass affords several advantages:

Firstly, it reduces the internal air temperature by some 5° C in a car standing in the sun. Secondly, it lowers the surface temperature of seats and storage areas, for example, by up to 10° C.

The bottom line: automatic air conditioning provides adequate cooling even at the start of a journey and on less power. It means the blowers can operate at a lower setting, thus reducing noise levels and improving fuel economy. 
Development and technology

This new glass was jointly developed by the BMW Group, 3M and Saint-Gobain Sekurit, Germany. Manufactured by the partners in an elaborate production process, this glass has made it possible for the first time to protect the car’s interior from undesirable heat from the sun’s rays by the use of glass and plastic exclusively. 

Previously, this “solar protection function” had only been achieved by means of metal thin-film systems in the glass. These also reflect the infrared component in the sun’s rays and reduce the build-up of heat inside a car, but the advantage of the non-metallic interlayers is that they do not screen off the cabin area, ensuring improved reception of communications systems. 


Climate comfort laminated glass thus offers several benefits compared with standard glazing, namely enhanced thermal and acoustic comfort coupled with some saving in weight but without increased absorption of visible incoming light. 


For questions please contact:

Corporate Communications

Michael Blabst, AK-21, Production and Research
Telephone: 089-382-24697, Fax: 089-382-27563

Dr. Tobias Nickel, AK-21, Technology Communications
Telephone: 089-382-27072, Fax: 089-382-25763
Media Website: www.press.bmwgroup.com

e-mail: presse@bmwgroup.com







PAGE  

